Table of Contents: Chapter 17 Market Surveillance


ERCOT Protocols

Section 5: Dispatch

January 5, 2001

5-15
Dispatch

5.1
ERCOT Control Area Authority
5-1
5.2
Standards and Practices
5-1
5.3
Routine Dispatch Duties of ERCOT
5-2
5.4
Dispatch Instructions
5-2
5.5
Changes in ERCOT System Status
5-5
5.6
Emergency and Short Supply Operation
5-5


5 Dispatch

5.1 ERCOT Control Area Authority

5.1.1 Single Control Area

ERCOT will assume authority as Control Area Operator for the revised regional control area encompassing the boundaries of the previous control areas within the ERCOT interconnection. 

5.1.2 Operating Guides and North American Electric Reliability Council Guidelines

ERCOT will perform all control area functions as defined in the Operating Guides and the North American Electric Reliability Council (NERC) guidelines.

5.1.3 North American Electric Reliability Council/ERCOT Tagging Procedures

5.2 Standards and Practices

The QSE will follow all NERC guidelines for tagging of control area interchange transactions.  ERCOT will be operated as a single Control Area.  Therefore, no internal ERCOT transactions require tagging.    Only transactions across ERCOT interconnections to SPP, WSCC or Mexico, will be tagged by the QSE as prescribed in the NERC tagging guidelines.  

5.2.1 Operating Standards

ERCOT and TDSPs shall operate the ERCOT System in compliance with Good Utility Practice and NERC and ERCOT standards, policies, guidelines and operating procedures. These Protocols shall control to the extent of any inconsistency between the Protocols and any of the following documents:

(1) Any reliability guides applicable to ERCOT, including the Operating Guides;

(2) The NERC Operating Manual and ERCOT procedures manual, supplied by NERC and ERCOT, respectively, as references for dispatchers to use during normal and emergency operations of the ERCOT Transmission Grid;

(3) Specific operating procedures, submitted to ERCOT by individual transmission Facility owners or operators to address operating problems on their respective grids that could affect operation of the interconnected ERCOT Transmission Grid; and

(4) Guidelines established by the ERCOT Board, which may be more stringent than those established by NERC for the secure operation of the ERCOT System.

5.2.2 Equipment Operating Limits

ERCOT Dispatch Instructions shall respect all equipment operating limits.  Except as stated in Section 5.6, if a Dispatch Instruction conflicts with a restriction that may be placed on equipment from time to time by a Transmission and/or Distribution Service Provider (TDSP) or a Generation Resource’s Qualified Scheduling Entity (QSE) to protect the integrity of equipment, ERCOT shall honor the restriction. 

The TDSP will notify ERCOT of any limitations on the TDSP’s system that may affect ERCOT Dispatch Instructions.  ERCOT shall continuously maintain a posting on the Market Information System (MIS) of any TDSP limitations that may affect Dispatch Instructions.  Any conflicts that cannot be satisfactorily resolved may be brought to ERCOT by any of the relevant affected Entities for investigation and resolution.

5.3 Routine Dispatch Duties of ERCOT

ERCOT will deploy Ancillary Services and dispatch transmission in accordance with these Protocols, including but not limited to Section 4, Scheduling, Section 5, Dispatch, and Section 6, Ancillary Services, respectively, including deploying Balancing Energy, Out of Merit (OOM), Replacement Reserve Resources, RMR Units, and Synchronous Condenser Units to ensure operational security and to remedy all Commercially Significant Constraints (CSC) and Operational Constraints. 

ERCOT is the regional security coordinator for the ERCOT Region and is responsible for all regional security coordination as defined in the NERC Operating Manual and applicable ERCOT operating manuals or Operating Guides.

ERCOT may issue Dispatch Instructions to a TDSP (for the Real Time operation of Transmission Facilities) or to a QSE (for the Resources for which the QSE has scheduling responsibility.)

5.4 Dispatch Instructions

5.4.1 Control Area Operator Authority

ERCOT, as Control Area Operator (CAO), is authorized to take the following actions for the limited purpose of securely operating the ERCOT Transmission Grid in conformance with the standards specified in Section 5.2, Standards and Practices, including, but not limited to:

(1) Direct the physical operation of the ERCOT System, including circuit breakers, switches, voltage control equipment, protective relays, metering and Load shedding equipment;

(2) Call on Resources that have committed to provide Ancillary Services;

(3) Direct changes in the operation of voltage control equipment;

(4) Direct the implementation of OOM obligations and the use of RMR Service and transmission switching in order to prevent the violation of ERCOT System security limits; and

(5) Take those additional actions required to prevent an imminent Emergency Condition or to restore the ERCOT System to a secure state in the event of an actual ERCOT System emergency.

5.4.2 Contents of Valid Dispatch Instructions

Dispatch Instructions consist of Verbal Dispatch Instructions and Dispatch Instructions issued via the Messaging System.

5.4.2.1 Verbal Dispatch Instructions

Valid verbal Dispatch Instructions shall contain the following information:

(1) Identification of the responsible Entity and instructing authority (to include ERCOT operator’s and receiving operator’s names);

(2) Specific Resources or TDSP Facilities that are the subject of the Dispatch Instruction;

(3) Specific action required;

(4) Current operating level or state of the Resources or TDSP Facilities that are the subject of the Dispatch Instruction;

(5) Operating level or state to which such Resources or Facilities will be dispatched;

(6) Time of notification of the Dispatch Instruction;

(7) Time at which the QSE or TDSP is required to initiate the Dispatch Instruction;

(8) Time within which the QSE or TDSP is required to complete the Dispatch Instruction; and

(9) Other information relevant to the specific Dispatch Instruction.  

5.4.2.2 Dispatch Instruction via the Messaging System

Valid Dispatch Instructions issued via the messaging system shall contain the following information:

(1) Identification of the responsible Entity;

[PIP 158:  At such time as the system is adjusted to facilitate entry of the instructing authority, the following language will replace (1) above:]

(1) Identification of the responsible Entity and instructing authority (to include ERCOT operator’s and receiving operator’s names);

(2) Specific Resources that are the subject of the Dispatch Instruction;

(3) Specific action required;

(4) Time of notification of the Dispatch Instruction via a timestamp on the Dispatch Instruction;

(5) Time at which the QSE is required to initiate the Dispatch Instruction;

(6) Time within which the QSE is required to complete the Dispatch Instruction; and

(7) Other information relevant to the specific Dispatch Instruction.

[PIP 158:  The current operating level, the dispatched operating level, and the time of notification of the Dispatch Instruction are not supported in the current design.  When the functionality is included in the system, the items will be replace items #4 onward above with the following::]

(4) Operating level of any specific Resource(s) that are the subject of the Dispatch Instruction at the instant prior to the time the Dispatch Instruction will become valid;

(5) Current Operating level of any specific Resource(s) that are the subject of the Dispatch Instruction;

(6) Operating level to which the Resources will be dispatched;

(7) Time of notification of the Dispatch Instruction;

(8) Time at which the QSE is required to initiate the Dispatch Instruction;

(9) Time within which the QSE is required to complete the Dispatch Instruction; and

(10) Other information relevant to the specific Dispatch Instruction.

5.4.3 Dispatch Instruction Procedures

The procedures for issuing and responding to Dispatch Instructions are as follows:

(1) All Dispatch Instructions to Resources — whether for dispatch of Ancillary Services, System Emergencies or any other reason — shall be directed to the QSE responsible for the affected Resource.

(2) Each QSE must immediately forward any valid Dispatch Instruction to the appropriate Resource or group of Resources or identify a reason for non-compliance to ERCOT in accordance with Section 5.4.4, Compliance with Dispatch Instructions. 

(3) If ERCOT believes that a Resource or group of Resources has inadequately responded to a Dispatch Instruction, ERCOT shall notify the relevant QSE.

(4) The recipient of an oral Dispatch Instruction shall confirm the Dispatch Instruction by repeating it orally to ERCOT.  

(5) The recipient of a written or electronic Dispatch Instruction shall acknowledge receipt of the Dispatch Instruction to ERCOT in writing or electronically, within one (1) hour.

(6) The recipient shall immediately request clarification of the Dispatch Instruction if the recipient fails to understand its responsibility under the Dispatch Instruction.

(7) ERCOT shall record all voice conversations that occur in the communication of Dispatch Instructions. 

(8) ERCOT will record or file all written or electronic Dispatch Instructions and acknowledgements as soon as practicable after the issuance of the Dispatch Instruction.

(9) By mutual agreement of the QSE and ERCOT, Dispatch Instructions to the QSE may be provided to the QSE’s designated agent.

(10) By mutual agreement of the TDSP and ERCOT, Dispatch Instructions to the TDSP may be provided to the TDSP’s designated agent.

5.4.4 Compliance with Dispatch Instructions

(1) Each TDSP and each QSE within the ERCOT System shall comply fully and promptly with valid Dispatch Instructions, unless in the sole and reasonable judgment of the TDSP or QSE, such compliance would create a threat to safety, risk of bodily harm or damage to the equipment, or is otherwise not in compliance with these Protocols.

(2) If the recipient of a valid Dispatch Instruction does not comply because in the sole and reasonable judgment of the TDSP or QSE, such compliance would create a threat to safety, risk of bodily harm or damage to the equipment, or is otherwise not in compliance with these Protocols, then, the TDSP or QSE must immediately notify ERCOT and provide the reason for non-compliance. 

(3) If the recipient of a Dispatch Instruction recognizes that the Dispatch Instruction conflicts with other valid instructions or is invalid, the recipient will immediately notify ERCOT of the conflict and request resolution.

(4) Upon receipt of notice of non-compliance or conflict with Dispatch Instructions, ERCOT may, but is not required to, issue new Dispatch Instructions in response to the TDSP or QSE’s notice.

(5) ERCOT’s final instruction in effect will apply for all Protocol-related processes. If the QSE does not comply after receiving the final instruction, the QSE shall remain liable for failure to meet its obligations under the Protocols and shall remain liable for any charges resulting from such failure. 

(6) ERCOT’s final instruction to a TDSP in effect will apply for all Protocol-related processes. If the TDSP does not comply after receiving the final instruction, the TDSP shall remain liable for such failure under these Protocols in accordance with the TDSP’s Agreement with ERCOT. 

(7) In all cases in which compliance with a Dispatch Instruction is disputed, both ERCOT and the QSE or TDSP shall document their communications, agreements, disagreements, and reasons for their actions, to enable resolution of the dispute through the ADR process in Section 20, Alternative Dispute Resolution.

5.5 Changes in ERCOT System Status 

5.5.1 Changes in Resource Status

The QSE will notify ERCOT of an unplanned change in Resource status as soon as practicable following the change. The QSE representing the Resource will report any changes in Resource status to ERCOT in the Resource Plan by the beginning of the next hour following the change in status.

5.5.2 Changes in Transmission Facility Status

The TDSP will notify ERCOT of any changes in status of Transmission Facility elements as specified pursuant to these Protocols.  The TDSP will notify ERCOT of any other Transmission Facility status as soon as practicable following the change.

5.6 Emergency and Short Supply Operation

5.6.1 Introduction

ERCOT, as the single Control Area Operator, is responsible for maintaining reliability in normal and emergency operating conditions.  The Operating Guides are intended to ensure that minimum standards for reliability are maintained. Minimum standards for reliability are defined by the Operating Guides and the North American Electric Reliability Council (NERC) standards and include, but are not limited to:  

(1) Minimum operating reserve levels, 

(2) Criteria for determining acceptable operation of the frequency control system, 

(3) Criteria for determining and maintaining system voltages within acceptable limits, 

(4) Criteria for maximum acceptable transmission equipment loading levels, and 

(5) Criteria for determining when ERCOT is subject to unacceptable risk of widespread cascading outages. 

In implementing these Protocols, ERCOT shall, to the fullest extent practicable, utilize market tools as prescribed in these Protocols before implementing command and control actions, such as OOMC, OOME, or RMR Service. It is anticipated that, with effective and timely communication, the tools available to ERCOT from the market will avert most threats to the reliability of the ERCOT System. However, these Protocols shall not preclude ERCOT from taking other actions to preserve the integrity of the ERCOT System.  
5.6.2 Communication

Good, accurate, and timely communication between ERCOT, TDSPs, and QSEs is essential. The QSEs must be provided adequate information to make informed decisions and must receive the information with sufficient advance notice to facilitate generation and/or Load responses.

The type of communication ERCOT will issue will be determined primarily on the basis of the time available for the market to respond before an Emergency Condition occurs. The timing of these communications could range from days in advance to immediate. If there is insufficient time to allow the market to react, ERCOT may bypass one or more of the communication steps.  

ERCOT shall consider the severity of the potential Emergency Condition as it determines which of the communications set forth in Section 5.6.3, Operating Condition Notice, through Section 5.6.6, Emergency Notice, to use. The severity of the Emergency Condition may be limited to an isolated local area affecting a small number of megawatts, or cover large areas affecting several hundred megawatts, or may be an ERCOT-wide condition potentially affecting all of the ERCOT Region. 

The following sections describe the types of communications that will be issued by ERCOT to inform all QSEs and TDSPs of the operating situation. These communications may relate to transmission, distribution, and/or generation. The communications shall specify the severity of the situation, the area affected, the areas potentially affected, and the anticipated duration of the Emergency Condition.

5.6.3 Operating Condition Notice

ERCOT will issue an Operating Condition Notice (OCN) to inform all QSEs of a possible future need for more Resources due to conditions that could affect ERCOT System reliability. OCNs are for informational purposes only, and ERCOT exercises no extra authority with the issuance of this type of notice. 

When time permits, ERCOT will issue an OCN before issuing an Advisory, Alert, or Emergency Notice. However, issuance of an OCN does not require action on the part of any Market Participant, but rather simply serves as a reminder to QSEs and TDSPs that some attention to the changing condition may be warranted. OCNs serve to communicate to QSEs the need to take extra precautions to be prepared to serve the Load during times when contingencies are most likely to arise.

Reasons for OCNs include unplanned transmission outages, and weather related concerns such as anticipated freezing temperatures, hurricanes, wet weather, and ice storms. 

ERCOT will monitor actual and forecasted weather for ERCOT and adjacent NERC regions. When adverse weather conditions are expected, ERCOT may confer with TDSPs and QSEs regarding the potential for adverse reliability impacts and contingency preparedness.

QSEs and TDSPs are expected to establish and maintain internal procedures for monitoring actual and forecasted weather and for implementing appropriate measures when the potential for adverse weather or other conditions (which could threaten ERCOT System reliability) arise. 

5.6.4 Advisory

ERCOT will issue an Advisory for informational purposes for the following reasons:  

(1) When it recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability, or 

(2) When weather or ERCOT System conditions require more lead-time than the normal Day Ahead market allows. 

(3) When communications or other controls are significantly limited, or 

(4) When ERCOT Transmission Grid conditions are such that operations within first contingency criteria as defined in the Operating Guides are not likely or possible because of Forced Outages or other conditions. 

The Advisory communicates existing constraints.  ERCOT will notify TDSPs and QSEs. QSEs will notify appropriate Resources and LSEs.  ERCOT will communicate with TDSPs as needed to confirm their understanding of the condition and to determine the availability of Transmission Facilities.  For the purposes of verifying submitted information, ERCOT may communicate with QSEs.  

Although an Advisory is for information purposes, ERCOT may exercise its authority, in such circumstances, to increase Ancillary Service requirements above the quantities specified in the normal Day Ahead plan in accordance with scheduling procedures. ERCOT may also increase the Day Ahead market to Two Days Ahead. 
5.6.5 Alert

ERCOT will issue an Alert when ERCOT determines:

(1) That conditions have developed such that additional Ancillary Services are needed in the Operating Period; 

(2) That market Congestion Management techniques specified in these Protocols will not be adequate to resolve transmission problems; or 

(3) Forced Outages or other abnormal operating conditions occur which require operations outside first contingency security limits as defined in the ERCOT Operating Guides;

(4) That there are insufficient AS bids.

ERCOT will post the Alert electronically and will notify all TDSPs and QSEs via the Messaging System of the posted Alert(s).  

ERCOT must issue an Alert before acquiring Emergency Short Supply Regulation Services, Emergency Short Supply Responsive Reserve Services or Emergency Short Supply Non-Spinning Reserve Services. With the issuance of an Alert pursuant to item (1) or (4) above, ERCOT may exercise its authority to immediately procure the following services from existing bids:

(1) Regulation Services, 
(2) Responsive Reserve Services, and 
(3) Non-Spinning Reserve Services. 
Emergency Short Supply Regulation Services, Emergency Short Supply Responsive Reserve Services or Emergency Short Supply Non-Spinning Reserve Services will be procured if there is insufficient availability of bids for any of the listed Ancillary Services. 

ERCOT will post the Alert electronically on the MIS and will notify all TDSPs and QSEs via the Messaging System of the posted Alert(s). 

Corrective actions identified by ERCOT shall be communicated through Dispatch Instructions to TDSPs and/or QSEs required to implement the corrective action. Each QSE shall immediately notify the Market Participants that it represents of such Alert.  To minimize the effects on the ERCOT System, all TDSPs will identify and prepare to implement actions, including restoring outaged lines as appropriate and preparing for Load shedding. ERCOT may instruct TDSPs to reconfigure ERCOT System elements as necessary to improve the reliability of the ERCOT System.   On notification of an Alert, each QSE and TDSP will prepare for an emergency in case conditions worsen. 

5.6.6 Emergency Notice 

ERCOT will issue an Emergency Notice only for the following reasons:

(1) ERCOT cannot maintain minimum reliability standards during the Operating Period using every Resource practicably obtainable from the market, 

(2) ERCOT is in an unreliable condition, as defined below, 

(3) Immediate action must be taken to avoid or relieve an overloaded transmission element, or

(4) ERCOT varies from timing requirements or omits one or more scheduling procedures, as described in Section 4.8, Temporary Deviations from Scheduling Procedures.

The actions ERCOT takes during an Emergency Condition will depend on the nature and severity of the situation. 

ERCOT is considered to be in an unreliable condition whenever ERCOT Transmission Grid status is such that the most severe single-contingency event presents the threat of uncontrolled separation or cascading outages and/or large-scale service disruption to Load (other than Load being served from radial transmission service) and/or overload of a critical transmission element, and no timely solution is obtainable from the market.

If the Emergency Condition is the result of a transmission problem that puts ERCOT in an unreliable condition, then ERCOT will act immediately to return ERCOT to a reliable condition, including instructing Resources to change output and instructing TDSPs to drop Load.

If the Emergency Condition is the result of a short supply situation, then the Emergency Electric Curtailment Plan procedures will be followed.

5.6.6.1 Emergency Electric Curtailment Plan (EECP)

At times it may be necessary to reduce electrical demand because of a temporary decrease in available electricity supply. To provide orderly, predetermined procedures for curtailing demand during such emergencies, ERCOT will initiate and coordinate the implementation of the Emergency Electric Curtailment Plan following the EECP Steps set forth below in subsection 5.6.7.1, Restoration of Market Operations.
The objective of the EECP is to provide for maximum possible continuity of service while maintaining the integrity of the ERCOT Transmission Grid in order to reduce the chance of cascading outages.

ERCOT’s operating procedures shall meet the following goals while continuing to respect the confidentiality of market sensitive data:

(1) Use of the market to the fullest extent practicable without jeopardizing the reliability of the ERCOT system;

(2) Use of Responsive Reserve Services and other Ancillary Services to the extent permitted by ERCOT System conditions;

(3) Maximum use of ERCOT System capability;

(4) Maintenance of station service for nuclear Generation Resource Facilities;

(5) Securing of startup power for Generation Resources;

(6) Operation of power Generation Resources during loss of communication with ERCOT;

(7) Restoration of service to critical Loads in the manner defined in theOperating Guides; and

(8) Restoration of service to all customers following major system disturbances, giving priority to the larger groups of customers.

ERCOT shall be responsible for coordinating with QSEs and TDSPs to monitor system conditions, initiating the EECP steps set forth in this Section, notifying all QSEs, and coordinating the implementation of the EECP Steps while maintaining transmission security limits.

During the EECP, ERCOT has the authority to obtain energy over the DC Ties when transmission capacity is available across the DC Tie. ERCOT will not supercede any market use of the DC Ties.

Some of the EECP Steps will not be applicable if transmission security violations exist. There may be insufficient time to implement all Steps in sequence, but to the extent practicable, ERCOT will use Ancillary Services which bidders have made available in the market to maintain or restore reliability.

ERCOT may immediately implement Step 4 of the EECP any time the steady-state system frequency is below 59.8 Hz and will immediately implement Step 4 any time the steady-state frequency is below 59.5 Hz.

Percentages for Step 4 Load shedding will be based on previous year’s TDSP peak Loads, as reported to ERCOT, and will be reviewed by ERCOT and modified annually.

5.6.7 EECP STEPS 

Step 1 — Maintain 2300 MW of ERCOT’s reserve service Resources comprised of: (1) undeployed Responsive Reserve Service; and (2) that amount of Non-Spinning Reserve Service which has been deployed but has not ramped to its specified output level. 

  ERCOT will: 

(1) Use those available Ancillary Services that can be deployed to increase Responsive Reserves.

(2) Provide Dispatch Instructions to QSEs to start all Resources that are available in the time frame of the emergency (OOMC). Similarly, ERCOT will provide Dispatch Instructions to QSEs to maximize Resource deployment (OOME) to increase Responsive Reserve levels on other Resources. QSEs intending to provide OOMC and OOME will comply as soon as practicable

(3) Use available DC Tie capacity that is not already being used by the market.

(4) Start RMR Units available in the time frame of the Emergency. RMR Units should be loaded to full capability.

Step 2 — Maintain ERCOT undeployed Responsive Reserve Service on Generation Resources  equal to the largest Generation Resource on-line in ERCOT.  In addition to measures associated with Step 1, ERCOT will:  

(1) Instruct TDSPs to reduce Load by using distribution voltage reduction management tools, if deemed beneficial by the TDSP. 

(2) Deploy all Responsive Reserve which is supplied from Load acting as a Resource (controlled by high-set under-frequency relays) to remove such Load.

(3) With the approval of the affected Non-ERCOT control area, may instruct TDSPs to implement BLTs, which transfer load from the ERCOT control area to Non-ERCOT control areas. Use of a BLT will be defined in the ERCOT operating Guides.   
Step 3 – Maintain system frequency at 60 Hz

In addition to measures listed above, ERCOT will issue an appeal through the public media for voluntary Load curtailment.

Step 4 — Maintain system frequency at 59.8 Hz or greater

In addition to measures listed above, ERCOT will instruct, in 100 MW blocks, all TDSPs having control over distribution feeder breakers and/or control of breakers serving retail customers to shed Load in order to maintain a steady state system frequency of 59.8 Hz. ERCOT will allocate manual Load shedding for ERCOT-wide emergencies based on the amount of Load that is served by each TDSP.

5.6.7.1 Restoration of Market Operations

ERCOT shall continue EECP until sufficient bids are received and deployed by ERCOT to eliminate the conditions requiring the EECP.

5.6.8 Emergency Compensation

Deployment and compensation for OOMC and OOME Resources that ERCOT instructs for deployment due to a supply-related Emergency will be according to Section 6.8.2, Capacity and Energy Payments for Out of Merit Services. 

There will be no compensation other than normal settlement methods to Loads in the event of Load shedding by the TDSP during EECP. 
5.7 Block Load Transfers between ERCOT and Non-ERCOT Control Areas
[PIP 209:  The following language is not complete and must be modified in order to implement BLTs.  There is no manual work-around to accommodate this process.   At a high level the gaps to be reconciled are:

a. While all BLTs would in effect be treated as inadvertent between control areas (similar to existing DC Ties), ERCOT has no mechanism to separately identify a BLT.  A BLT flag would need to be created in the system and the DC Tie aggregation program code modified to support the processes detailed below.
b. Additional issues include how ERCOT will schedule exports to offset the inadvertent imports without actually having control of resources.

When the system functionality is implemented for BLTs, the following language should be added to the Protocols.]

Under Alert or EECP conditions it may be necessary for ERCOT to request the implementation Block Load Transfers (BLT) schemes which will transfer loads normally located in the ERCOT Control Area to a Non-ERCOT Control Area.  Similarly, when Non-ERCOT Control Areas experience certain transmission contingency or short supply conditions, it may be necessary for ERCOT to agree to the implementation of BLT schemes which will transfer loads normally located in a Non-ERCOT Control Area to the ERCOT Control Area.  BLTs will be restricted to the following conditions:  

(1) 
BLTs shall only occur under a specific Dispatch Instruction from ERCOT.  

(2) 
BLTs that are looped systems should not be tied to the other power pool’s electrical system through two interconnection points at the same time.  Transfers of looped configurations through two interconnection points will not be permitted.  

(3) 
BLTs of load to the ERCOT Control Area will only be permitted if such transfers will not jeopardize the reliability of the ERCOT System.

(4) 
BLTs of load from the ERCOT Control Area will only be instructed with the permission of the affected Non-ERCOT Control Area. 

(5) 
Restoration of service to outage customers using BLTs will be accomplished, as quickly as possible if the transfers will not jeopardize the reliability of the ERCOT System.
(6)
The necessary Market Participant agreements, metering, and ERCOT settlement systems are in place prior to the implementation of any BLT.

(7) 
Installation of any BLT metering point or any metering point for transfer of Load from another control area into ERCOT requires the approval of ERCOT.  The TDSP responsible for such point is required to complete and submit all applicable documentation, including registration, of BLT “tie” points.  ERCOT may impose additional requirements as deemed necessary to ensure ERCOT Grid reliability and integrity.       
5.7.1  BLTs between ERCOT and the Southwest Power Pool (SPP)

 5.7.1.1. ERCOT Settlement

A QSEs load obligation in ERCOT will not be impacted as a result of the implementation of any BLT with SPP.  The ERCOT Settlement will include loads normally located within ERCOT regardless of their actual location during a particular settlement interval. The ERCOT Settlement will not include loads normally located in SPP regardless of the load’s actual location during a particular settlement interval.

5.7.1.2  Inadvertent Energy Account

The amount of load transferred under the BLT shall be added to the Inadvertent Energy Account between the ERCOT Control Area and the interconnected SPP Control Area.  Corrections of inadvertent energy between ERCOT and the interconnected SPP Control Area shall be in accordance with Section 4.4.18.5.

5.7.2  BLTs involving Comision Federal De Electricidad (CFE)

5.7.2.1  ERCOT Settlement

A QSEs load obligation in ERCOT will not be impacted as a result of the implementation of any BLT of ERCOT load to the CFE Control Area.  The ERCOT settlement will include loads normally located in ERCOT regardless of their actual location during a particular settlement interval. 

Loads normally located in the CFE Control Area are not considered as BLTs by definition and shall be treated as standard retail service delivery points for all ERCOT processes and purposes.  

5.7.2.2.  BLTs from ERCOT to CFE 

ERCOT shall establish an Inadvertent Energy Account between the ERCOT and CFE Control Area.  The amount of load transferred from ERCOT to CFE under the BLT shall be added to the Inadvertent Energy Account between the ERCOT Control Area and the interconnected CFE Control Area.  Corrections of inadvertent energy between ERCOT and the interconnected CFE Control Area shall be in accordance with Section 4.4.18.5. 

5.7.2.3  Load  from CFE served by  ERCOT 

 Load transfer from the CFE Control Area to the ERCOT Control Area shall only occur if an ESI-ID has been established and is identified as active in ERCOT systems.  In the event no active ESI-ID exists for the CFE Load point,  a request to implement  a Load transfer from the CFE Control Area to the ERCOT Control Area shall be denied.  Any ESI-ID established for purposes of serving a Load transferred from CFE shall comply with all ERCOT Protocols and Guides.    
1

