[image: image1.png]



          Services provided by Northern Natural Gas

          And Transwestern Pipeline Company

Enron Transportation Services
Gas Control Event Log

0630 to 1830 Shift Thurs., Jan. 17, 2002  ( Muntz, Henley, Spain, Monden)

	0635
	Faribault
	It is on line pumping north to Farmington.

Monden



	0813
	Atoka 3
	Louis called in and said unit 761 would be down for approx. six hours so they can change out a head.

Muntz



	0822
	P3
	Unit 5 just shut down.  Steve is going to check it out.

Muntz



	0834
	Station 2
	Hank called in and said they will be pigging the line from station 2 to station 1 this morning.

Muntz



	0910
	Beaver Co. 6
	They are running a pig back behind the station. Al called in and said he was going to shut the station down for a little while.

Spain



	0915
	Kermit
	Darrel called in from Kermit.  They’ve got an oil leak, pretty bad oil leak on the small turbine there.  He’s going to have to take it off line.  That’s the only thing we’ve got going down in the subsystem now.  He anticipates being down for about an hour if all of these parts fit like they should.  Until then I did call Jal. I’m waiting for a return call for the on call person.  I’ve got to slow the units down or bypass them until he can get back on line with that small turbine.

Spain



	0920
	Beaver
	I called and spoke to Oscar.  Asked him to back into the “KB” line, bring it up about 40#.  I need some pressure on the downstream side of the Plainview turbine.  They’d like to start that and trouble shoot the problems they are having there with that turbine.  I need to equalize the pressure there through Plainview.

Spain



	0925
	Jal
	Got a hold of Bill Houston.  He’s going to go out and bypass Jal until we get the small turbine at Kermit back on line.

Spain



	0940
	Spraberry
	Jimmy called back in.  I had called him earlier and informed the station that we had a 27 million delivery to Valero.  I need to put another unit on there.  Jimmy informed me that they’ve got a unit with an oil line that’s plugged up. It’s going to probably be down for about two hours so there will be a delay in making that delivery, unit #10.

Spain



	1005
	Kermit
	Darrel just called in concerning the oil leak.  His spare part, the fittings are wrong.  It won’t fit.  He has to go to Odessa and get a new hose made.  It will probably be about three hours before we have anything available at Kermit.

Spain



	1040
	Beaver
	I paged Oscar again up at Beaver.  I didn’t see any results from backing in on the “KB” line.  He informed me that he was going to take a big unit off, unit #20, and put a 2000 on and back in.

Spain



	1045
	Beaver Co. 6
	Al just called in.  He’s got the units back on line.  The pig is in.

Spain



	1123
	Owatonna
	It is put back on line.  They got their electrical supply issue fixed.

Monden



	1253
	
	Had shut Faribault down due to the recycle was open 100%.  At Oakland I asked Mike to shut down one on the “D” to “D” and go to the minimum units he can run, 5 to 6 units on the “BC”.

Monden



	1336
	Galena
	Put the second unit on line.  The discharge from Belleville is 703 and LS Power is at 552 on pressure.

Monden



	1415
	Lone Star Ward
	The Lone Star Ward point keeps bouncing back and forth between #1 and #2 fuel runs.  I tried different flow setpoints.  It doesn’t seem to matter.  Called the Monahans team and spoke with Baker.  He said they will have to make a plate change.  They’ll have somebody out there in about 20 minutes.  In the meantime I’ve shut the interconnect in.  I’ve contacted Lone Star gas control and let them know that we were going to have to make a plate change and asked if they could have someone meet our person at the interconnect in about 20-30 minutes.

Muntz



	1345
	Kermit
	Darrel just called in.  He got the part a little earlier than expected for the oil line there for the small turbine.  He’s got it back on and ready to crank it up.  I put a call into Bill over at Jal.  Asked him to fire those units up also.

Spain



	1424
	Station 9
	Station 9 is down.  Talked to Richard.  He said they thought they knocked it off when they were doing some testing.  He said they would have it back on as soon as possible.

Muntz



	1450
	Clark Co. 2
	Dale called in.  He’s got unit #7 back on line.  It’s gone down twice today but he’s got it back on.

Spain



	1400
	Clifton
	Took unit 28 down.  It was on with the turbine on the “E” line.

 Henley



	
	
	Also earlier this morning I called about Finney Co. 3 due to their H2S.  Rick Weatherford ran a sample to check the level to see if it is that high.  We’ve been monitoring it the last couple of days.  He verified that it was that high and that the calibration was OK.  He contacted Oneok and asked them to identify the source.  After that I called the Holcomb operator.  He is going to follow through with that and check on it further.

Henley



	1505
	Lone Star Ward
	Baker just called in.  They’ve changed the plate at Lone Star Ward to a 4.750 plate.  Requested that we flow a minimum of 20 million setpoint or higher.

Muntz



	1538
	Atoka 1
	Unit 794 was back on at 1400 this afternoon.  Louis said it will be on the rest of the night.

Muntz



	1553
	Atoka 2
	Terry Youngren has the total sulfur instrument working again at Atoka 2.  Currently reading 0.9406.

Muntz



	1624
	Agave Red Bluff
	The flow went to zero.  I called the pager number 1-800-588-4966 and left a phone number.

Muntz




1830 to 0630 Shift Thurs., Jan. 17, 2002  (Hall, Armitage, Barry, Braswell)

	2045
	
	Due to line pressures and volumes from the south we have cut Cunningham to 350.  It’s a good possibility we might have to go back after some more but until that time shows up we’re going to leave it at 350 withdrawal.

Armitage



	2100
	Beatrice
	Called the OCC.  They got a hold of the on call Larry.  At that time I had 994, just got up there on the Beatrice turbine.  It is down now, just shut itself down.  He is headed out though to take care of it.

Hall



	2130
	Beatrice
	Just got through talking to Larry.  We’re going to leave his turbine down for now because he’s got 828 and a 935. The station isn’t bypassed on the high side yet.  Don’t have any room to put him back on anyhow.  It would be right up there again.  We’re going to leave it down until a little later tonight and maybe we can get it back on later.

Hall



	0117
	Oakland
	Called the OCC.  They got a hold of Jim.  He’s going to go out and put three units back on the high side for us.

Hall



	0124
	Farmington
	Called the OCC.  They got a hold of the on call. Larry is going to get a hold of Tony and see if he can get it reset, the electric, because it went down. Really didn’t get a flag on it as to why it went down.  It just dropped off for some reason.  They will reset it and see if they can get it back on for us.

Hall



	1900
	
	Got a call from Calpine informing me that they were having some problems with their turbine and that they would probably be taking half of what they actually scheduled on their nom which is 85.2 million.  They are spotting right now 44.9 MMBtu.

Braswell



	0020
	
	Shut down unit 402.  Touched bases with Howard out there.  I was telling him that pressures were pretty much growing out at Needles and that they were needing to kind of stabilize those pressures at this time so was needing to drop a unit at station 4.  I did drop it from here remotely.

Braswell



	0122
	WFS Milagro
	Got a call from Warren.  He told me that they had lost two turbines behind the Milagro point.  They had people dispatched out there to get the units back on.  Right now we have scada at Milagro of about 315.  They backed all the way down to about 195.

Braswell



	0119
	P3
	Lost unit 5 there.  Called the OCC for the on call.  Touched base with Tony.  Informed him that the unit had gone down.  He was going out to put it back on hopefully.

Braswell



	0145
	Farmington
	Just got a call back from Tony.  He said he couldn’t reset it from the house. It has a RTD alarm.  He asked about possibly using 4 or 5.  I told him that I was afraid that it might throw me up too high on the discharge and eventually I’d probably lose it.  He is going to go out and see what he could do, if he can get it reset when he gets out there.  If not he may have to call somebody else out to help him out with it.

Hall



	0248
	Oakland
	Just got a call from Jim.  He’s got two of the units on at Oakland on the high and he was getting ready to put a third one on as soon as his air built up enough.

Hall



	0440
	Harrisburg
	Got a hold of the OCC.  They got a hold of the on call.  I let him know that our pressure at Brookings at that time was 482.  Right know it’s currently 479.  He said that he would be out there within an hour or so and take a look at it.

Hall



	0500
	Ogden
	Called the OCC.  They got a hold of the on call.  Asked him if for sure the Redfield pressures at Ogden are common on the “B” and the “C”.  He said yes.  He also said that he would be out there in about an hour, spoke to Darrel.  He will get a hold of Redfield and visit with them.

Hall



	0511
	Heartland Corn
	Called out there.  I had talked to Steve our boss.  He said to go ahead and give them a call and ask them to back off a little bit.  I called out there and talked to Paul Ecker, one of the plant operators.  He said he would have them switch one of their units to propane.

Hall



	0515
	Beatrice
	Just called the OCC.  They got a hold of Larry the on call.  He is going to go out and put the  turbine on at Beatrice.

Hall



	0521
	P3
	Lost unit 5 again.  Called the OCC for the on call.

Braswell



	0542
	WT2
	Lost unit #1.  Called the OCC for the on call.

Braswell




slb  

