[image: image1.png]



          Services provided by Northern Natural Gas

          And Transwestern Pipeline Company

Enron Transportation Services
Gas Control Event Log

0630 to 1830 Shift Sun., Jan. 27, 2002  (Armitage, Munson, Welsh, Cummings)

	0754
	Needles
	Socal’s flow has just dropped off significantly.  I called their gas control to see what the problem is.  Phil their operator said that they are having unit problems at their north station.  He said that they are addressing the problem, trying to get their HP back on.

Munson



	0815
	Needles
	I have not heard anything back from SoCal regarding their problem.  Our pressure has grown about 50# in the last 20 minutes.  I called Phil one more time to get a status report.  He said that he’d have to get back with me.  He wasn’t sure what the status was at this time.  He thought his people were still trying to get their HP started.  I asked him if he knew how much longer it was going to be.  He said he’d have to get back to me.  I let him know that this is altering our pressure profiles a great deal and that I was going to have to start making my own HP changes to compensate for the increase in pressure at the border if he didn’t get HP on line soon.

Munson

 

	0820
	Needles
	I heard back from Phil at SoCal.  He said it’s going to be at least another 20 minutes before we get this HP back on.

Munson



	0915
	Needles
	The SoCal flow at Needles has recovered.

Munson



	1115
	Oneok Ward
	The Oneok Ward flow went to zero.  I called Gary at Oneok and he said it was caused by the pressure override kicking in.  He was able to open it right back up again.  I notified Dean and Robert Baker both on the Monahans team and let them know that this occurred.  Robert is going to check the settings on the pressure override shut-in first thing in the morning and make sure that everything is calibrated and working properly because it shut in about 10-15# sooner than it should have.  He will look into that and make sure everything is functioning properly over there but we’re back open at this time.  It was only shut about 10 minutes.

Munson



	1150
	Valverde 
	I called and talked to Larry.  Asked him if he would bring his CO2 level coming out of the Meridian Valverde plant back down below 2.0.

Munson



	1250
	Station 5
	Unit 3 has gone down. Eugene is on location.

Munson



	1315
	WT1
	Johnny Lamb called from Duke Energy concerned about his flow going to zero at the WT1 location.  I hooked up a conference call with Johnny and Jim Risher from our Carlsbad team.  We confirmed that on scada and telemetry everything looks fine at that location on our end.  We show that the valve has gone 100% open though from an earlier standing of about 15% open.  The digitals show valves are open and there is no H2S indicators.  Jim Risher on our team thinks that the problem lies on their side with a possible freeze problem.  He relayed this information to Johnny.  Johnny said that he would then send people from his group out to look things over and to get back with us if needed.  The alarm came in earlier for the flow rate going to zero.  This is usually an indication that they are having compression problems.

Munson



	1320
	Station 5
	Unit 3 is back on line.

Munson



	1320
	Station 7
	We have a unit alarm on unit 703.  This unit has been backed off to a very low speed on RPM due to the roll back in pressure from station 5 having the unit down.  It put unit 7 down to a very low RPM.  We need to have 7 on.  Now that 5 is back on with their unit things should start looking up and put 7 in a better light.  In the meantime I did notify Keith on the Mountainair team.  He said that either he or Norm would check it out and make sure that the unit is OK even though it’s functioning on low RPM’s

Munson



	0856
	Ogden
	Have put a start in unit 13.

Armitage



	1000
	Farmington
	Tried to put a start in #2 and it wouldn’t go.  Have called one of the men out and he’s put #2 on the line going west.

Armitage



	1050
	Ogden
	Unit 15 has been started and put on the line to go north with.

Armitage



	1315
	
	Sent a start to Owatonna to bring it on line.

Armitage



	1414
	Farmington
	Have put a start in #5.

Armitage



	1406
	Station 5
	Unit 503 is down.  The personnel are still on site.

Munson



	1430
	Station 5
	Unit 503 is back up.

Munson



	1758
	Maljamar
	Unit #1 has gone down.  Jim Risher is on his way out to put it back on.

Munson




 1830 to 0630 Shift Sun., Jan. 27, 2002  (Hardy, Henley, Spain, Monden)

	1839
	Farmington
	Unit #5 has gone down.  Notified OCC for the contact.  They got a hold of Ken on the Farmington team.  He said he would get somebody to check it out.

Monden



	1942
	Farmington
	Dan on the Farmington team had called in.  He got unit #5 reset.  Suggested that we put unit #6 on which is an electric.  It would be a little bit more flexible to run the unit. Appeared to be the good decision due to the fact that we were coming up on our discharge and a smaller unit would fit the situation better so we put unit #6 on the line.

Monden



	2144
	Station 4
	Received an AC power fail alarm.  Talked with Robert from the station 4 team.

Henley



	2210
	Clifton
	Unit #1 went down.  It was on the “E” service.  I called the on call and spoke to Mike.  I asked him to go out and check it out.  We’ll need to put it back on line.

Spain 



	2310
	Clifton
	We’ve got that unit back on line.  Mike didn’t know what the problem was but it seems to check out all right so it’s back on line.

Spain



	2305
	Stevens Co. 7
	Unit 4 went down.  I called the on call person and spoke to Bruce.  He is going to go out for us.

Spain



	2310
	Sublette
	I’ve had an alarm on unit #26, the turbine out there.  I called the on call person, spoke to Mike.  He said it was nothing to worry about, probably some kind of air filter alarm, something minor that wouldn’t make any difference on the performance of the unit so I let it be his call.

Spain



	2313
	Farmington
	The alarm on #3 was a low oil pressure alarm but not enough to shut it down.  The unit was running slowed down.  I spoke to Dan at Farmington.  I didn’t really want the gas to keep going north since I was already  up to 864 and still gradually climbing, didn’t want it to spill over from the west to the north.  We went ahead and took unit #3 down pumping west.

Monden



	0108
	Guthrie Center
	The turbine is on the line on the “BC”.

Monden



	0120
	Stevens Co. 7
	Bruce called in.  He’s got the unit back on line.

Spain



	0330
	Cunningham
	I gave an order to start a withdrawal rate of about 100.  We’ll be increasing that as the day goes on but 100 is about all that will fit at this time.

Spain



	0349
	Station 4
	Earlier tonight we had an AC power failure at station 4.  The generator didn’t come on like it was supposed to.  Robert went out there.  They ended up having to use some additional batteries to keep the power on and keep the air going.  They did get the alarm cleared earlier and then it came back now.  Robert is still out there with it.

Henley



	0445
	Duke Energy
	Received a call from Johnny Lamb.  He called regarding the receipt point at WT1.  He is complaining that our pressure is coming up.  I trended it and it is coming up but they are still making rate and our control valve isn’t closed. I’m going to call the on call anyway.  He said that earlier today that they had a freeze off and that it was causing the valve to pinch and causing them to have a pressure problem.  We’re going to call them out and have the on call check it out.

Henley



	0520
	Duke Energy
	WT1 receipt point – Just talked with Jim Risher.  He went out and check that out for us.  Johnny Lamb had called earlier regarding that point.  Jim doesn’t see a problem with it.  He looked at the control valve.  The control valve is doing exactly what it’s supposed to do.  He’s going to talk with Johnny about that because earlier today he did say that they did have a freeze off and it was acting the same way as it is now with the pressure but at this point he doesn’t see a problem.

Henley




slb  

