 [image: image1.png]



          Services provided by Northern Natural Gas

          And Transwestern Pipeline Company

Enron Transportation Services
Gas Control Event Log

0630 to 1830 Shift Mon., Oct. 15, 2001  (Alvarado, Cummings, Welsh, Calhoun)

	0656
	Jal
	Unit #2 has gone down.  I’ve contacted the OCC.  Waiting to hear from the on call at this time.

Alvarado



	0731
	Jal
	Unit #2 is back on.  Russell was at the location and put the unit back on.

Alvarado



	0610
	Galena
	Dave was finally able to get unit #2 on line.  He said that #1 was available now also.

Calhoun



	0745
	Redfield
	Cut their order back.  They were doing 230 injection.  Cut it back to 200.  Also told him to run on the low side of that.  That’s primarily due to the loads picking up and having cooler temperatures up north.

Calhoun



	0810
	Guthrie Center
	Started the turbine.  I had moved most all of the gas off the south at the interconnect and Ogden was running out of suction.  I put Guthrie on to get them some suction up there from the south.

Calhoun



	0830
	Beatrice
	I had Ron put another unit on the “D” line.  He was going to add 200 HP.  We only had 100# differential or spread across the “D” line.  We needed more flow north anyway.

Calhoun



	0900
	Madison
	They wanted to go from 23 to 35.

Calhoun



	0915
	Waterloo
	Did a station start.  That was 8000 HP.  That’s primarily due to we were falling off at Galena and Earlville.

Calhoun



	0925
	Madison
	They wanted to go from 35 to 43 million.

Calhoun



	1005
	
	Got a call from Greg Lindblom from North Branch.  He was out at the MN1D interconnect.  He said Minnegasco wanted him to raise the inlet pressure from 175 to 250. That way it would be set up for wintertime operations.

Calhoun



	0949
	Hugo
	Sent a start command to the unit to start it up.  We were needing more gas up north to North Branch.  They were running out of suction and the discharge was falling off on the unit and also on up north at Carlton.

Calhoun



	1020
	South Sioux
	Got a low alarm of 340 on the “A” line.  I called and talked to Darrel Belt.  He was going to raise the regulator at the South Sioux yard from 340 up to 375.  Also Duane was going to go up to the Fremont #1 NE area.  There is another regulator there that they’ve got a 380 in.  They are going to raise it up to 400.

Calhoun



	1040
	Palmyra
	Talked to Mike Butts.  He was going to split the “B” out from the “CD” suctions.  He was going to put a unit directly on “B” to “BC”.  That way we can get the block valve open down at Beatrice and start flowing on the “B”.

Calhoun



	1100
	Faribault
	Put a start command in on the unit.

Calhoun



	1100
	
	We just got notification that CIG Meadowlark, CIG plans to run a smart pig tomorrow on 25 miles of pipe.  It will cut CIG’s flow to us down to roughly 8-10 million for 4-6 hour period.  Normally our order is 37.8 million there.  I did call Rich Vargo with Bushton Products and tell him exactly what is to happen.  It will happen at 0730 in the morning and that the flows from CIG will be cut back 8-10 million for 4 hours Tuesday, 10/16.

Welsh



	1045
	LaCrescent
	Got a call from Dave Cammock.  He said the was finished with the outage at Tomah and he was raising the regulator pressure back up to 700#.  I raised my alarming values.

Calhoun



	1100
	Faribault
	Started the unit.

Calhoun



	1115
	Alexandria
	Put the unit on because our pressures were dropping off out at Benson.

Calhoun



	1125
	North Branch
	Put unit #2 on.  We finally started getting some suction and needed some gas up north as well as west.

Calhoun



	1128
	Portage 1B
	Got a call from Don Soppe.  He said that he was going to have to blow the run down.  He had to check his meter out there.  He will call us back when he put it back in service.  I inhibited the point.

Calhoun



	1136
	Pierz
	The unit went down. Our pressures dropped below NSP’s so the unit is trying to free flow right now.

Calhoun



	1200
	Oakland
	Talked to Joyce and asked her to put the “D” station on line.  We asked for three verticals.

Calhoun



	1200
	Palmyra
	Lost the high side turbine.  It was pumping “CD” to “D”.

Calhoun



	1215
	Galena
	Put unit #1 on with unit #2.  Our discharge pressure was down to 908 and we were still dropping off on the east.  Put Waterloo on earlier in the morning.  We starting to see the suction show up at Galena so we just wanted to keep the differential up and also get the discharge pressure up going east.

Calhoun



	1350
	Ventura
	Got a call from Wayne Kozisek.  He said he had problems with the program at the Ventura station.  He had to lower the setpoint pressure going to the “A” suction and put it in manual so it would catch it.  He was going to turn it back over to me.  He wanted me to raise the “A” suction from 430 back up to 465.

Calhoun



	1400
	Pierz
	Got an alarm on the unit when it went down.  I called Mitch.  He cleared the alarm and got the unit back on line.  Actually the unit didn’t come on but it started free flowing at the interconnect.

Calhoun



	1450
	Galena
	Sent a stop command to unit #1.  Ended up knocking both units down.  Had both of them down at 1510.  As soon as the unit #1 coasted down and reset I put a start back in it and it came right back on line.

Calhoun



	1500
	Palmyra
	Had them put the “A” line turbine on to the “B” suction.  Mike went out and put unit #27 on.  That was to help get off MAOP on the “A” line, plus it would help boost him to move a little more gas going north.  We needed it on the “B” suction.

Calhoun



	1500
	
	Got a call from Stacey at Ogden.  Said unit #14 will be down, not available until Thursday  They had an air aftercooler that was leaking into the air manifolds.  They had to take it apart to fix it.

Calhoun



	1535
	Palmyra
	Had Mike add another unit on the “C” suction to the “D” discharge.

Calhoun



	1630
	Redfield
	Called and talked to Brian.  He was running low on his suction pressure going out on the Redfield “B” that was actually free flowing into the St. Peter formation.  Told him that we were running a little short on suction at Ogden and didn’t look like we’d be able to get him anymore discharge for awhile so we’re just going to cut the total injection rate at Redfield back some more.  Decided to take them from 200 down to about 160.  That should help.

Calhoun



	1645
	Oakland
	Got a hold of the on call person Joyce.  She was going to go out and put the third vertical on.

Calhoun



	1700
	CC
	Got a call that they were done testing.

Calhoun



	1735
	Oakland
	Lost unit #18.  Got a call from Jeff who was at the station.  He was going to put it back on line.

Calhoun



	1740
	Ventura Interconnect
	Switched service codes from 1 to 2.  That’s for higher north flows.

Calhoun




1830 to 0630 Shift Mon., Oct. 15, 2001  (Braswell, Hall, Armitage, Ellington)

	1823
	Oakland
	Just got a call from Jeff.  He said the unit that went down out there went down on crankcase explosion.  It will be down for awhile.  He said he has sent a report into Jerry on it.

Hall



	1850
	Palmyra
	Got a hold of the OCC who got a hold of Mike.  Going to probably put one on “B” to “BC” with the other one that’s on as well as adding one “CD” to “D” with the turbine and two other verticals.  Also got a hold of the OCC for Beatrice.  We’ll either add one on the “D” line or put one on “C” to “C” there.  Right now I don’t have room to put it on “D” to “D” but we’ll see when he gets out there and we get some of the gas moving at Palmyra.

Hall



	2023
	Stevens Co. 6
	Unit 5 is down.  Ron called in.  He received an auto page.  He’s on his way out.

Ellington



	2139
	Hubbard
	Called OCC and they got the on call.  When I was talking to him it went ahead and did come up on the HP.  It is on line.  It had come up to 78 and then it went to zero and then I didn’t have anything.  The RPM’s weren’t even up at all.  The RPM’s are now 5625 so the unit is on line again.  Don’t know why it went down.  I had called Corn Belt and let them know I was going to restart it.  They said there was no storms or anything. 

Hall



	2150
	Stevens 6
	Unit 5 is back on line.

Ellington



	2335
	Calpine
	Received a call from Chuck.  He informed me that their unit that they’ve been having problems with all day came on for a little bit tonight.  Their nom is 71.8 million and right now they are spotting about 42.9.  They were up on their nom at one point tonight, about 71-72 million but now he was just calling to let us know that they were still having problems with their unit and they were down, probably will be down for 5-7 hours at the most.

Braswell



	0242
	WT1
	Received a unit 3 alarm.  Called the OCC for the on call.

Braswell



	0433
	WT2
	Unit 3 down. 

Braswell



	0505
	Clark Co. 1
	Unit 2 – We lost that unit.  Stacey is going out.

Moore




slb  


