
EXHIBIT "A"

Scope of Work

Instromet, Inc. and Enron Machine and Mechanical Services, Inc. Subcontract for the Fabrication of Meter Spools

· Procure material for fabrication of meter spools as per Instromet Specifications from Instromet's drawings and prints.

· Inspect material upon delivery to Enron Machine and Mechanical Services, Inc. (Subcontractor) Clifton, Kansas machine shop and ensure material meets specifications & that heat numbers and paper work match.

· Store material 


Pipe and flanges will be stored outside Subcontractor's machine shop facility


Nozzle material will be stored inside Subcontractor's machine shop facility

· Meter Spool Manufacture and Weld-up Per Instromet Design Specifications


Saw spool pipe to rough length


Machine spool pipe to finish length and turn OD to finish size


Weld flanges onto spool pipe according to Instromet specifications


Weld “weld-o-lets" in place on pipe


Install nozzles onto pipe and weld in place


Drill holes through pipe for transducer


X-Ray flange welds and magnaflux nozzle welds


Hone ID of pipe to specification


Inspect nozzle placement

Hydro test


Sandblast and Paint as per specifications

· Up and Down Stream Spools Manufacture Per Instromet Design Specifications


Saw spool pipe to rough length


Machine spool pipe to finish length


Weld flanges onto spool pipe according to Instromet specifications


Weld “weld-o-lets in place on pipe


X-Ray flange welds


Hydro test 


Sandblast and Paint as per specifications

· Fill out final inspection sheet for Instromet 

· Mark spools with correct serial numbers

· Package X-Ray film, mill test reports and hydrostatic test documents with each spool assembly

· Crate and deliver fabricated spools at Point of Delivery described in this Agreement for shipment  of spools byInstromet to Instromet's destination

1
Instromet 2000Jan24 SubK.doc


