[image: image1.png]Enron Wind



CONFIDENTIAL
Proprietary Information







DO NOT COPY  without

written   consent   from

Enron Wind

DRAFT
[image: image2.png]Enron Wind





Updated 3/14/02

Using the 1.5 turbine simulator we have verified that the following counters increment during the listed faults or turbine statuses:

Counter
Fault/status number
Description

System OK
1

2

3

6

20

21

87

94

109

110

147

152

153

161
166

181

182

183
184

216
-program start PLC

-no error

-manual stop

-system OK

-cable twisting right

-cable twisting left

-restart time delay

-control call

-generator carbon dust filter dirty

-speed in tower resonance range

-load shutdown

-obstruction light error/failure

-brush wear generator

-over voltage protection triggered*

-normal operation

-idling position

-start up

-load operation**

-shutdown

-obstruction light switch over to auxiliary lamps

Down time
13

14

15

16

17

19

22

23

25

35

45

52

57

59

60

63

64

65

71

75

76

77

81

82

86

88

97

98

99

100

101

102

103

105

106

107

113

114

115

118

124

125

129

134

141

142

144

145

176

177
-asymmetric generator current

-generator over speed

-rotor over speed

-rotor speed monitor

-centrifugal switch

-maximum motor power

-yaw drive end position

-peak value for excessive output

-no speed reduction with primary braking (pitch)

-emergency stop

-hydraulic pump time too long

-gearbox oil pressure too low

-ratio generator/rotor speed

-vibration switch

-yaw limit switch activated

-safety chain

-motor protection

-main switch activated

-gearbox oil soiled (shutdown)

-generator bearing A over temperature

-generator bearing B over temperature

-gearbox oil over temperature

-shaft bearing over temperature

-yaw motor over temperature

-brake pads worn, secondary brake

-multiple over temperature shutdown

-generator cooling air over temperature

-gearbox bearing over temperature

-generator 1 over temperature

-generator 2 over temperature

-motor protection fuse

-motor protection top cabinet

-generator 3 over temperature

-rotor CCU fault voltage

-rotor CCU fault current

-rotor CCU fault temperature

-line CCU fault voltage

-line CCU fault current

-line CCU fault temperature

-emergency stop top-box

-pitch overrun 0°

-pitch overrun 90°

-generator speed implausible

-battery charging voltage not o.k.

-rotor CCU collective fault

-line CCU collective fault

-blade angle asymmetry

-blade pitch control deviation

-drive train vibration

-tower vibration

Line out time
8

116

117
-over voltage

-line fault voltage

-line fault frequency

Weather out time
7

78

146
-storm shutdown

-outside temperature too low

-turbine stopped due to calm

External stop time
58

172
-demand to stop by an external device

-ice sensor alarm

Maintenance time
47

108

148

155

171
185
186
187
188
189
191
-hydraulic service switch top cabinet

-rotor locking

-generator fan service switch

-maintenance

-service key brake release*

-manual operation pitch*

-manual operation speed*

-manual operation torque*

-manual operation, line separation*

-manual operation yaw*

-manual operation generator fan*

Repair time
156
-repair

* - Fault/status 161, 171, 185, 186,187, 188, 189, 191 increment counters that may not be correct

** - Status 183- “load operation” also increments the “operational time” counter

Faults/turbine statuses that cannot be duplicated on the simulator:

Counter (theoretical)
Fault/status number
Description

System OK
4

5

44

89

95

138

162

165

167

169

170

173

193

199

200

204
-remote stop

-remote start

-hydraulic filter polluted (warning)

-gearbox oil dirty (warning)

-PC restart

-temperature warning pitch motor

-fault battery charge switching

-service switch pitch

-voltage limitation activated

-waiting for gearbox oil pressure

-increasing speed with falling blade angle

-low-noise operation

-marine obstruction light (offshore only)

-test fault CCU cooling system

-pre-heating CCU cooling system active

-FastBus service mode

Down time
24

27

28

31

34

36

38

41

42

43

46

51

53

54

55

56

62

83

112

119

120

121

122

123

130

131

132

133

135

135

136

137

139

140

143

149

150

151

154

157

163

180

190

194

195

196

197

198

201

202

203

205

206

207

208

209

212

213

214
-primary braking time too long

-secondary braking time too long

-no speed reduction with secondary braking

-timeout yaw counter

-brake not released secondary brake

-speed with braked turbine

-secondary brake test

-vibration sensor defect

-wrong yaw drive direction

-hydraulic filter polluted (switch off)

-hydraulic pump (interval) time too short

-gearbox oil level too low

-wind vane defect

-anemometer defect

-rotor pulse sensor defect

-generator pulse sensor defect

-PLC fault collection

-gearbox bearing B over temperature (offshore only)

-response signal from CCU

-pitch controller timeout

-communication fault pitch controller

-axle disturbance 1 pitch controller

-collective fault pitch controller

-gearbox oil cooler switch contact

-braking time blade 1 too high

-braking time blade 2 too high

-braking time blade 3 too high

-battery charging rotor blade drive
-battery charging rotor blade drive

-battery test

-limit switch rotor blade 0° defect

-limit switch rotor blade 90° defect

-over temperature pitch motor

-motor protection pitch motor

-pulse sensor rotor monitor defect

-axle 2 fault, pitch controller

-axle 3 fault, pitch controller

-response circuit breaker generator brush

-generator cooling water pressure/level not o.k.

-brake does not close

-torque deviation

-wind deviation

-temperature error container

-by-pass filter soiling

-gearbox pressure loss by-pass filter

-pre-heating fault CCU cooling system

-over temperature CCU cooling system

-pressure fault CCU cooling system

-pre-heating CCU cooling system active

-PLC module failure

-fault temperature measurement

-over temperature top box (EW900 only)

-gearbox pressure too high (EW900 only)

-ground fault LVMD (EW1.5/70.5(60Hz) only)

-no activity CAN-Bus CCU

-no activity CAN-Bus MFR13

-battery voltage low- axis 1

-battery voltage low- axis 2

-battery voltage low- axis 3

Line out time
9
-under voltage

Weather out time
84
-gearbox oil temperature to low

External stop time



Maintenance time
164
-maintenance switch pitch

Repair time



4- remote stop

5- remote start

9- under voltage

24- primary braking time too long

27- secondary braking time too long

28- no speed reduction with secondary braking

31- timeout yaw counter

34- brake not released secondary brake

36- speed with braked turbine

38- secondary brake test

41- vibration sensor defect

42- wrong yaw drive direction

43- hydraulic filter polluted (switch off)

44- hydraulic filter polluted (warning)

46- hydraulic pump (interval) time too short

51- gearbox oil level too low

53- wind vane defect

54- anemometer defect

55- rotor pulse sensor defect

56- generator pulse sensor defect
62- PLC fault collection

83- gearbox bearing B over temperature (only offshore)

84- gearbox oil temperature to low

89- gearbox oil dirty (warning)

95- PC restart

112- response signal from CCU

119- pitch controller timeout

120- communication fault pitch controller

121- axle disturbance 1 pitch controller

122- collective fault pitch controller

123- gearbox oil cooler switch contact

130- braking time blade 1 too high

131- braking time blade 2 too high

132- braking time blade 3 too high

133- battery charging rotor blade drive

135- battery test
136- limit switch rotor blade 0° defect
137- limit switch rotor blade 90° defect
138- temperature warning pitch motor
139- over temperature pitch motor
140- motor protection pitch motor

143- pulse sensor rotor monitor defect

149- axle 2 fault, pitch controller

150- axle 3 fault, pitch controller

151- response circuit breaker generator brush

154- generator cooling water pressure/level not o.k.

157- brake does not close

162- fault battery charge switching

163- torque deviation

164- maintenance switch pitch

165- service switch pitch

167- voltage limitation activated

169- waiting for gearbox oil pressure

170- increasing speed with falling blade angle

173- low-noise operation

174- over temperature transformer warning (offshore only)

175- over temperature transformer disturbance (offshore only)

180- wind deviation

190- temperature error container

192- wrong parameter check sum 

193- marine obstruction light

194- by-pass filter soiling

195- gearbox pressure loss by-pass filter

196- pre-heating fault CCU cooling system
197- over temperature CCU cooling system
198- pressure fault CCU cooling system

199- test fault CCU cooling system

200- pre-heating CCU cooling system active
201- PLC fault 24V supply

202- PLC module failure

203- fault temperature measurement

204- FastBus service mode

205- over temperature top box (EW900 only)
206- gearbox pressure too high (EW900 only)
207- ground fault LVMD (EW1.5/70.5(60Hz) only)

208- no activity CAN-Bus CCU
209- no activity CAN-Bus MFR13
212- battery voltage low- axis 1
213- battery voltage low- axis 2
214- battery voltage low- axis 3

215- direction dependent shut-down

217- end of parameter state service

218- frequent cut-ins

219- Ice sensor fault

220- external Stop caused by curtailment

221- external power limitation







Page 4 of 5
Memo- Counters incrementing by fault.doc

_1023275158.unknown

