[image: image1.png]E CALIFORNIA ISO




[image: image2.png]E CALIFORNIA ISO



OMAR-Online 
Meter Data File Submission
Automation Specification

REVISION HISTORY

REVISON NO.
DATE
DESCRIPTION

1.0
08.30.2001
Initial composition of specification. Draft version.

1.1
02/05/2002
Revised for system name change, url changes, change from  smartcard to software digital certificates.

TABLE OF CONTENTS

11.
Overview

1.1
Purpose
1
1.2
General Application Requirements
1
1.3
Security requirements
2
2. 
Manual Processes
2
2.1
Initiate Connection
2
3.
File Submission Automation
3
3.1
Make URL Connection
3
3.2
Multipart/form Data Request
3
3.3
Response
4
3.4
Timing considerations
4


1.
Overview

1.1
Purpose

This document specifies how an automation tool can access and interact with the California ISO's metering data application, OMAR-Online.  The document outlines how an automation tool can communicate with OMAR-Online to submit meter data files.

Two other documents are available that specify how an automation tool can access file submission status and download meter data files.

An automation tool for submitting meter data is not provided as part of the OMAR-Online application.  This document specifies how a Market Participant's automation tool would need to interact with the OMAR-Online application to submit meter data files.

1.2
General Application Requirements

A tool developed to perform automated tasks with the OMAR-Online application must meet the following general requirements.

1.2.1
SSL Communications

The automation application must perform Secure Socket Layer (SSL) communications with the OMAR-Online application.  

1.2.2
Cookies

The automation application must support cookies.

1.2.3
Software Digital Certificate

The OMAR-Online application uses software digital certificates for user authentication.  The software digital certificate must be properly installed in the Netscape browser on the workstation used to access the OMAR-Online application. The software digital certificate provides authentication for all data submitted to OMAR-Online.

Reference RFC 2459 - Internet X.509 Public Key Infrastructure Certificate and CRL Profile for more information on certificates and PKI authentication.

1.2.4 Internet Browser

The OMAR-Online application must be accessed via Netscape Navigator versions 4.5 through 4.78. (US Domestic 128 bit encryption).

1.3
Security requirements

Due to the sensitive nature of the meter data, the security requirements are even more stringent for users with extended login sessions.  OMAR-Online users needing extended logon sessions must sign the Network Connectivity Security Agreement form in order to assure CAISO they are taking the appropriate precautions to protect the connecting device.  Any company connecting directly to any CAISO network must sign this form.  The Information Security Department, upon request by Market Participants identifying they require extended logon sessions, will send them the Network Security Connectivity Form.
2. 
Manual Processes

The secure connection to the OMAR-Online application must be established as a manual process.  

2.1
Initiate Connection

This process is manual.  Your particular browser settings may require additional steps. 

A user with a valid software digital certificate must login to the OMAR-Online application.

Note:  If you currently use a smartcard to access the production system, MDAS-Online, remove the smart card from the reader prior to accessing the OMAR-Online test system.

1. Open an instance of Netscape Communicator (see requirements in section 1.2.4.).  

2. Make a SSL connection (https) to the following PRODUCTION URL to create the login session:
https://omar-online.caiso.com

Or, make an SSL connection (https) to the following TEST URL to create the login session:
https://omar-onlinetest.caiso.com
3. Your browser security may require you to acknowledge a Certificate Name Check.  If so, click on the Continue button.

4. In the “Select a Certificate” window, select your OMAR–Online software digital certificate and click on Continue.

5. If your browser certificate database is password protected, you will next be prompted to enter that password.

The login session will be established if the user has selected a valid certificate.  The login session will terminate if the browser is closed or power to the system being used is interrupted.

3.
File Submission Automation

This section defines the process used for submitting a meter data file through the OMAR-Online application using a Market Participant's automated tool.  

3.1
Make URL Connection

Make connection to webserver and call the upload meter data file program:

https://omar-online.caiso.com/uploadProcess.jsp

3.2
Multipart/form Data Request 

Information on multipart/form data can be found in RFC 1867.

Send a request packaged in multipart/form data with the following restrictions. 

Here is an example of how the HTML form for file submission would look:

<FORM METHOD="GET" ACTION="uploadProcess.jsp" ENCTYPE="multipart/form-data">

Upload File

<INPUT TYPE="FILE" NAME="datafile" SIZE="50">

   <INPUT TYPE="SUBMIT" VALUE="Upload">

</FORM>

3.2.1
File Restrictions

For the file NAME field - use 'datafile'.

Only one meter data file can be uploaded at a time.

Files must be less than 500KB 

Files must have one of the following file extensions - .csv or .dat 

Uploaded filenames must not exceed 40 characters in length, including the ".csv" or ".dat"

3.3
Response 

An HTML page will be presented as a response to the file submission.  The page can be parsed to get the file ID, the file size, the source file name.

Here is an example of what will be presented in the response page.  This is provided as an example only, the production version of this page may differ slightly from this format.

<table>

        <tr><td class=formgray>

        myfile.csv  Meter file uploaded and Saved As renamedmyfile.csv

        </td></tr>

        <tr><td class=formgray> File Size : 13133

        </td></tr>

        <tr><td class=formgray> File Id: 1 (save this file id for future reference and file status inquiries)

        </td></tr>

</table>

3.4 Timing considerations

Once a data file is successfully submitted, please allow time for the file's contents to pass numerous validations before the data is truly loaded into our system.  Another consideration involves CAISO's internal system interfaces.  Data that has been submitted, but not internally loaded may be delayed due to other processes, such as the daily push of meter data to the Settlements system.  This could delay final processing of a submitted meter data file by approximately one hour.  Another example of delayed processing involves numerous Market Participants submitting many data files within the same timeframe.

These timing considerations are important if your system also automates the download of Meter Data File Status information.  Please see the OMAR-Online Meter Data File Status Automation Specification for details.

