Pacific Gas & Electric PX Credit Calculation


The hourly PX price (PX) was determined (from April 17, 2000 through January 19, 2000) by multiplying the forward market cost (FM) adjusted for certain factors by the allowance for uncollectibles (U) and by the distribution loss factor (DLF). 

PX (day n, hr) = [FM (day n, hr) + Adj (day n-91, hr) + PSA + RT Est Adj (day n-7, hr)] * U (day n, hr) * DLF (day n, hr) 



The forward market cost (FM) is determined as the sum of all energy costs (a combination of hourly ISO costs and the appropriate Block Forward costs) plus the PX administrative costs (PXAdm) and the ISO grid management charge (GMC). 

FM (day n, hr) = ([HC (day n, hr) + BFC (day n, hr)] / SL) + PXAdm (day n, hr) + GMC (day n, hr)

HC = Hourly Market Cost
HC = MCP (day n, hr) * (SL – BFL)

SL = System Load

BFL = Block Forward Load

BFC = Block Forward Cost (including Block Forward administrative costs)

PXAdm = $0.3064/kwh (as of June 10, 1998)

GMC = $0.8300/kwh (as of January 1, 2000) 

Adj = Difference between the Settlement Cost for each hour and the FM cost

Adj (day n-91, hr) = [T (day n-91, hr) + PXRT Admin (day n-91, hr)] – [{FM (day n-91, hr) * FL (day n-91, hr)} / BL (day n, hr)]

T = Total Settlement Charges ( includes ISO management, black-start capability, voltage support)

FL = Final Settlment Load (MWh)

BL = Bid Load (Mwh)

PXRTAdmin = Estimated positive deviation in metered load relative to day ahead load multiplied by $0.3038

PSA = Post Real Time Settlement Adjustment 

PSA = (PSA$ accrued / totaldays month) / DL givenday

DL = Daily Scheduled Loads

RT Est Adj = Adjustment for Estimated Real Time Settlement Costs

RT Est Adj (day n-7, hr) = Est RT$ (day n-7, hr) / Est FL (day n, hr)

DLF = the distribution loss factor defines three distinct hourly PX prices – 

(T) Transmission Voltage, (P) Primary Voltage, (S) Secondary Voltage.

U = 1.003396 (as of March 1, 2000)

�   The subscript "day n, hr" denotes that this calculation is performed for each hour of day "n". �





