3-A-VI. Lincoln Energy Center - Will County, Illinois 

MAIN Region

[image: image1.jpg]



Overview - Lincoln Energy Center

Description: 
656 MW (nominal) natural gas-fired, simple cycle power generation facility (the “Lincoln Energy Center”) owned by Des Plaines Green Land Development, L.L.C., an indirect wholly owned subsidiary of Enron North America. The Lincoln Energy Center consists of eight General Electric Model 7EA gas turbines, each rated at approximately 82 MW (nominal).

Location:
The Lincoln Energy Center is located on 50 acres of land at 27155 South Kankakee Street in Manhattan, Illinois, directly across from the Commonwealth Edison Wilton Center substation (“ComEd Substation”).

Constructor:
NEPCO, an indirect wholly owned subsidiary of Enron Corp.

NERC Region:
MAIN-ComEd

Control Area:
ENLC. The Lincoln Energy Center has been designated a control area in accordance with NERC policy.

Power Purchase Agreement:
The Lincoln Energy Center has a limited term commitment for the plant’s capacity and energy.

Transmission:
The Lincoln Energy Center is interconnected directly to the ComEd Substation at the 345 kV level. The ComEd Substation has five transmission lines to which it is connected: three 345 kV lines and two 765 kV lines. Each of the 765 kV lines has significant available transmission capacity during periods of peak load. In addition to the direct access to Commonwealth Edison's service territory, the ComEd Substation has direct access to eastern markets (such as AEP) via the 765 kV line.

Fuel:
The Lincoln Energy Center is permitted to use natural gas as its sole fuel source. The plant is interconnected to a natural gas pipeline system owned by Northern Border Pipeline Company (“Northern Border”).  Des Plaines Greenland Development, L.L.C. and Northern Border each own, operate and maintain specified portions of the interconnection facilities as defined in the interconnect agreement. Natural gas transport delivery to the Lincoln Energy Center is achieved through Northern Border’s pipeline pursuant to an interruptible gas transportation agreement with Northern Border.  The term of the interruptible gas transportation agreement began on March 1, 2000 and continues until February 28, 2002.

Zoning:
To develop the Lincoln Energy Center, the plant site was re-zoned as industrial.

Air Permit:
The Lincoln Energy Center received its PSD air permit in September 1999. 

Baseload Emission Information

NOx Cap (Per Unit):

< 9 PPM

CO (Per Unit):

< 25 PPM

Commercial Ops:
June 2000

Ops Hours:
The Lincoln Energy Center is permitted for 3,250 hours of annual operation at full load.

Heat Rate:
At 90 degrees Fahrenheit and at full operation, the plant is able to achieve a net heat rate of approximately 11,900 Btu/kWh (HHV).

Peaking Capacity:
At 90 degrees Fahrenheit and at full operation, the capacity of the plant is approximately 632 MW.

Expansion Potential:
The Lincoln Energy Center has been designed to facilitate a future expansion or conversion to combined-cycle.

Anticipated Water Usage:
175 gpm during operation.

Water Supply:
On-site wells.


Site Pictures - Lincoln Energy Center
[image: image3.png]d
superion
= wi
WEP Ay WLaP oy
pat
commonweaki—7-
ied i

N.W.
DisON wisEoNsIN

NORTHERN STATES
POWER (W1)

ver,
R

SCONSIN
c. POWER

WISCONSIN PSC

wesc
NsPw1 West
Ohoaton WiISCONSIN
f PaL ;
G Weston
apin R wep Polliom
U
WISCONSIN
Shechie NsPw1 RowIRiR T\t
O French tsland LIGHT o
s /
@ Cenco 4 wer
cenTRAL Columbia 5
ILLINoIS Sacicng)
opc wep
psc e ¥
INTARSTATE POWER MADISON Gar—— HOL® o
weL
Doveey
3
hoe
s
wane Amold
O st £2
. vrics
= op!
- Crook
.
= & e
oW @ciE
ST no®  Fair
g
Lovisa
CENTRAL
ILLINGIS  TLLINOIS
POWER LIGHT
1ES UTILITIES .
cENTRAL IL
Sl psc
L
Duck Creok g Powerton
b e,
R fLpc  Vemilon®
uTiLicore
CENTRAL ILLINOIS PSC
Sy
cirs seRL
Marodosia ® Dallman @ bash
Kansas
- a
Kincal Hotsonil
s
ot
s
[ Coffoon
g | River cenpAhL it
NN sc
. | oum Yol ILLINOLS
o | PowER
T e . Al
X Hawthom UTILICORP Vermon
s’ 9 * e
d  Wearndory VT
e
cirs
Grand T
aban
C]

'Mid-American
~Interconnected
 Network (MAIN)

usceL
Oyl Shoals

werER
PENINSULA]
POWER





Plant View (7/17/00)

[image: image2.jpg]



Turbine Air Inlets (7/17/00)




























































































































































































































































Substation





ComEd





345 kV line to:





345 kV





GSU = Generator Step-up Unit





B = Breaker





B





B





B





Chicago LDC's





Gas Daily Index:





Northern Border





Gas Transporter:





88





82





79





Unit MW Rating:





Winter





Nominal





Summer





B





GSU T31





GSU T32





GSU T33





GSU T34





B





B





B





B





B





B





B





B





G2





G1





G8





G7





G3





G6





G5





G4





ENLC





LINCOLN CENTER











Lincoln Energy Center





Lincoln Energy Center


























