PLUG & ABANDONMENT PROCEDURE

FOR

NAPOLEONVILLE STORAGE #1

WELL DATA:

TUBLARS

WEIGHT

DEPTH

DESCRIPTION

    16”


122.15 #/FT.

2,009 FT.

LAST CEMENTED










CASING

    8 5/8”

  36 #/FT.

3,211 FT.

HANGING STRING

PROCEDURE:

1. Submit Work Permit to Department of Natural Resources (DNR).  Contact DNR at least 24 hours prior to plugging operations.

2. Displace natural gas by filling cavern with brine or freshwater.

3. Circulate until gas is cleared from wellbore.

4. Rig up workover rig and remove top section of the wellhead components.

5. Rig up preventers and pull hanging string.

6. Pick up bridge plug and set at 2,000 feet.

7. Fill up 16” casing from the bridge plug to the surface with cement, using the circulation method.

8. Cut the 16” off approximately 3 feet below ground level and plate over.  Weld into the plate the Serial Number of the well and the date.

9. Prepare and submit appropriate government reports, Form P & A.

