Bullet Points on California Energy Market:

California Power Exchange (CalPX)

1. Determine market clearing price for supplies bid to serve load in California

a. Administers Spot-market (Day-ahead and Hour-Ahead)

b. Provides block-forward products as well (not very liquid)

2. Acts as the scheduling coordinator for PG&E, SCE and SDG&E to the ISO (submits balanced supply/demand schedule)

3. Buy/Sell Mandate-CPUC mandated that all utility-owned/controlled generation must be sold to the PX and all supplies to serve load must be purchased from the PX.

California Independent System Operator (Cal ISO)

1. Created by enabling legislation AB 1890, September 1996

2. Utilities submitted control, but not ownership, of transmission lines to ISO

3. Munis participate on a voluntary basis

4. The ISO is governed by a 26-member stakeholder board (utilities, marketers, consumers, munis and CEO)

5. Tasked with ensuring reliable operation of transmission grid in a restructured market

6. Accepts balanced supply/demand schedules from scheduling coordinators

7. Ensure adequate supply to meet demand on a real-time basis

a. Administers imbalance market

b. Seeks “out-of-market” (OOM) purchases if deficit

8. Conduct ancillary services auction 

9. Conduct congestion auction-method of allocating transmission capacity

FERC Draft Order Issued 11/1/00

1. Increase in electricity prices are result of market structure, not illegal price manipulation.

2. Comments due 11/22/00

3. Eliminate mandatory buy/sell to CalPX.  Encourage more forward contracting.

4. Require market to schedule 95% of transactions in day-ahead markets to reduce chronic underscheduling of load and generation.

5. Establish a “soft” cap of $150.  Bids can exceed cap with either marginal cost or opportunity cost justification.  Highest bid will no longer set the clearing price for all bids.

6. Establish new independent, non-stakeholder governing boards for PX and ISO.

7. Establish generation interconnection procedures.

Order said that in order to keep the lights on and protect consumers against high prices, California needs:

1. additions of new generation and transmission capacity

2. demand response programs

3. adequte reserve requirements

4. to allow load serving entities to buy forward.

