WORKSPACE Cheat Sheet

Workspace can be found on the private site at https://iso_si/iso/
Login is epmi, password will change periodically

Only fields necessary to fill or check in appear below…all other fields or steps should be ignored.

1. Checking Phase II

Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

ACTIONS for Current Work Area + Selected Information:

X Submit Schedules for Phase II Validation

Click on SUBMIT ACTION button.

2. Creating Load Schedules
Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Commodity:  X LOAD

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Load Schedule-Hourly

Load ID: Desired Load Region (SCE1, PGE1, etc.) Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate

Load (MW):Input Load  Cong Mgt Flg: Yes/ No-yes if you are making congestion bids


Large empty rectangle is for congestion bids.  

If importing congestion bid is-((Load MW),$),((Load MW+import MW),$).

If exporting congestion bid is-((Load MW-export MW),$),((Load MW), $).

Click on SUBMIT button.

3.  Creating Internal Interchange Schedules (SC to SC trades)

Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Commodity:  X INTER SC TRADES

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Inter SC Trades Schedule-Hourly

Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate 

Trading SC: Desired Trade SC (NES1, WESC, etc.)

Point of Interch: (SP15, NP15, etc.)

Sched Type: (Energy, etc.)

Type of Interchange: (Fixed, etc.)

Direction: (Send-selling,Receive-purchase)

Cong Mgt Flg: Yes/ No-yes if you are making congestion bids

Hrly MW: (Amount of sale)

Large empty rectangle is for adjustment bids and will not usually be necessary.  
Click on SUBMIT button.

4. Creating Energy Import or Export Schedules

Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Individual Interchange:  X Energy Interch

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Hourly Individual Interchange Schedule

Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate 

Tie Point: (Import/Export Tie Point-Can be gotten from CAPS)

Energy Type: (Firm for import, Non-Firm for export)

Ext Cntrl ID: (CA at Tie Point-Can be gotten from CAPS, not necessary)

InterChange ID: (EPMI_CISO_REALTIME)

Output (MW): (MW import/export)

Cong Flag: (Yes/No-yes if you are making congestion bids)

Cap Res Price($): (Only necessary for ancillary services)
Ramp Rate (MW/min): (necessary for ancillary services)

Adjustment Bids (MW, $): ((0,$),(MW,$)) necessary for import/export or ancillary services

Click on SUBMIT button.

5. Creating Generation Schedules
Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Commodity:  X Generation

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Generation Schedule - Hourly

Unit ID: (Enter Unit ID-Can be gotten from CAPS)

Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate 

RRate (MW/min): (Enter Ramp Rate)

Output (MW): (Enter Output)

Click on SUBMIT button.
6. Creating Supplemental Energy Schedules (in California) For Generators

Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Supplemental Energy Bids: X Generation

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Supplem Energy Bids Gen Schedule - Realtime

Unit ID: (Enter Unit ID-Can be gotten from CAPS)

Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate 
Ramp Rate (MW/min): (Enter Ramp Rate)

Op Lims (MW, MW): ((MW,MW))-limits for inc or dec

Large empty rectangle is for bids,(MW,$),(MW,$) for example,

to dec by 10 mws for $25, the bid would be (-10,25),(0,25) 

to inc by 10 mws for $25, the bid would be (0,25),(10,25)

Click on SUBMIT button.

7. Creating Supplemental Energy Schedules (at Tie Points)

Current Work Area:

Market: Hour Ahead/Supplemental-1hr
Date: Current Date
Hour: Current Hour

SELECT TYPE OF INFORMATION:

Schedule Classification: X PREFERRED

Individual Interchange: X Supplem Energy Bids

ACTIONS for Current Work Area + Selected Information:

Click on Create Schedule button


New screen appears: Supplem Energy Bids (Imports)
Market: Hour Ahead-1 Hour 


Check transaction date and hour to see that it is appropriate 

Tie Point: (Tie Point for supplemental transaction-Can be gotten from CAPS)

InterChange ID: (EPMI_CISO_REALTIME)

Ramp Rate: (typically 1)
Pre Dispatch Flag: (MUST be YES!)
Adjusted Bids: (Enter bids for supplemental incs or decs here

to dec by 10 mws for $25, the bid would be (-10,25),(0,25) 

to inc by 10 mws for $25, the bid would be (0,25),(10,25)

Click on SUBMIT button.
