SoCalGas

 “We Don’t Do Windows”

Purpose:

· Reform the SoCalGas “windowing” procedures by eliminating the receipt point specific calculation and open the SoCalGas system based on customer requests.

Requirements:

1. Provide enhanced customer access to desired receipt points.

2. Maintain SoCalGas’ operational control of its system.

3. Ensure customers are responsible to deliver natural gas approximately equal to their consumption and do not exceed the appropriate balancing levels.

4. Actions by SoCalGas conform to current tariffs or tariffs on file for approval with the CPUC.

Planned Changes and Continued Practices:
I. Windowing Procedures

1. SoCalGas will no longer “window” its receipt points.  Instead, the capacity limit for each receipt point will be equal to the Operational Maximum as determined by SoCalGas’ Gas Control, subject to maintenance, outages or other emergency situations.

2. For Wheeler Ridge (“WR”), SoCalGas will calculate each interconnecting pipeline’s Operating Maximum for an entire flow day as a percentage share of the total WR capacity, using the most current final (Cycle Four - Intraday 2) scheduled deliveries

3. Customers are responsible for selecting receipt points and delivering only those quantities they intend to burn or store. 

II. Over-Nominations (OFO) Calculation

1. SoCalGas will continue to determine if an over-nomination event shall be called based on total prior scheduled volumes verses total system available capacity.

2. Over-Nominations will now be called Operational Flow Orders (OFO) to be consistent with industry practice.

3. An OFO event can be called prior to any of the four nomination cycles.

4. Once an OFO is called, it will remain in effect for the entire day.

5. OFO Calculation Procedure:

a) Over-nomination status will be calculated before each cycle, using the most current scheduled deliveries, and when declaring an OFO provide shippers at least two hours notice prior to nomination deadlines.  


b) If we don’t have scheduled deliveries for one or more pipelines for a given cycle, SoCalGas will use the most recent scheduled deliveries available. 


c) SoCalGas will eliminate HUB and interruptible storage injection nominations during an OFO event.

d) After receipt of customer nominations, SoCalGas will continue to reduce customer nominations in the current manner if customer changes don’t result in the appropriate reductions.

e) The elapsed pro rata rules will apply to Cycle Three - Intraday 1 and Cycle Four - Intraday 2.

f) SoCalGas will continue to accept customer requests to modify usage on OFO days based on expected rather than historical information. Customer requests must be submitted on the appropriate form for each day.

5) SoCalGas’ Gas Acquisition Department will continue to abide by the same rules as all other transportation customers.

6) If scheduled volumes are greater than available capacity even after an OFO has been declared and SoCalGas has reduced nominations to expected usage plus 10%, SoCalGas will reduce receipt capacity for each of the connecting pipelines on a prorata basis.

III. Nomination/Scheduling Changes

5) SoCalGas will change its nomination practice of only rolling over scheduled volumes to rolling over customer confirmed nominations from cycle to cycle.  This process will take place during the confirmation process on each cycle, including OFO days.  


6) Customers can enter new nominations for Evening, Intraday 1, or Intraday 2 only after confirmation process is complete and nominations for that cycle have been created.

1
3

