


======================================================================





Control Number: 9800110





    Category:  NSPS


    Region:    Region 3


    Date:      07/02/1998


    Title:     Alternative Monitoring Approval


    Recipient: Faggert, P.F.


    Author:    McGuigan, David


    Comments:  


               





    Abstract:





    Q: Will EPA approve a custom fuel monitoring schedule,


    waive fuel nitrogen monitoring requirements, and approve


    the use of an alternative location for fuel sampling?





    A: Yes, EPA will approve these alternatives under certain


    specified conditions.





    Letter:





    In Reply Refer To: 3AP13





    CERTIFIED MAIL 


    RETURN RECEIPT REQUESTED





    P.F. Faggert


    Team Leader


    Environmental Regulations and Permits


    Virginia Power


    Innsbrook Technical Center


    5000 Dominion Boulevard


    Glen Allen, Virginia 23060





    Re: Various Alternative Requests under NSPS Subpart GG





    Dear Ms. Faggert:





    The Philadelphia Regional Office of the U.S. Environmental


    Protection Agency (Region III) has received and reviewed


    your requests, dated April 29, 1998, for alternative


    methodologies/procedures, as provided under the New Source


    Performance Standards (NSPS) program, to be applied to


    stationary gas turbines subject to Subpart GG requirements


    at your Darbytown Station.  You had requested alternatives


    to three separate Subpart GG conditions, namely: an


    alternative, custom fuel monitoring schedule for


    pipeline-quality natural gas fuel; a waiver of the


    nitrogen-in-fuel  monitoring requirement; and, finally, an


    allowance for obtaining fuel samples for analysis at a


    different location in the gas pipeline in order to minimize


    the sampling/analysis necessary for compliance with Subpart


    GG at a number of gas turbine stations. In regard to the


    first two requests, EPA does have a National Policy, as you


    indicated, which allows affected facilities that combust


    pipeline-quality natural gas fuel to monitor the fuel


    sulfur content at a reduced rate, as specified in the


    Policy and upon approval by EPA, and to be granted a waiver


    from the fuel nitrogen monitoring requirement as natural


    gas fuel has been determined to contain no fuel bound


    nitrogen and having free nitrogen which does not contribute


    appreciably to the formation of Nox emissions.





    As your Darbytown Station turbines combust pipeline quality


    natural gas fuel and the fuel quality has been established,


    based on EPA's current understanding of the situation, as


    equivalent to that utilized by the Doswell Limited


    Partnership turbines which are already on a custom fuel


    monitoring schedule, EPA, hereby, approves the use of the


    fuel sulfur monitoring schedule contained in EPA's August,


    1987 Policy for the Darbytown Station turbines and waives


    the fuel nitrogen monitoring requirement for your natural


    gas fuel as allowed under the same Policy.





    EPA is also, hereby, approving the use of natural gas


    samples taken at your Chesterfield Power Station, for


    analytical purposes, to fulfill the fuel analysis


    requirement for the Darbytown turbines as the two stations


    both combust identical fuel from the same gas pipeline.  As


    you have indicated, EPA had approved a similar sampling


    allowance for Transcontinental Gas Pipeline Corporation


    under specified conditions which will now apply to Virginia


    Power as well, namely, that your Company will supply one


    month of data, prior to the initiation of the reduced


    monitoring schedule, from daily fuel monitoring to verify


    the sulfur content of the fuel and that Virginia Power will


    follow the schedule specified in EPA's August 14, 1987 memo


    regarding custom fuel monitoring schedules under NSPS


    Subpart GG.  It is EPA's understanding, based on our review


    of the pipeline schematic supplied by Virginia Power, that


    Doswell, Darbytown and Chesterfield Stations utilize the


    same fuel supply for turbine operation and that no other


    piping is introduced into the pipeline system at or between


    these described Stations which would have the effect of


    mixing any other fuel or material into the natural gas fuel


    being combusted in the turbines so as to alter the fuel


    analysis between the mentioned Stations.





    If you should have any questions or comments in regard to


    these approvals, do not hesitate to contact me, at (215)


    814-2158, or James W. Hagedorn, of my staff, at (215)


    814-2161.





                             Sincerely,











                             David B. McGuigan, Ph.D., Chief


                             CAA Area II Enforcement Branch





    cc: Charles King, VaDEQ-Piedmont Regional Office


    Jordan Spooner, OECA

















======================================================================





Control Number: 9800109





    Category:  NSPS


    Region:    Region 3


    Date:      05/27/1998


    Title:     Waiver of Nitrogen Monitoring


    Recipient: Hoffman, P.M.


    Author:    McGuigan, David


    Comments:  


               





    Abstract:





    Q: Will EPA approve a waiver of the fuel nitrogen


    monitoring requirement under NSPS GG?





    A: Yes, under certain circumstances a waiver can be


    granted under current EPA National Policy.





    Letter:





    In Reply Refer To: 3AP13





    CERTIFIED MAIL


    RETURN RECEIPT REQUESTED





    Mr. P.M. Hoffman


    Environmental Coordinator


    Columbia Gas Transmission


    1700 MacCorkle Ave. SE


    Charleston, West Virginia 25314





    Re: Approval of Nitrogen Content Monitoring Waiver Under


    NSPS Subpart GG





    Dear Mr. Hoffman:





    The Philadelphia Regional Office of the U.S. Environmental


    Protection Agency (EPA), Region III, has received and


    reviewed your letter, dated April 30, 1998, requesting a


    waiver of the NSPS Subpart GG requirement for fuel nitrogen


    monitoring under Section 60.334(b) at the stationary gas


    turbines in the Goochland Compressor Station.  Based on the


    statements made in your referenced letter, EPA, hereby,


    grants your requested waiver as the Goochland gas turbines


    only combust, and are only allowed to combust, pipeline


    quality natural gas fuel.  By Policy, dated August 14,


    1987, EPA has already determined that nitrogen monitoring


    can be waived for pipeline-quality natural gas since there


    is no fuel bound nitrogen and the free nitrogen does not


    contribute appreciably to Nox emissions.





    If you should have any comments or questions in regard to


    this matter, do not hesitate to contact me at (215)


    566-2158 or James W. Hagedorn, of my staff, at (215)


    566-2161.





                                  Sincerely,











    David B. McGuigan, Ph.D., Chief CAA Area II Enforcement





    cc: Cheryl Mayo, VaDEQ-Piedmont Regional Office




















======================================================================








