# Question 2

Please provide a simple and clear example to explain the operation of the Ancillary Services Markets for Regulation Up, Regulation Down, Responsive Reserve, and Non-Spinning Reserve.  In particular, please emphasize on the following areas:

- Determination of MCPC for each AS product

- Determination of MCPC if there are zonal differences in ERCOT for each AS product

- Determination of MCPE if each of these AS products are deployed

- Reliance on optimization models, if any, to select AS (succinct description of the model)

- Actions to be taken if there is insufficiency of bids

## Answer

The market operation for each of the four ancillary services in this question is identical. The amount of requirement and the prices may be different, but the methodology will be the same.

This example uses 4 QSE’s, each with a different set of circumstances and market philosophy. The assumptions below describes each QSE.

## Assumptions

QSE 1:

This QSE represents generation only with no Load Serving Entities. It makes bilateral sales and bids in ERCOT run ancillary services markets. Because it does not represent loads, it has no ancillary service requirements except for sales of ancillary services.

QSE 2:

This QSE represents Load Serving Entities only with no generation. It buys its energy needs through bilateral contracts. It has an ancillary service requirement of 5 MW and it chooses to purchase that requirement bilaterally from QSE 1 rather than through the ERCOT run ancillary service market.

QSE 3:

This QSE represents both load and generation and chooses to get its ancillary services from the ERCOT run ancillary service market. It has an ancillary service requirement of 5 MW.

QSE 4:

This QSE represents both load and generation. It has an ancillary service requirement of 10 MW and chooses to get one half of its ancillary services from the ERCOT run ancillary service market and self arrange the other half.

The schedule for this generic ancillary service reported to ERCOT for each of the QSE is shown below.

## Balanced Ancillary Service Schedule

QSE 1

Resource 				Obligation

5 MW internal generation		5 MW sold bilaterally to QSE 2

QSE 2

Resource				Obligation

5 MW purchased from QSE 1	5 MW requirement for its Load

Serving Entities

QSE 3

Resource				Obligation

5 MW from the ERCOT		5 MW requirement for its Load

Serving Entities

QSE 4

Resource				Obligation

5 MW internal generation	10 MW requirement for its

5 MW from the ERCOT		Load Serving Entities

## ERCOT Market Requirements

The ERCOT run ancillary service market must buy 5 MW for QSE 4 and

5 MW for QSE 3, totaling 10 MW.

## Assumed Bids for the ERCOT Market

The ERCOT run ancillary service market receives bids from QSE 1 and QSE 4. QSE bids must be in monotonically increasing ordered pairs, that is, points on a straight-line graph.

Bids:   QSE 1 (1 MW, $2.10) (21 MW, $4.10)

QSE 1’s bid may be converted into a straight-line equation of

$/ MW = 0.1 x MW + 2.

QSE 2 (1 MW, $1.20) (21 MW, $5.20)

QSE 2’s bid may be converted into a straight-line equation of

$/ MW = 0.2 x MW + 1.

## Least Cost Solution

The ERCOT system will now select the bids that minimize the cost and acquire the needed ancillary service capacity. In this generic example, this is achieved by solving the following simultaneous linear equations:

MWQ1 + MWQ4  = 10 MW

.1 X MWQ1 + 2 = .2 X MWQ4 + 1

The first equation assures that the needed capacity is the amount bought. The second equation minimizes the cost by assuring that the MCPC paid to both QSEs is the same. When both of these conditions are met, the following results are achieved:

## MW arranged through ERCOT

QSE 1		QSE 2		MCPC

3.33 MW		6.67 MW		$2.33

There are no zonal differences in ERCOT for the ancillary services asked about in this question. The MCPE is the energy price paid for any energy requested by ERCOT from these services. There are no energy price bids for these services. The MCPE used for payment is the Balancing Energy prices. The optimization model used is similar to the linear equation solution above. There must be provisions for limits included so it becomes a piecewise linear solution. If there is an insufficiency of bids, available capacity that is qualified for the ancillary service may be OOMC as described in Protocol Section 6.5.10.

Question 3

Please provide a simple and clear example to explain the operation of the Ancillary Services Market for Replacement Reserves.  In particular, please emphasize on the following areas:

- Determination of the MCPC and the amount of Replacement Reserve

- Determination of the MCPC and the amount if there are zonal differences in ERCOT for Replacement Reserve

- Determination of the MCPE ffor a unit being deployed

- In the event that two or more resources can equally be deployed, what is the mechanism to select the winning resource ? What are the capacity and energy payments ?

- Reliance on optimization models, if any, to select Replacement Reserve (succinct description of the model)

- Reliance on RMR units

- Actions to be taken if there is insufficiency of bids

## Answer

#### Amount of Replacement Reserve

ERCOT shall determine if sufficient Resource capacity is on line, in appropriate locations, and available to ERCOT to meet the potential needs of the ERCOT System. This is based on the ERCOT load forecasts and the QSE resource plans. This assumes that generation on-line will have Balancing Energy bids at the proper time. A study is made considering the upper and lower limits of resources shown on the resource plan as well as the expected transmission configuration to determine if there is a possible reliable solution. If there is a reliable solution, then no Replacement Reserve is needed. If there is not a reliable solution, then Replacement Reserve is needed, either for a zone or for all of ERCOT. The amount is dependent on what it would take to make ERCOT reliable and is determined to be zonal or ERCOT wide.

#### Determination of MCPC

Each bid for Replacement consists of two components, the one-time price for capacity and the hourly price for capacity. ERCOT will divide the capacity price component of the QSEs bid by the number of hours the Resource is needed and add the result to the QSEs hourly price of capacity. This forms the resource bid price that will be used in all determinations of bid award for RPRS.

## Example Bids

## QSE 1

## 120 MW @ $6 per MW Capacity plus $2 per MW hourly

## QSE 2

## 100 MW @ $4.50 per MW Capacity plus $2.75 per MW hourly

## ERCOT has determined that it will only need the 90 MW of Replacement for 3 hours. To compare the bids, ERCOT must perform the mathematics described in the protocols.

## For QSE 1, $6 divided by 3 hours plus $2 = $4.00 per MW

## For QSE 2, $4.50 divided by 3 hours plus $2.75 = $4.25 per MW

## Selection of Bid and MCPC

## Based on the information above, ERCOT would select QSE 1 bid and the MCPC would be $3.67

## If Replacement was needed in zones, ERCOT would use this process for only those resources in that zone.

## MCPE

QSEs that are selected to provide Replacement Reserves are required to bid Balancing Energy. The MCPE for Balancing Energy described in Question 4 is the MCPE used for any extra energy that the QSE generates.

Equal Bids

If in the example above ERCOT only needed the resources for 2 hours the outcome and resulting MCPC would be different.

## For QSE 1, $6 divided by 2 hours plus $2 = $5.00 per MW

## For QSE 2, $4.50 divided by 2 hours plus $2.75 = $5.00 per MW

The MCPC would be $5.00 and QSE 2 would be selected because the 100 MW is closer to the exact need of ERCOT thus it optimizes the cost.

RMR and Insufficient Bids

RMR units do not provide Replacement Reserve. If the RMR unit is needed for its intended purpose, local problems, it will be used by ERCOT and that use will be considered part of the resource plan. If there are insufficient bids, OOMC will be used.

The optimization model portion of this question can only be addressed by the POS vender.

## Question 4

Please provide a simple and clear example to explain the operation of Balancing Energy Market. In particular, please emphasize on the following areas:

- How is the aggregate Balancing Energy bid curve derived (taking into account ramp rates and commercial zones)

- Determination of the amount of energy deployed

- Determination of the amount of energy deployed if there are zonal congestion in ERCOT

- Reliance on optimization models, if any, to select Balancing Energy (succinct description of the model)

- Actions to be taken if there is insufficiency of bids

Balancing Energy, similar to the Ancillary Service bids in Question 2, must be bid by QSEs with ordered pairs of MW and $/MWH. The bids are by fleet but must indicate the congestion management zone. The aggregate curves may be derived for the total ERCOT or by congestion management zone if congestion exists. Consider the example below which considers 2 QSEs and 2 Congestion Management zones.

## Balancing Energy Bids

QSE 1				QSE 2

Zone A	(1 MW, $21) (21 MW, $41)	(1 MW, $12) (21 MW, $52)

Zone B	(1 MW, $12) (21 MW, $52)	(1 MW, $21) (21 MW, $41)

## Bid Adjustments for Ramps

The ramp rate limits the amount of the bid to the amount of MW that could be increased in 10 minutes considering all other responsibilities. If the bid is for more, then ERCOT must limit the amount that can be selected in one settlement period. Assume in this example that both QSEs could ramp at least 42 MW in 10 minutes. There would be no limitation in the bids as submitted.

## Composite Curve without Congestion

The composite curve for all of ERCOT is the summation of the bids. If the bids are converted to linear equations as follows, they may be summed.

QSE 1				QSE 2

Zone A	$/Mwh = 1 x Mwh + 20		$/Mwh = 2 x Mwh + 10

Zone B	$/Mwh = 2 x Mwh + 10 		$/Mwh = 1 x Mwh + 20

## Single Zone

For a single zone in ERCOT, a series of straight lines may be developed by combining the equations above into equations for each range.

Range				Equation

# 1 Mwh to 2 Mwh		$/Mwh = 12

# 2 Mwh to 11 Mwh		$/Mwh = MW + 10

11 Mwh to 13 Mwh	$/Mwh = 21

13 Mwh to 73 Mwh	$/Mwh = MW/3 + 50/3

73 Mwh to 84 Mwh	$/Mwh = Mwh – 32

## Two Zones

For two zones in ERCOT, a process similar to the process used for a single zone is used except the equations represent only the bids for that zone.

Range				Equation

# 1 Mwh to 5.5 Mwh	$/Mwh =2 x MW + 10

5.5 Mwh to 6.5 Mwh	$/Mwh = 21

6.5 Mwh to 36.5 Mwh	$/Mwh = 2 x MW/3 + 50/3

36.5 Mwh to 42 Mwh	$/Mwh = 2 x (Mwh – 21) + 10

## Amount of Balancing Energy

The amount of Balancing Energy deployed is determined by ERCOT considering their short term load forecast and the bilateral schedules planned for the same time period.  The system is designed to select the amount of Balancing Energy so that the amount of Regulation energy during the period is minimized. This leaves the maximum amount of flexibility for contingencies and minimizes the amount of regulation capacity needed.  When zonal congestion occurs, this same process is used to determine the balancing energy needed in a constrained zone.

## Insufficient Bids

Insufficient balancing energy bids with sufficient capacity on line will cause OOME in Emergency Condition.  If there is insufficient capacity on line, Non-Spin and/or Replacement is used. If necessary, Regulation Up and Responsive Reserve may also be deployed in Emergency Condition operations

## Question 5

Please provide a simple and clear example to explain the clearing of congestion management. In particular, please emphasize on the following areas (please feel free to use congestion management example at the end of this list of questions:

- Determination of MCPC for the selected resource(s)

- Determination of MCPC for selected resource(s) deployed to clear congestion

- Reliance on the use of selective incs and decs

- Reliance on optimization models, if any, to select Replacement Reserve (succinct description of the model)

- Reliance on RMR units

- Actions to be taken if there is insufficiency of bids

Congestion Management is the use of Replacement and Balancing Energy as described in the questions above including the calculation of MCPC and MCPE plus the actions taken for insufficient bids.

RMR units are used when the market does not provide a solution to a localized voltage or congestion problem.  RMR units are operated under the direction of ERCOT within the limitations in their contracts. RMR units may also be used in an Emergency Condition. The RMR owner may also sell Balancing Up Energy when ERCOT has brought the unit on line.