# RETAIL RATE DESIGN

The following information provides insights into the various types of rate designs and how various load management techniques benefit from various rate designs.

### Load Management Definitions

Shave	To reduce a customer’s grid    metered maximum demand and consumption without changing the customer’s actual consumption.    This may be achieved with on-site generation.    Rate Design for Standby Service will impact the economics.

Shift	To move a customer’s metered maximum demand and consumption from a more expensive time-of-use period to a less expensive time-of-use period, without reducing the customer’s maximum demand or total consumption.    This might occur naturally for some industries.

Shape	To improve a customer’s load factor by reducing a customer’s metered maximum demand without necessarily reducing the customer’s metered consumption.    This may be achieved by installing chillers.

Shed or Shear	To reduce a customer’s metered consumption without reducing the customer’s metered maximum demand.    This may be achieved with energy efficient facilities.

Shrink	To reduce a customer’s metered maximum demand and consumption.    This may be achieved with energy efficient facilities.

|  | Reduce Grid Metered On-Peak Demand | Reduce Grid Metered Max Demand | Improve Grid Metered Load Factor | Reduce Grid Metered Total Consumption |
| --- | --- | --- | --- | --- |
| Shaving | Yes | Yes | Yes | Yes |
| Shifting | Yes | No | No | No |
| Shaping | Yes | Yes | Yes | No |
| Shedding | No | No | No | Yes |
| Shrinking | Yes | Yes | No | Yes |

#### End-User Retail Rate Design Options

|  | SHAVE | SHIFT | SHAPE | SHED | SHRINK |
| --- | --- | --- | --- | --- | --- |
| ENERGY (VOLUMETRIC) RATES |  |  |  |  |  |
| Flat Energy Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - Savings - Same Average Rate | - Savings - Same Average Rate |
| Time-of-Use Energy Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Higher Average Rate | - Savings - Same Average Rate |
| Declining Block Energy Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - Savings - Higher Average Rate | - Savings - Higher Average Rate |
| Inverted Block Energy Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - Savings Lower Average Rate |
| Step Energy Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - Savings Lower Average Rate |
| Load Factor Energy Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - Negative Savings - Higher Average Rate | - Savings - Same Average Rate |
| DEMAND RATES |  |  |  |  |  |
| Maximum (Non-Coincident) Demand Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - No Savings - Higher Average Rate | - Savings - Same Average Rate |
| Maximum Peak Period (Coincident) Demand Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - No Savings - Higher Average Rate | - Savings - Same Average Rate |
| Time-of-Use Demand Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Lower Average Rate | - Savings - Same Average Rate |
| Inverted Block Demand Rate | - Savings - Lower Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - No Savings - Higher Average Rate | - Savings - Lower Average Rate |
| Declining Block Demand Rate | - Savings - Lower - Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - No Savings - Higher Average Rate | - Savings - Higher Average Rate |
| CUSTOMER CHARGES |  |  |  |  |  |
| Flat Customer Charge | - No Savings - Same - Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - No Savings - Higher Average Rate | - No Savings - Higher Average Rate |
| Customer Reservation Charge | - Possible Savings - Same Average Rate | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - No Savings - Higher Average Rate | - Savings - Same Average Rate |
| Customer Non-Metered (KVA) Connected Load Charge | - No Savings - Same Average Rate | - No Savings - Same Average Rate | - Savings - Lower Average Rate | - No Savings - Higher Average Rate | - Savings - Same Average Rate |

Based on the analysis put forth above, Time-of-Use rate designs provide the most competitive opportunities.    Similar to Time-of-Use rate designs, yet not too different from the Max Demand rates put forth by PG&E, a Peak Period Demand Charge provides many competitive opportunities and savings through load management.    In lieu of debating Time-of-Use cost allocation, supporting a Peak Period Demand Charge may be the most conservation friendly and easily defensible rate design.

#### Standby Charges

PG&E offers two types of Standby Service.

Under Rate Schedule S, PG&E offers Back-Up Standby Service to customers (1) whose premises are regularly and completely supplied by electric energy from a nonutility source of supply that consistently produces at least enough power to meet the customers’ total electric demand; (2) who at times take auxiliary service (by means of a double-throw switch) from another public utility; (3) who require PG&E to provide reserve capacity and stand ready at all times to supply electricity on an irregular or noncontinuous basis; or (4) whose nonutility source of generation does not regularly supply all the power necessary at their premises provided the rated capacity of the customer’s onsite generator is at least 50 percent of the customer’s maximum kW demand and load profile recorders are installed to separately meter the net on-site generation and the on-site load.    Customers taking Back-Up Standby Service pay a combination of a T&D reservation charge and a volumetric charge that recovers distribution, amongst other unbundled volumetric charges, when energy is delivered from the grid.    PG&E’s present volumetric rates for this service reflect unbundled distribution rates on a time-of-use basis roughly equivalent to unbundled distribution rates offered to other customers charged on a volumetric basis only.    Under the present rates, the premium for taking energy from the grid is reflected in the unbundled generation costs which recovers PX costs and CTCs.    PG&E’s proposed volumetric rates for this service reflect flat unbundled distribution rates roughly equivalent to unbundled distribution rates offered to other customers charged on a volumetric basis only.    (Note: PG&E states in its GRC Phase II testimony that the volumetric rate for Back-up Standby Service is based on a 150kWh/kW break-even point or 21% load factor, which is comparable to the level of capacity costs recovered in PG&E’s present Standby Volumetric Rates.)    Further, the premium reflected in the unbundled generation is basically swapped out with a fixed CTC rate and the actual cost of PX energy.    With respect to the unbundled distribution rate, the only difference between the present and proposed rates is a TOU versus flat rate design.    No usage premium is reflected in the unbundled distribution rate in either case.    Hence, PG&E charges a reservation charge plus a volumetric charge for distribution when energy is taken from the grid.    It does not appear that the volumetric distribution charge is discounted for distribution reservation charges in the rate design or as a subsequent credit to the Standby customer who takes energy from the grid.

PG&E also offers Supplemental Standby Service in conjunction with the customer’s applicable rate schedule for customers whose premises are: (1) supplied only in part by electric energy from a non-utility source of supply, and who do not qualify for or elect to take Back-Up Standby Service; or (2) whose regular non-utility source of supply is subject to an extended outage of at least one complete regular billing cycle.    Customers taking Supplemental Standby Service pay a combination of a T&D reservation charge, in addition to the charges on the applicable rate schedule when energy is delivered from the grid. Under PG&E’s present rate structure unbundled distribution rates are charged in a TOU demand and energy charge for large customers.    Under PG&E’s proposed rate structure unbundled distribution rates would be charge on a max demand charge for large customers. Hence, PG&E charges a reservation charge plus the applicable charge for distribution when energy is taken from the grid.    It does not appear that the applicable charges for distribution are discounted for distribution reservation charges in the rate design or as a subsequent credit to the Standby customer who takes energy from the grid.

The fundamental question regarding Standby Service is whether a reservation charge is appropriate for Standby Service given that a customer who takes energy from the grid will pay for distribution when energy is taken from the grid.    In other words, should customers with onsite generation be treated differently from any other customers that pay for distribution based on the energy that is taken from the grid.    Utilities may view the elimination of the reservation charge as a free option, but this is no different then the free options that other customers receive.    PG&E does recognize that loads served under Schedule S are served by multiple, independent generators which will not simultaneously require service and PG&E’s local operating areas need not hold in reserve 100 percent of the locally subscribed standby capacity.    As a result, PG&E has discounted the reservation charge to approximately 38% of the distribution demand charge for non-Standby customers.    Still, there may be double recovery issues through recovery of distribution through reservation charges and charges applied when energy is taken from the grid.

We may have two alternatives: (1) argue that since no customers pay for wires services through a reservation charge, customers with onsite generation should not be required to pay a reservation charge; or (2) argue that a customer’s wires charges when energy is taken from the grid should be reduced by the amount paid in a reservation charge either through a credit or in the rate design.    In either case, the Standby Customer should have to pay for energy based on real-time prices and should not pay based on a class average load profile since the customer’s usage pattern does not conform with the class average load profile.

# DISTRIBUTION BYPASS

DISTRIBUTION BYPASS OPPORTUNITIES

On-Site Generation with Standby Service

Disconnect Load served by On-Site Generation

Voltage Buy-up

Demand-Side-Management

Move Load to another Service Territory

Disconnect Load and Connect Load to a Neighboring Service Territory

UTILITY RESPONSES TO DISTRIBUTION BYPASS

Exit Fee – PG&E is proposing a exit fee be assigned directly to the individual departing customers for customers with load greater or equal to 20 kW without CTC exemptions from irrigation districts and customers installing cogeneration units for all or a portion of their load taking standby service from PG&E.    This exit fee is based on 10 years of lost margins.    Enron should oppose the exit fee because it penalizes individual customers for exercising competitive options.

Flexible Pricing Options – In lieu of an exit fee, PG&E is requesting that shortfalls from individual customers bypassing PG&E’s distribution system be passed onto other ratepayers and requests continuation of flexible pricing options to mitigate such shortfalls.    First, this is certainly the lesser of two evils: exit fee versus flexible pricing.    Second, this is consistent with our position that distribution service is competitive.    Third, this supports our Utility Representation Activities.    However, this is all in theory.    In reality, the utilities do not necessarily flexibly price on a non-discriminatory basis, perhaps providing preferential treatment to default customers or customers of the utility’s affiliate.    Further, utilities may use this option to thwart any potential deals between ESPs and customers.

Geographic Rate Deaveraging – See below.

## GLOBAL DISTRIBUTION RATEMAKING ISSUES BETTER SUITED IN DISTRIBUTION OII

The following issues are distribution ratemaking issues that are not being addressed in PG&E’s Phase II GRC, but are nonetheless important to Enron: (1) Aggregation Rates; (2) Geographich Deaveraging; and (3) Distribution PBR.