ENRON GPG ENVIRONMENTAL NOTICE

Subject:    Transmission and Storage Maximum Achievable Control Technology

Today’s Date: August 9, 1999

Reference:    June 17, 1999    (64 Fed. Reg. 32610-32664)

Status:    Final Regulations

General: The Transmission and Storage Maximum Achievable Control Technology (MACT) Regulation governs Hazardous Pollutant (HAP) emissions such as benzene, toluene, ethyl-benzene and xylene (BTEX) from Major Sources with glycol dehydrators. This regulation governs these emission sources in the transmission system downstream of gas plants and at underground storage facilities. A companion notice has been prepared for another regulation governing emission sources in the gathering and pipeline system up to and including sources at gas processing plants.

New sources (constructed on or after February 6, 1998) are regulated immediately. Existing sources (constructed before February 6, 1998) have until June 17, 2002 to comply with the requirements.

Only Major Sources for HAPs (locations that potentially emit 10 tons/year of one HAP or 25 tons/year of all HAPs) are regulated. When determining whether a location is a Major Source you must include all HAP emission sources including formaldehyde from engines. When calculating glycol dehydrator potential emissions you are required to use 120% of gas throughputs based on the highest annual value in the last 5 years for transmission stations and a formula based on potential withdrawal cycles for underground storage. Locations, without other HAP emission sources such as engines, with total throughputs less than 1.0 MMSCF/day are deemed to be Area (Non-Major) Sources. EPA does not plan on regulating Area Source glycol dehydrators associated with transmission and storage.

Individual dehydrators can be exempted from the control, monitoring and most of the recordkeeping requirements if the actual emissions from a unit, calculated using the software program GRI-GlyCalc, are less than one ton/year of benzene or actual gas throughput is less than 10 MMSCF/year (monitored to +/- 2% or documented). Records must be maintained demonstrating the calculation of the exemption values. If controls are installed to reduce emissions to less than the benzene threshold, a federally enforceable limit such as a state permit must be obtained.

The regulating agency must be notified about existing Major Sources for HAPs by June 17, 2000, even if there are plans to reduce emissions later. Controls on existing sources to reduce emissions to less than Major Source or exemption thresholds must be installed by June 17, 2002. A startup/shutdown/malfunction plan must be prepared for HAP Major Sources even if all glycol units are exempted from the regulations. The Title V operating permits should be updated to include this new regulation.

If glycol dehydration units cannot be exempted from this regulation, emissions must be reduced by 95% either through controls or process modifications, and there are other significant and burdensome requirements including initial performance testing, "enhanced" monitoring, recordkeeping, and semi-annual agency reporting.

Impact: The impact of this rule should not be consequential because of the limited number (except at Redfield storage) of glycol dehydrators at transmission and storage facilities as defined in the regulation. Efforts should be undertaken to reduce emissions to less than the Major Source or exemption thresholds. If successful it is anticipated that few glycol dehydrators will be subject to the extensive control, monitoring reporting and recordkeeping requirements in this regulation. It will require some effort to determine Major Sources under the new methodology specified in this regulation, install controls if required to reduce emissions, calculate emissions to determine if the exemption criteria have been met, modify Title V permits, and obtain state agency permits to enforce the reduced emission levels.

Recommended Actions: The Division Environmental Specialists should determine if their inventories of locations with glycol dehydrators are complete. They should re-calculate emissions based on the new EPA criterion at locations near the Major Source thresholds to determine if their Title V status is affected. For the HAP Major Sources, the permitting state agency should be contacted to determine how to incorporate these requirements into the Title V operating permit. The above three items should be completed by October 1, 1999. Once the Major Source locations have been determined they should develop a strategy to reduce emissions below exemption levels. The strategy should be developed before the June 17, 2000 deadline to register Major Sources. It has been indicated that GPG does not have any new sources (installed on or after February 6, 1998) so this notice does not discuss the new source requirements.

This Environmental Notice (Technical Version) has been distributed to appropriate directors of technical services and environmental and technical staff.    A Management Version has been distributed to appropriate management personnel.    If there are any questions concerning this regulation, contact Marc Phillips at (713) 646-7646.

Distribution:

| Operations Support | Tech Directors | DES |
| --- | --- | --- |
| Terraso, Mike | Cates, Rick | Ayers, David |
| Environmental Team | Weatherford, Allan | Campbell, Larry |
| Reg Tech Team |  | Harkrader, Ronald |
| Fernandez, Sue | Environ Coord | Loveless, Rick |
|  | Poston, Tom | Nichols, Leo |
| Legal | Stebens, Monte | Purvis, Kyle |
| Nutt, David | Trout, Lonnie | Riedel, Mike |
| Reich, Bret |  | Russell, Butch |
| Smith, Frank |  |  |
| Soldano, Lou |  |  |