1.	GENERAL

This standard covers testing and inspection for the acceptance of newly fabricated orifice meter tubes and is recommended as a guide for evaluating the condition of existing orifice meter tubes.  This standard is to be used in conjunction with Engineering Standard 5421 for the purchase of new orifice meter tubes.  The Company reserves the right to conduct any other inspections deemed necessary to ensure the quality of the meter tube assembly.

2.	ORIFICE FITTING BLANK PLATE LEAKAGE TEST

This test is part of the overall orifice meter tube inspection and shall be done prior to the final meter tube inspection and after hydrostatic strength testing.  Unless specified otherwise, this test will be done by the meter tube fabricator.  The test will be conducted in accordance with this standard and a record of the test will be furnished using the form herein or suitable equivalent.

Step 1:	Isolate and vent the orifice meter run.  Ensure the upstream and downstream isolating valves do not leak.

Step 2:	Install an unbored orifice plate in the orifice fitting.

Step 3:	Attach the recommended test manifold illustrated in this standard.

Step 4:	Connect the pressure source to the inlet to the manifold regulator.

Step 5:	With valve "b" partly open, valve "a" fully open, and valve "c" closed, adjust the regulator to deliver 55 inches of water column (approx. 2 psig).

Step 6:	Close valve "b" and open valve "c" and any other valves required to admit gas from the manifold to the orifice meter run.

Step 7:	When the orifice meter run is pressured to 55 inches of water column, close valves "a" and "c" and open valve "b".  If no leakage is observed at valve "b", proceed with the test.

Step 8:	Observe the orifice meter recorder for ten minutes.  If no differential pressure decrease is observed proceed.  If decrease is observed orifice meter run must be disassembled and inspected using liquid leak detector or other method to check seal and tap hole integrity.

Step 9:	Vent enough gas through valve "c" to reduce the pressure to 10 inches of water column.  Observe the orifice meter recorder for five minutes.  If no differential pressure decrease is observed, proceed to the next step.  If a decrease is observed, follow the remedial steps detailed in Step 8.

Step 10:	Return the orifice meter run to normal operation.

Step 11:	Attach orifice meter chart to Test Report.

RECOMMENDED TEST MANIFOLD

ORIFICE FITTING BLANK PLATE LEAKAGE TEST REPORT

| DATE: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VENDOR: |  |  |  |  |  |  |  |  |
| LOCATION OF TEST: |  |  |  |  |  |  |  |  |
| ORIFICE FITTING SERIAL NO: |  |  |  |  |  |  |  |  |
| NOMINAL PIPE SIZE: |  |  |  |  |  |  |  |  |
| ANSI CLASS: |  |  |  |  |  |  |  |  |
| METER TUBE SERIAL NO: |  |  |  |  |  |  |  |  |
| TEST METHOD (attach this procedure or describe) |  |  |  |  |  |  |  |  |
| ORIFICE METER RECORDER CALIBRATION DATE: |  |  |  |  |  |  |  |  |
| TEST GAUGE CALIBRATION DATE: |  |  |  |  |  |  |  |  |
| RESULTS: |  |  |  |  |  |  |  |  |

| COMPANY INSPECTOR'S SIGNATURE: |  | DATE: |  |
| --- | --- | --- | --- |
| VENDOR'S SIGNATURE: |  | DATE: |  |

3.	ORIFICE METER FINAL INSPECTION AND CALIBRATION REPORT

3.1	Equipment and Materials

The Vendor shall make available any or all of the following equipment and materials or other necessary test equipment as requested by the Company’s inspector during the inspection:

| • | Drawings, specification and purchase order |
| --- | --- |
| • | Clipboard and pencil |
| • | AGA Report No. 3 (Orifice), part 2 |
| • | Inspection Check Sheet (copy included in this standard) |
| • | Meter Tube Calibration Report Form (copy included in this standard) |
| • | Hand calculator |
| • | 2'-0" level |
| • | 100' steel tape measure |
| • | 20' steel tape measure |
| • | Tap hole gauge |
| • | Inside/Outside caliper with vernier (not dial) |
| • | Inside micrometers |
| • | Outside micrometers |
| • | Standard bar |
| • | Straight edge |
| • | Carpenters square |
| • | Vacuum pump equipped with gauge and provision to accept 1/4" ID tubing |
| • | 1/4" ID tygon tubing 2;-0" long |
| • | 1 each adapter 1/4" ID tube to 1/2" male NPT |
| • | 1 each rubber stopper to plug 1/2" male NPT |
| • | Flashlight |
| • | Soap stone marker |
| • | Orifice plate with approximately a 0.75 beta ratio |
| • | Seal ring for orifice plate |

3.2	Inspection

Inspect the meter tube assembly following the order of the subsequent outline which progresses from simpler checks to the more complex and time consuming measurements.  This may avoid expenditure of time performing complex measurements on a tube to be rejected for not meeting basic configurational requirements.  Record the results on the Meter Tube Inspection Sheet and the Orifice Meter Tube Calibration Report provided herein.

3.2.1	Check for dents, cracks, scratches, gouges, pits, laminations, correct piping material specifications and ratings and proper type orifice fitting.  Refer to AGA Report No. 3, latest edition, and the purchase specifications for acceptance criteria.

3.2.2	After all items above are checked and found acceptable, check the following:

Physical Dimensions:  Overall length of upstream and downstream sections cannot exceed ± 1/8-inch from drawing(s).

Flange Faces:  Must be serrated (Phonograph groves).

Flange "Two-Bolt " Level:  Put meter tube in exact upright position, the top two bolt holes on each end of the tube should be level with each other.

Flange Tilt:  Maximum allowable is 1/4 degree.  To determine this, plumb the flange face vertically with a level by raising or lowering one end of the tube.  When the flange is plumb, place a 2'-0" level horizontally on top of tube.  Raise the low end of the level and measure the vertical gap between the end of the level and the tube.  A gap of 1/8-inch or more per two feet will be cause for rejection.

Bad or Poorly Finished Welds:  Welds should be clean and smooth on the outside and inside and ground smooth to near the ID of the tube inside except at the orifice fitting or flange where inside welds should be the same ID as the tube.

Communication Between Tap Holes:  Each tap hole, except on an orifice flange union (OFU), should be checked to be sure it will hold pressure.  Place a rubber stopper in the tap hole inside the fitting and pull vacuum pump.  If the vacuum pump will hold for fifteen seconds, the test is acceptable.

Straightening Vane Location:  This distance should be determined from the drawing but generally should agree with AGA Report No. 3, (latest edition) Part 2, figure 2-5, using a Beta ratio of 0.75.

3.2.3	Tap Holes

Check tap holes for slant:  Tap holes to be perpendicular to meter tube longitudinal axis and circumferential tangent.

Check tap hole I.D.:  I.D. is 1/2-inch at the inner surface on tubes 4-inch and larger, and 3/8" on tubes on 2-inch and 3-inch diameter.  Tolerance is  ± 1/64-inch as detailed in AGA No. 3.

Check flange tap locations:  Flange tap locations are to be one (1) inch from the face of the installed orifice plate to the center of the tap hole.  The tolerance is ± 0.030" for 4-inch tubes and larger and ± 0.015" for tubes smaller than 4-inch.  This measurement is done with a tap hole measuring gauge to determine the spacing between the orifice plate and the downstream tap hole on the orifice fitting.  This spacing is then measured with a caliper and recorded.  The orifice plate is then removed and the spacing between the two tap holes is determined.  A caliper then measures this distance on the tap hole measuring gauge.  The location of the upstream tap is calculated by subtracting the plate thickness and the downstream tap hole distance from the distance between the two tap holes.  Measurement on an orifice flange union (OFU) is done with a carpenter's square and includes the thickness of the gasket.

Check pipe tap locations (Pipe taps are not to be used with new tubes.):  Pipe Tap Locations are as follows:

a.	Upstream:  2-1/2 A.I.D. pipe diameters from the face of the plate to the center of the tap hole.

b.	Downstream:  8 A.I.D. pipe diameters from the face of the plate to center of the tap hole.

c.	Tolerance is ± 0.30” for 4-inch tubes and larger and ±-0.15" for tubes smaller than 4-inch.

3.2.4	Orifice Plate Centering

Allowable eccentricity must be equal to or less than the tolerance given in AGA Report No. 3, Part 2, (latest edition) Table 2.7 for 2-inch to 10-inch tube.  For any size tube, use formula  < 0.0025Dm/(0.1+2.3m4) for allowable eccentricity, where:

| Dm | = measured tube ID at temperature at the time of diameter measurement |
| --- | --- |
| m | = dm/Dm |
| dm | = measured orifice plate bore ID at temperature at the time of diameter measurement |
|  | = eccentricity |

To determine if orifice plate is centered within the allowable tolerance, proceed as follows.

Step 1:	Measure vertically from top inside wall of tube to top edge of orifice.

Step 2:	Measure vertically from bottom inside wall of tube to bottom edge of orifice.

Step 3:	Use equation:  distance from center = (larger number - smaller number)/ 2.  From the example in the illustration above, distance from center = (2.549-1.971)/2 = 0.289.

Step 4:	Measure horizontal distances as shown above and use same equation as in Step 3. to find distance from center of plate.  From the example in the illustration above, distance from center of plate = (2.442-2.078)/2 = 0.182.

Step 5:	Compare the values obtained from Steps 3 and 4 against allowable eccentricity per latest AGA-3, part 2. .  Eccentricity tolerance should be based on 0.7 Beta.  For 4-inch and smaller tubes, eccentricity can be 0.02” or less with a 0.7 Beta plate

3.2.5	Meter Tube Inside Diameter

Step 1:	Standardize the inside micrometers by using a standard bar in an outside micrometer to establish a standard "gap".  Now measure the "gap" with the inside micrometer and add or subtract a "mike correction" to each subsequent mike reading taken within the meter tube.

Step 2:	Mark with soap stone, using a square and straight edge, the tube diameters and locations specified on the meter tube calibration form being sure to include the location two pipe diameters upstream.

Step 3:	Measure the points located as above to one-thousandth of an inch and record on the meter tube calibration form.  Record meter tube temperature at the time of diameter measurements.

Step 4:	Check for any small irregularities in the pipe to be sure their depths are included when comparing the longest and shortest diameters.

Step 5:	Subtract the shortest diameter from the longest diameter upstream.  The measured A.I.D. (Dm) taken one (1) inch upstream of the upstream face of the orifice plate must be within 0.25% of Dm from any diameter measured within one (1) Dm in the upstream side of the orifice plate.  The difference between maximum I.D. and minimum I.D. for all measurements upstream and downstream of the orifice plate shall not exceed 0.5% of Dm.

4.	DOCUMENTATION

4.1	Make sure the upstream and downstream tube sections are properly stamped with matching serial numbers and I.D. and that these numbers, along with the orifice fitting numbers, appear on all inspection reports.

4.2	Complete the Meter Tube Inspection Check sheet.

4.3	Complete the Meter Tube Calibration Report.

4.4	Ensure that the Meter Tube Specification Sheet (see Engineering Standard 5421) and all drawings are revised to reflect the final fabrication.

METER TUBE INSPECTION CHECK SHEET

| Tube Number |  |  |  |  | Date |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fitting Number |  |  |  |  | Plate Holder Type |  |  |  |  |  |  |
| Manufacturer |  |  |  |  | Pipe Size |  |  |  |  | ANSI CLASS |  |
| Destination of Runs |  |  |  |  | P.O. Number |  |  |  |  |  |  |
| Inspector |  |  |  |  | W.O Number |  |  |  |  |  |  |

| ACCEPTABLE |  | NOT ACCEPTABLE |  | ITEM |
| --- | --- | --- | --- | --- |
|  |  |  | Company/Vendor Drawing(s) Checked to Meter Tube Specifications |  |
|  |  |  | Welding |  |
|  |  |  | Circumferential butt welds (ground smooth with inside tube surface) |  |
|  |  |  | Welds on Taps and Wells (no burrs on inside wall) |  |
|  |  |  | Measurements |  |
|  |  |  | Tap Holes Right Size |  |
|  |  |  | Tap Hole Location (1" for Flange Taps, 2-1/2 x ID for Pipe Taps) |  |
|  |  |  | Straightening Vane Location |  |
|  |  |  | Length of Pipe |  |
|  |  |  | Roundness, UpstreamTube, (Mike) |  |
|  |  |  | Orifice Plate Centered |  |
|  |  |  | Miscellaneous |  |
|  |  |  | Pipe free of Grooves, Pits or Deep Scratches |  |
|  |  |  | Flanges Mounted Straight on Pipe |  |
|  |  |  | Tap Holes Not Slanted |  |
|  |  |  | Tap Hole Leak Tests |  |
|  |  |  | Meter Tube Marked per Specification |  |
|  |  |  | Hydrostatic Test Chart Attached |  |
|  |  |  | Orifice Fitting Blank Plate Leakage Test Report Attached |  |
|  |  |  | Vendor Certifications per ES-5421 Attached |  |
|  |  |  | Vendor Calibration Sheets Attached |  |

| Remarks: |  |
| --- | --- |

| Station Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Run No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date (mmddyy) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Company No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 060-TW 179-NNG 062-FGT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 012-HPL 018-IGC 028-OPL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 014-VPL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 172-NBP |  |  |  |  |  |  |  |  |  |
| Make |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ANSI Class |  |  |  |  |  |  |  |  |  |  |  |  | 300 600 |  |  |  |  |  |  |  |  |  |  |  |  | Serial Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 000-OTHER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fabricator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pipe Schedule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Stamped Meter Size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P.O Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| USE DRAWINGS ON BACK OF FORM FOR MEASUREMENT LOCATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  | UPSTREAM TUBE LOCATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MICROMETER READINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DOWNSTREAM TUBE LOCATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  | D |  |  |  |  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  | F |  |  |  |  |  |  |  |  |  | G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SURFACE ROUGHNESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALLOWABLE AVERAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AVERAGE TUBE ID (Dm at Location (C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ROUGHNESS = 250 MICROINCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TEMPERATURE OF METER TUBE DURING CALIBRATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DEGREE F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PASS |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  |  | CALCULATED REFERENCE ID AT 68 DEGREES F (Dm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | METER TUBE LENGTH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ACTUAL TUBE LENGTH PER COMPANY SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ANSI/API 2530 FIGURE No. 2-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MINIMUM REQUIRED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ACTUAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ** DISTANCE FROM FACE OF PLATE TO FIRST OBSTRUCTION: THERMOWELL, FITTING, ETC. |
|  | TYPE |  | FLANGED |  |  |  |  |  |  |  |  | LINE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STRAIGHTENING VANES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | MAX. L 0.5 C’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  | MAX. AREA A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SQUARE INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MAX. A Dr/4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |
|  | REQUIRED DIMENSION |  |  |  |  |  |  |  |  |  |  | MIN. L 10a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | REF: ANSI/API 2530 (2-5.5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ACTUAL DIMENSIONS |  |  |  |  |  |  |  |  |  |  | ACTUAL L. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  | ACTUAL A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SQUARE INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ACTUAL a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ORIFICE FITTING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAKE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TYPE: |  |  |  |  |  |  |  | SR |  |  |  |  |  |  |  |  | JR |  |  |  |  |  | SIMPLEX |  |  |  |  |  |  |  |  |  |  |  |  |  | FLANGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SERIAL No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALIGNMENT OF PIPE & FITTING |  |  |  | OK: YES/NO |  |  |  |  |  |  |  |  | WIDTH OF PLATE SLOT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  | WIDTH OF SEAL W/PLATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TYPE OF SEAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ELASTOMER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TEFLON |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ORIFICE TUBE ROUNDNESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REF: ANSI/API 2530 (2.5.1.3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1ST DIAMETER MEASUREMENT REF: ANSI/API 2530 (2.5.1.3.1.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAXIMUM ALLOWABLE DIFFERENCE UPSTREAM OF ORIFICE = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Dm X 0.0025 = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANY DIAMETER - Dm = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAXIMUM ALLOWABLE DIFFERENCE FOR ALL UPSTREAM AND DOWNSTREAM MEASUREMENTS = 0.005 X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Dm = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LARGEST DIAMETER - SMALLEST DIAMETER UPSTREAM = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LARGEST DIAMETER - SMALLEST DIAMETER DOWNSTREAM = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REF: ANSI/API 2530 (2.5.4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PRESSURE TAPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE FROM |  |  |  |  |  |  |  | UPSTREAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  | DOWNSTREAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | INCHES |  |  |  |  |  |  |  |  | HOLE DIAMETER ALLOWABLE ± 0.015 INCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FACE OF PLATE TO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CENTER OF TAP |  |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ACTUAL INCHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |  |  |  | FAIL |  |  |  |  |  |  |  |  |  |  |
| COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNATURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WITNESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

CALIBRATION REPORT

DATA COLLECTION SHEET

| UPSTREAM TUBE LOCATION |  |  | MICROMETER READINGS |  |  |  | DOWNSTREAM TUBE LOCATION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LOCATION | A | B |  | C | D | E |  | F | G |
| V |  |  |  |  |  |  |  |  |  |
| LV |  |  |  |  |  |  |  |  |  |
| RV |  |  |  |  |  |  |  |  |  |
| H |  |  |  |  |  |  |  |  |  |
| SURFACE ROUGHNESS |  |  |  |  |  |  |  |  |  |

| ALLOWABLE AVERAGE |  |  |  |  | AVERAGE TUBE ID (Dm at Location (C) |  | INCHES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ROUGHNESS = 250 MICROINCHES |  |  |  |  | TEMPERATURE OF METER TUBE DURING CALIBRATION |  | DEGREE F |
| PASS |  | FAIL |  |  | CALCULATED REFERENCE ID AT 68 DEGREES F (Dm) |  | INCHES |