# Correspondence Summary Sheet

| By: Alisa Wade, ENSR | Date: September 11, 2001 |
| --- | --- |
| ## Talked With: Bill Olson | Project Number: 6792162-100 |
| Title: Hydrologist | Project Name: Sun Devil Fatal Flaw Analysis |
| Of: Oil Conservation Division, NMEMNRD | Subject: Hydrostatic Test Permit and Compressor Station Discharge Permit, New Mexico |
| Telephone Number: 505-476-3491 | Facsimile Number: |
| Email or Internet Address (if applicable): |  |
| ### Supplemental Information Attached? YES |  |
| Indicate Documentation Type: Telephone |  |

No form is required for discharge of hydrostatic test water under 100,000 gallons from a NEW pipe to land or waters that are not waters of the U.S.  A letter of notice, explaining location, volume, water source, etc. should be sent to Roger Anderson at the NM EMNRD at PO Box 6429, Santa Fe, NM  87505.  Guidelines for Hydrostatic Test Dewatering (attached) should be followed.  No discharge can occur until NM EMNRD provides a letter of approval in response.

For volumes over 100,000 gallons, discharge from a USED pipe, or in instances where other requirements for a general approval are not met (see guidelines), then a discharge permit must be obtained from the OCD.  A letter should be sent to obtain this permit as well.  The guidelines provide information on information to be included in letter of application.

There is no fee for these permit/approvals.  Applicant should allow 60 days for approval.  All discharge locations can be authorized under one permit/approval.

Compressor stations must have discharge permit.  A Discharge Plan Application for Service Companies, Gas Plants, Refineries, Compressor, Geothermal Facilities and Crude Oil Pump Stations must be submitted.  See attached application.  Guidelines for the Preparation of Discharge Plans should be followed.  There is a fee of $50 for the application and the additional permit fee is determined based on the horsepower of the station.

- Guidelines for Hydrostatic Test Dewatering

- Discharge Plan Application for Service Companies, Gas Plants, Refineries, Compressor, Geothermal Facilities and Crude Oil Pump Stations

- Guidelines for the Preparation of Discharge Plans

_________________________________________

Signature

| Distribution: | (1) Author | (2) ENSR File 6792162-100 | (3) |
| --- | --- | --- | --- |