# Correspondence Summary Sheet

| By: Alisa Wade, ENSR | Date: September 11, 2001 |
| --- | --- |
| ## Talked With: Kristie Kilgore | Project Number: 6792162-100 |
| Title: Aquifer Protection Program Specialist | Project Name: Sun Devil Fatal Flaw Analysis |
| Of: Arizona Department of Environmental Quality | Subject: Aquifer Protection/Hydrostatic Test Discharge Permit, Arizona |
| Telephone Number: 602-207-4696 | Facsimile Number: |
| Email or Internet Address (if applicable): |  |
| ### Supplemental Information Attached? YES |  |
| Indicate Documentation Type: Telephone |  |

Hydrostatic Test Discharge Permit:

Falls under a type 1 general permit, which is a permit by rule as long as all conditions and requirements are met.  Permit is regulated under section R18-9-B301 of the Arizona Administrative Code.  There are two types of permits:

Paragraph B. 1.02 permit for discharges from new pipelines

- quality of the water used for test must not violate any aquifer water quality standard

- discharge is not to waters of the U.S. unless discharge is under NPDES permit

- test site must be restored to natural grade

Paragraph C. 1.03 permit for discharges from existing pipelines

- all liquid discharge must be contained in an impoundment lined with flexible geomembrane material with a thickness of at least 10 mms

- the liner material must be placed over layer, at least 3 inches thick, of sand or finer grained material

- water must be removed from impoundment to appropriate landfill or treatment works within 60 days

- liner must be removed and disposed of an approved landfill unless liner can be reused

- test site must be restored to its natural grade

There is no application, notice, nor fee required.

Construction Dewatering Permit:

Falls under a type 1 general permit, which is a permit by rule as long as all conditions and requirements are met.  Permit is regulated under section R18-9-B301 of the Arizona Administrative Code.

Paragraph A. 1.01 permit for discharges from construction dewatering

- only physical processes may be employed

- no hazardous substances are in water except those naturally occurring in sand, gravel, or other rock material

There is no application, notice, nor fee required.

Attachments:

1.  Aquifer Protection Program Regulations

_________________________________________

Signature

| Distribution: | (1) Author | (2) ENSR File 6792162-100 | (3) |
| --- | --- | --- | --- |