SW OPERATIONS WEEKLY REPORT

February 14, 2002

Amarillo Region

# SAFETY

|  | THIS WEEK |  |  | YEAR TO DATE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NNG | TW | EOTT | NNG | TW | EOTT |
| Vehicle Accidents | 1 |  |  |  |  |  |
| Lost Time Injuries |  |  |  |  |  |  |
| OSHA Recordable |  |  |  |  |  |  |
| Animal Hits (not SGA Reportable) |  |  |  |  |  |  |

- A vehicle accident occurred when a company truck was pulled out of the snow after an earlier winter storm.  The company vehicle was unable to stop before hitting the vehicle that pulled it out.  There was no damage to the other vehicle.

COMPRESSION

NNG:

- Elk City – Unit #3 at Roger Mills is down due to an oil cooler leak.  The cooler is being repaired and is expected to be back by Friday.

- Spearman – The rental unit at Hutch Co 1 was inspected by J-W Operating Company. This inspection is part of the return agreement.  When we receive approval, the unit will be returned.

- Beaver Plant – Unit #18 is down while the machine shop is repairing the water pump.

- Ellis 2 – Ellis 1#2 was down this week for a water pump repair, units #1 and #3 had 1500-hour inspection, as well as Woodward 1.

- Sunray – received word from marketing and gas control that Duke will be pulling their volume off the system on April 1.  This will be around 20-30 mmcf/d.

TW:

- Canadian – Unit #3 at the Canadian River Station is down for a 1500-hour inspection.  Also changed two heads and a turbo gasket.

- Pampa – Unit#5 was down for a few hours today, but was checked out and put back on line.

EOTT:

- Oklahoma City – 5 generators are still being used at various stations due to the continuing electrical outage problems.  Power is being restored daily but some sites may be down until the end of the month.

PIPELINE

NNG:

- Pampa – the team responded to gas quality alarms at Cargray last weekend.  High H2O and O2 resulted from upsets at the Cargray Plant.

- Perryton – the team responded to High H2O alarms at Barlow.  They are working on the dehy.

- Corrosion Specialists met with team members this week to discuss corrosion issues and projects.

TW:

- Pampa – March 5 is the scheduled date for the tie-in of the bypass line at P-3.  This is part of the interconnect project.

EOTT:

- Teams are preparing for DOT audits that are scheduled to begin later this month.  They are holding pre-audit meetings.

ENVIRONMENTAL

NNG:

- Ellis 2 – the team and Rick Loveless are working on air permits and a remediation project for the suction line into Woodward 2.

TW:

EOTT:

- Remediation is on going for the Elk City 10” line.

OTHER

NNG:

TW:

EOTT:

|  | ###### Monday | Tuesday | Wednesday | Thursday | Friday |
| --- | --- | --- | --- | --- | --- |
| LeBeau | Holiday | Amarillo | Amarillo | Houston | Houston |
| Williams | Holiday | Audit | Audit | Audit | Audit |
| Jordan | Holiday | Beaver | Office | Ellis 2 | Beaver |
| Lowe | Holiday | Canadian/Elk City | Offfice | Perryton | Office |
| Ford | Holiday | Oklahoma City | Duncan | Cimarron | Oklahoma City |