|  | California | Texas |
| --- | --- | --- |
| Market Rules |  |  |
| ISO | - Governance - Stakeholder - Fee - $0.15/MWH - Charge to maintain system security - Charge to maintain system security - may dispatch out of order merit. | - Governance - Stakeholder - Fee $0.80/MWH - Publishes forecast for the day-ahead market. - Charge to maintain system security - may dispatch out of order merit |
| PX | - July99 Utilities can buy forward from the PX - PUC approves Edison & PG&E under emergency order to engage in bilateral contracts, but scheduled through the PX - PX voted recently to lower the cap in the exchange to $350 per MWH (8/16) - Fee $____/ MWH | None - bilateral structure |
| SC (QSE) | Yes - Brings Ancillary Services, and schedules to the ISO | - Acts like a mini exchange for the entities it represents. - Represents REPS and/or PGCs in schedules at the ISO. - The only entity entitled to submit balanced schedules to the ISO. - Entity submitting Ancillary services in the scheduling process or submitting bids in the ancillary service market. |
| Price Caps | ISO - $250, PX - $350 | None |
| RMR | - Assigned by the ISO - Can be ordered by the ISO to run | - Requested by the PGC - Can be ordered to run by the ISO |
| OOM (out of order merit) | - ISO can accept higher bid units to run over less expensive for reliability | - ISO can accept higher bid units to run over less expensive for reliability |
| Ancillary Services |  | - Self provided (limited), scheduled by QSE - QSE submits ancillary service bids to the ISO in the day-ahead - Sequential clearing of bids, Reg. Up, Reg. Down, Responsive, Contingency reserve |
| Control Areas | Multiple | Single |
| Price Transparency | None - market-clearing prices published only. | - Drafting publication of supply stack for all ISO markets cleared. - Provide loads the opportunity to respond to price signal, same as generators. |
| New Facilities | No incentive for new Transmission or cumbersome siting process | - ISO identifies new transmission needs, which assists the PUC with Certification of Need and Necessity. - ISO can go out for bid of new transmission facilities if originating utility rejects of delays project progress. (process not tested yet) |
| Customer Registration /switching | Customer switching is performed by each utility under different rules | Customer switching performed by the ISO under standard rules under a centralized process |
| Regulatory Rules |  |  |
| Date Certain | Full Competition | Pilot program then full competition 6-1-01, 1-1-02 |
| Public Power | - Municipals - opt out of joining the ISO and are not required to follow same market rules. - City of Vernon opt in to ISO | Municipals and Coops can opt out of retail competition, but must follow all the wholesale market rules of the ISO. |
| Structural Separation | - Divestiture of fossil generation - Transmission Scheduling performed by the ISO - WireCo still in the merchant business | - No divestiture - IOUs must place at least 15% of the installed capacity up for entitlement auction - Separation of WireCo, PGC, and REP |
| Mandated Price Reduction | - Rates frozen at 1996 level - Grant 10% reduction to all | - Rates frozen at 1999 level - Grant 6% rate reduction for customer < 1 MW for 3 years |
| Stranded Cost Recovery | - Variable based on PX price of MWH - No audit after the collection goal | - Fixed charge / kwh - True up after two years |
| Revenue Cycle Services | Open on market opening | - Billing open on market opening - Metering opens to commercial and industrial customers 2004, Residential customer 2005 |
| Customer Education | Yes, before market opening | Yes, before market opening |
| Interconnection | - Rules unique to each utility - Difficult and cumbersome | - Standard system study process by ISO, between 54 and 254 days, averaging 60-90 days - Favorable Standard Interconnection Agreement approved by the PUC for all ERCOT utilities. Changes approved by consensus of both parties only. |
| Siting | Cumbersome | Function of water, gas, and transmission availability. |