NAPOLEONVILLE STORAGE #1

CAVERN CAPACITY DETERMINATION METHOD

WELL DATA:

# TUBLARS		WEIGHT		DEPTH		DESCRIPTION

16”			122.15 #/FT.		2,009 FT.		LAST CEMENTED

CASING

8 5/8”		  36 #/FT.		3,211 FT.		HANGING STRING

PROCEDURE:

- Run a pressure and temperature survey.   Tag the cavern bottom and determine the brine / gas interface level.

- Inject or withdraw a known amount of gas.

- Shut-in the well for several days, to allow stabilization.  Monitor the pressures and check the wellhead components for leakage.

- Run another pressure and temperature survey.  Calculate the cavern volume based on the data gathered, i.e. cavern depth, brine / gas interface level, and known gas volume injected or withdrawn.

- Prepare and submit the appropriate government reports.