The attached table summarizes the capital and operating cost estimate for the proposed 5 and 10 million gallon Butane and Natural Gasoline atmospheric storage tanks.  These costs are based on the following data:

- Tanks containing butane will be insulated with 3 inches of polyisocyanurate foam in two layers with a polar white metal jacket to maintain mechanical integrity of the foam and provide a vapor barrier.  The  gasoline tanks will be insulated similarly with 1 ½” of foam.

- Tanks will be designed, constructed (near Bakersfield CA) and tested to API Standard 620.

- Manways, stairway and platform,  nozzles, hatches, level indicator and controls and instrumentation to protect against over/under pressure is included.

- The foundation will comply with API standard 650 and the Butane tank foundation will be designed to prevent “heaving” due to water freezing.

- Cathodic protection (internal and external) will be provided.

- Gasoline and Butane will be delivered to the tank (and withdrawn) at a rate of 100,000 GPD for 180 days per year equating to 18 million gallons of product being stored.

- The product cooling system design inlet temperature is 120F for both butane and NGL and a storage temperature is 25 F and 70F respectively.

- Total Capital costs include installation.

- No costs for permitting are included.

- Operating costs are based on $2.50/MCF and $.08/KWH.