ETS Optimization – Weekly Report (Jan 03)

TRANSWESTERN

San Juan Lateral - WinFlow Model:

Finalizing the input data for the WinFlow model of the TW system from the suction side of the La Plata Station through the discharge side of the Gallup Station.  Hope to have completed by today the "tuned" transmission factors for this segment and both site rated horsepowers and head/acfm compressor data included such that the model can somewhat accurately predict the transport volumes based on actual pipe and compression capabilities.

##### NORTHERN NATURAL

Continental Plant Flow Re-route

The Beaver Continental processing plant restarted after shutting down in early December.  Since the receipt gas from Continental is now processed, it has been rerouted south to the Beaver station and compressed to the Bypass stream.  It had been routed north instead of south so that the Bushton processing plant could process it.

Bushton Processing Unplanned Shut Down

The Lean side of the Oneok Bushton processing plant had an unplanned shut down for about 8 hours on 12/31/01.  The potential for hydrocarbon liquids problems was analyzed.  The A line was reconfigured at Palmyra to help blend out the higher C6+ concentration.  No production was interrupted and no liquids problems have surfaced.

FLORIDA GAS

No Action

NORTHERN BORDER

No Action.

EOTT

No action

MISCELLANEOUS

### TGB (Transportadora de Gas Boliviano)

Preparing information requested by GTB with respect to  Line Break on their system.  Provided input concerning the desired set points that would trigger the closure of the mainline valves in the event of a break.