ETS Optimization – Weekly Report (Jan 24)

##### TRANSWESTERN

San Juan Lateral – Evaporative coolers.

Met with Planning and Gas Control to determine the operational impact of  the proposal.

The anticipated 10 mmcfd is expected to come from the hub. The proposed enhancement is expected to increase the total available HP at Bloomfield by 600-700HP but does not address the current pipeline efficiency decline on the Ignacio line.

TW San Juan

Extracted SCADA daily average history for the San Juan Lateral from 6/1/01 to 9/1/01 for assistance in design assumptions for the hydraulic model for this section.

##### NORTHERN NATURAL

NNG Operations Support

Provided guidance to the Canadian team on when to use thermal sizing and when to use fire sizing for relief valves on vessels.

Compression Variable O&M Cost

A presentation of the Power Services/Optimization effort to determine the variable cost of ETS compression will be presented to North Operations management on 01/24/02.  The potential benefit of expanding past work to include all units as a best practices tool will be reviewed.

##### FLORIDA GAS

# FGT - Excessive Pressure Drop Between Stations 11A & 12A

Visited Stations 11A & 12A to look at operations and measurements.

Noted that the actual flow measured and reported on SCADA - is not the true flow at Station 11A.

This is contaminated with additional flow due to unit recycling and station recycling.

Working with 12 months of data to determine effective roughness.

Developing transient model for the segment between stations 11 and 12.

This method can be used once we have more reliable flow data.

Met with Gas Control to try to determine surge conditions at station 11A. Awaiting further clarification.

###### Compressor Curves and Coefficients

Worked on determining coefficients applicable for detailed centrifugal stations - on the Gregg Simulator. Used an Excel Spreadsheet that provides the input required by Gregg's Winflow/Wintran models for definition of a centrifugal map.  This information is required if Centrifugal #2 or #4 compression modeling is to be used.

NORTHERN BORDER

NBPL/MW Compression Service - Joliet:

Continued working with Solar regarding a lease agreement for two Solar Saturns for a compression service between MW and NBPL.

EOTT

No Action

##### MISCELLANEOUS

HP Performance utility

Working with Waterloo to get the necessary unit data into the SCADA system.  Continuing to develop the software tools necessary to automatically analyze the data and report the results.

IT Tools Support

Working with IT Support to outline which programs I've developed are necessary for the NNG Planning team.

EMRT Support

Completed a two week rotation for the Emergency Management Response Team.