Exhibit 1

Existing Transactions and ET Fuel Costs

| Resource | Amount MW | Hour Limitation | Reliability | Term | Delivery Point | Scheduling | Minimum Delivery |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEPA | 11 | 1500 |  | None | Into Entergy | 1. Submit annual schedule by June 20 2. Monthly breakdown (by week) 10 days prior to beginning of month 3. Weekly breakdown (by hour) by preceding Wednesday |  |
| Cajun | 7 | NA | Firm System Capacity (as firm as native load) | 5/31/2010 | Into Entergy | Week ahead by COB Thursday Hourly changes with approval | At least 25% of contract capacity |
| EPI-Isis | 7 | NA | Unit Contingent, Transmission Contingent | 5/31/2010 | Into Entergy | 16 hours in advance |  |
| EPI-Ritchie | 1 | NA | Unit Contingent, Transmission Contingent | 5/31/2010 | Into Entergy | 16 hours in advance |  |

Gas Contracts

- SNS transportation agreement between Clarksdale and Texas Gas

Firm transportation agreement between Yazoo City and Mississippi Valley Gas (LDC)

EXHIBIT

Facilities

| CPU UNITS |  |  |  |
| --- | --- | --- | --- |
|  | Unit 9 | Unit 8/6 | Unit 7 |
| Unit Type | STAG Combined Cycle | Simple Cycle/CC | Conventional 400lb Steam |
| Min Capacity | 12.00 | 5/8 | 4.00 |
| Max Capacity | 22.00 | 12.7/6.8 | 7.50 |
| Min Heat Rate | 12,500 | 21,000/13,100 |  |
| Max Heat Rate | 11,500 | 17,300/11,900 | 19,500 |
| Preferred Min Run Time | 16.00 | 16.00 | 120.00 |
| Acceptable Min Run Time | 12.00 | 12.00 | 60.00 |
| Min Off Time |  |  |  |
| Cold Start Time | 2.00 | 3.00 | 12.00 |
| Warm Start Time | 1.00 | 2.00 | 8.00 |
| Hot Start Time | 0.50 | 1.00 | 2.00 |
| Ramp MW/min | 2.00 | 1.50 | 0.50 |
| Weekly Starts | 7 | 5 | 1 |
| Monthly Starts | 28 | 20 | 4 |
| Yearly Starts | 96 | 48 | 20 |
| Start Costs |  |  |  |
| VOM | $ 2.00 | $2.00 | $ 2.00 |
| Dual Fuel? | Yes | Yes | No |
| 2nd Fuel | LS #2 | LS #2 | -- |
| Dual Fuel Heat Rate |  |  | -- |
| Dual Fuel VOM | $2.00 | $2.00 | -- |
| Historical Availability |  |  |  |
| Project Availability |  |  |  |

| Yazoo City UNITS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Unit # CT1 | Unit # ST2 | Unit # ST3 | Unit # CT1/ST3 | TOTAL |
| Unit Type | SIMPLE CYCLE | 400 LB STEAM | 600 LB STEAM | COMBINED CYCLE | SYSTEM |
| Min Capacity | 3.00 | 2.00 | 6.00 | 15.00 |  |
| Max Capacity | 14.50 | 5.00 | 12.00 | 23.00 | 32.00 |
| Min Heat Rate | ? | ? | ? | 12,000 |  |
| Max Heat Rate | 17,000 | 16,000 | 15,000 | 10,700 | 12,000 |
| Preferred Min Run Time |  | 16.00 | 120.00 | 16.00 |  |
| Accepwtable Min Run Time |  | 12.00 | 60.00 | 12.00 |  |
| Min Off Time |  |  |  |  |  |
| Cold Start Time | 0.17 | 1.00 | 6.00 | .17/2.00 |  |
| Warm Start Time | 0.17 | 0.50 | 1.00 | .17/.50 |  |
| Hot Start Time | 0.17 | 0.10 | 0.50 | .17/.10 |  |
| Ramp MW/min | 2.00 | 1.00 | 1.00 | 2.00/1.00 |  |
| Weekly Starts | 14 | 7 | 1 | 7 |  |
| Monthly Starts | 31 | 31 | 4 | 31 |  |
| Yearly Starts |  |  |  |  |  |
| Start Costs |  |  |  |  |  |
| VOM | $ 2.00 | $ 2.00 | $ 2.00 | $ 2.00 |  |
| Dual Fuel? | YES | NO | NO | YES |  |
| 2nd Fuel | LS #2 |  |  | LS #2 |  |
| Dual Fuel Heat Rate |  |  |  |  |  |
| Dual Fuel VOM | $ 2.00 |  |  | $ 2.00 |  |
| Historical Availability |  |  |  |  |  |
| Projected Availability |  |  |  |  |  |
| HRSG is rated at 95,000 lbs./hr and conventional boiler is rated at 120,000 lbs./hr. The two steam turbines are supplied from a common header off these two boilers. ST2 cannot be operated alone. |  |  |  |  |  |
| Availability factors are as of June 15, 2001, unless otherwise updated periodically. |  |  |  |  |  |

Exhibit XX

MDEA EPMI Incentive Fee Calculation

MDEA and EPMI will share the Profits and Savings according to the following percentages:

- MDEA 60% and

- EPMI 40%.

Profit per MWh is calculated as the sum of the product of MWh sold and the realized market price less the Target Production Cost.

Savings per MWh is calculated as the positive difference between (a) the [Target Production Cost] for units that would have run if market power were not purchased to serve system load, and (b) the delivered cost of market power purchased to serve system load.

Total Daily Savings is the product of (a) the Savings per MWh calculated for the day, times (b) the total MWh for the same day of market power purchased to serve system load.

[Production Cost] = [(delivered gas cost x unit heat rate) + VOM] + [unit start charge/MWh block]

Delivered gas cost is the daily, midpoint posting for ________ under the Gas Daily Daily Price Survey.

Unit heat rate is the heat rate for each effected unit as specified in Exhibit “1”, Facilities.

VOM is the variable operation and maintenance cost for operating each effected unit as specified in Exhibit “1”, Facilities.

Unit Start Charge is the cost to start-up each effected unit as specified in Exhibit “1”, Facilities, and includes (a) start-up gas quantity required for the effected unit times the delivered gas cost.

Exhibit

Target Production Cost Calculation

The Target Production Cost (TPC) is calculated based on the cost of generating power to meet the MDEA day-ahead load forecast after adding long term contract power.  The generated power is calculated by optimizing the available generating assets for heat rate.  The TPC is calculated for each hour then averaged for the day.  Generation is added during peak times for 16 hours if possible with a minimum run of 12 hours.

The gas cost per MW is calculated based on the assumed heat rates and start up costs for each unit.  The initial valuse are based on data received from MDEA. As more current data is received, the heat rate and start-up costs can be adjusted accordingly.  The other cost component is the fixed O&M cost per MW.  The following table lists the assumptions used to determine heat rate, start-up and run costs.

The gas cost is determined by the delivered price to each city.  The gas cost is negotiated on a day-ahead basis with the gas suppliers.  Similar calculation will be made for fuel oil.

The load forecast is a correlation based on historical load data and forecast weather conditions.  A software package called MetrixND is used to calculate the forecast for a two-week forward period each day.

An Excel spreadsheet application is used to calculate the TPC based on all of the above information.  This spreadsheet has macros that collect the load forecast data and the unit data.   The first step is to load the forecast load data.  Once this data in input, generation is added until the forecast load is met.  The generation is added based on the available units with the best heat rates.  The order of preference if all units are available is:  CD9, YZ1/3, CD8/6, YZ2, CD7.  The unit availability sheet faxed to EPMI each day is used to determine which units are available.

A unit can only be added if its minimum MW output is at least as much as the requied generation.  For example if the forecast is for 25MW load and the long-term contracts fill in 20MW, the difference if 5MW.  The CD9, YZ1/3 and CD8/6 units cannot be used since they all have a minimum MW rating of more than 5MW.  Therefore YZ2 or CD7 would have to be used.

If the required MWs are in the range of a generator it is added by itself.  For instance, if the forecast is 35MW and the long-term contracts fill in 17MW, the difference is 18MW.  CD9 would be added to the generation stack at 18MW.

If two or more generators are needed to meet the load, the heat rates are optimized.  For example if the forecast predicts a 45 MW load and the long-term contracts cover 10MW, there is still 35 MWs that need to be filled.  The first unit to be added is CD9.  CD9 adds a maximum of 22 MWs leaving 13 MW required.  The next unit to be added would be YZ1/3.  The outputs of the two generators are then adjusted so that the combined heat rate is lowest for an output of 35 MW.

Once the generation is added to the stack and the load is met, the TPC is calculated for each run hour.  The calculations for determining the TPC follow:

As an example, CD9 is set to generate 20MW for 16 hours and the gas price is $4.35/MMBtu.

Gas Price = $4.35/MMBtu

MWhr/hr = 20 MWhr

Heat Rate for CD9 at 20 MW = 11.6 MMBtu/MWhr

Run Time = 16 hrs.

Start-up cost for CD9 = $1500

Fixed O&M for CD9 = $2.00 /MWhr

Gas Cost/MWhr 			=  $4.35/MMBtu * 11.6MMBtu/MWhr

=  $50.34/MWhr

Amortized Start-up Cost/MWHr	=  $1500/(16Hrs*20MWhr)

=  $4.69/MWhr

Fixed O&M for CD9/MWhr		= $2.00/MWhr

These are then added to get the total cost per MWhr.

Gen Cost/MWhr 			= Gas Cost + Amortized Start-up Cost + Fixed O&M Cost 					= $50.34 + $4.69 + $2.00

= $57.03

For a 16 hour run, the costs are averaged for the entire run and the TPC is determined.  In this case, 20MW are generated for each hour so the TPC would be  $57.03.

If more than one generator is running, a weighted average heat rate is used to determine the TPC using the calculations above.

A sample screen print of the TPC calculation spreadsheet is shown below:

The sheet shows at the top left the gas price for the day for each city, the date of the forecast, the TPC price, which generators are committed and how many MWs are dispatched for the day with the average heat rate.  To the right is a chart showing the forecast load with the contract and generation stack required to meet the load.  At the bottom of the sheet is an hour by hour table of the load forecast, the contract MWs, total generation, total generation costs and generation costs per MWhr.  A copy of this sheet will be saved for each day until a database with all of the data can be created.

Exhibit

Invoicing and Payment Schedule

| Invoicing Date | Payment Date |
| --- | --- |
| July 5, 2001 | July 13, 2001 |
| August 9, 2001 | August 17, 2001 |
| September 6, 2001 | September 14, 2001 |
| October 4, 2001 | October 12, 2001 |
| November 8, 2001 | November 16, 2001 |
| December 6, 2001 | December 14, 2001 |
| January 3, 2002 | January 11, 2002 |
| February 7, 2002 | February 15, 2002 |
| March 7, 2002 | March 15, 2002 |
| April 4, 2002 | April 12, 2002 |
| May 9, 2002 | May 17, 2002 |
| June 6, 2002 | June 14, 2002 |
| July 4, 2002 | July 12, 2002 |
| August 8, 2002 | August 16, 2002 |
| September 5, 2002 | September 13, 2002 |
| October 3, 2002 | October 11, 2002 |
| November 7, 2002 | November 15, 2002 |
| December 5, 2002 | December 13, 2002 |
| January 9, 2003 | January 17, 2003 |
| February 6, 2003 | February 14, 2003 |
| March 6, 2003 | March 14, 2003 |
| April 3, 2003 | April 11, 2003 |
| May 8, 2003 | May 16, 2003 |
| June 5, 2003 | June 13, 2003 |

Exhibit

Cancellation Fee Schedule

| Jul-01 | $ 276,414 |
| --- | --- |
| Aug-01 | $ 265,254 |
| Sep-01 | $ 254,019 |
| Oct-01 | $ 242,710 |
| Nov-01 | $ 231,325 |
| Dec-01 | $ 219,864 |
| Jan-02 | $ 208,327 |
| Feb-02 | $ 196,713 |
| Mar-02 | $ 185,022 |
| Apr-02 | $ 173,253 |
| May-02 | $ 161,405 |
| Jun-02 | $ 149,478 |
| Jul-02 | $ 137,472 |
| Aug-02 | $ 125,385 |
| Sep-02 | $ 113,218 |
| Oct-02 | $ 100,970 |
| Nov-02 | $ 88,640 |
| Dec-02 | $ 76,229 |
| Jan-03 | $ 63,734 |
| Feb-03 | $ 51,156 |
| Mar-03 | $ 38,494 |
| Apr-03 | $ 25,748 |
| May-03 | $ 12,917 |