- FLORIDA ENERGY MARKET OVERVIEW

In order to address concerns with regard to possible Florida natural gas and electricity shortages in the upcoming year, the gas fundamentals group has conducted extensive research of the Florida energy market. We analyzed market fundamentals, historical trends in supply and consumption of natural gas.  We have also identified possible developments in the Florida energy market over the next 1-2 year period.

## Introduction

Florida is the nation’s fourth most populous state with a population of 15.5 million people. The population growth rate is expected to be 1.81% in the year 2001 and an average of 1.54% over the period of 2000-2007.  The Census Bureau projects that Florida will become the nation’s third largest state between 2015 and 2020.

In recent years the prime source of growth in energy demand came from residential consumers. The number of private housing starts is expected to grow on a steady rate by 145,000 – 150,000 every year over the next 5 – 7 years. The total number of Florida households is currently around 6.12 million and is expected to grow to 6.89 million by 2007 (or 1.8% on average every year).

The Florida energy market is dominated by vertically integrated utilities that maintain substantial generation resources (ranked 3rd in generation capacity in the US). These utilities have weak economic incentives to provide transmission access to non-affiliated merchants on the same basis as they do to their affiliated merchants.

Florida law restricts construction of merchant power plants by independent power producers, thus complicating the development of a competitive market for the sale of electricity.

- Florida power market fundamentals.

Florida is a net importer of energy resources, including electricity. The primary source of electric generation in the state is coal, followed by natural gas and heating oil.

# Florida's electric utilities consumed 10.25% of the total natural gas used to generate electricity in the U.S. in 1999. Florida is isolated from the rest of the South - Eastern power grid because of transmission constraints in northern Florida. Florida is a net importer of baseload power during the summer and winter.  Importing capabilities are limited to about 3,600 MW/hour.  Normal baseload flow is 2,418 MW/hour, but flows reach 3,900 MW/hour during peak summer load.

Most of the power load increase over the last few years was covered by the

introduction of new power plants fired by natural gas and heating oil.

Currently, Florida’s generation pool has the following structure:

Florida Power Stack (MW)

|  | 2000 | 2000 | 2001 | 2001 | 2002 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Summer | Winter | Summer | Winter | Summer | Winter |
| Waste | 3,022 | 3,146 | 3,022 | 3,146 | 3,022 | 3,146 |
| Coal | 9,987 | 10,186 | 9,987 | 10,186 | 9,987 | 10,186 |
| Nuke | 3,869 | 3,966 | 3,869 | 3,966 | 3,869 | 3,966 |
| No. 6 Only | 2,604 | 2,634 | 2,604 | 2,634 | 2,604 | 2,634 |
| No. 6 to NG | 6,796 | 6,852 | 6,796 | 6,852 | 6,796 | 6,852 |
| NG to No. 6 | 3,668 | 3,743 | 3,668 | 3,743 | 3,668 | 3,743 |
| NG Only | 1,060 | 1,100 | 1,060 | 1,100 | 1,060 | 1,100 |
| NG to No. 2 | 3,783 | 4,274 | 7,335 | 7,826 | 14,850 | 15,341 |
| No.2 Only | 2,779 | 3,296 | 2,779 | 3,296 | 2,779 | 3,296 |
| Total | 37,568 | 39,197 | 41,120 | 42,749 | 48,635 | 50,264 |

Fuel used for FL Power Generation as a Percentage of Total

|  | 2000 | 2000 | 2001 | 2001 | 2002 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
|  | %Sum | %Win | %Sum | %Win | %Sum | %Win |
| Waste | 8% | 8% | 7% | 7% | 6% | 6% |
| Coal | 27% | 26% | 24% | 24% | 21% | 20% |
| Nuke | 10% | 10% | 9% | 9% | 8% | 8% |
| No. 6 Only | 7% | 7% | 6% | 6% | 5% | 5% |
| No. 6 to NG | 18% | 17% | 17% | 16% | 14% | 14% |
| NG to No. 6 | 10% | 10% | 9% | 9% | 8% | 7% |
| NG Only | 3% | 3% | 3% | 3% | 2% | 2% |
| NG to No. 2 | 10% | 11% | 18% | 18% | 31% | 31% |
| No.2 Only | 7% | 8% | 7% | 8% | 6% | 7% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% |

These charts reflect Florida’s generation capacity by primary fuel type, as well as fuel switching capabilities.   The stack shows that 88% percent of Florida’s gas-fired power plants can switch to heating or residual oil, while 55% of heating and residual oil – fired plants have a capability of switching to natural gas.

Peak power load was 38,730 MW in 1998 and 37,493 MW in 1999. It is projected to be 37,728 Mw this year.  Forecasting the power load is crucial to understanding gas consumption in the state of Florida.

Industrial, commercial and residential gas consumption combined together comprise less than a half of the average daily consumed gas volumes.  These areas have shown no signs of significant growth over the last few years. At the same time, electric natural gas consumption is a significant part of total gas consumption. Gas fired electric generation capacity is expected to double by 2007 to satisfy growing electricity needs.

As displayed in the following table, generation capacity is expected to grow by 3,552 MW in 2001 and another 7,515 MW in 2002.  It is important to note that all of the gas-fired plants scheduled to be in service in the next two years have the ability to switch to No. 2 oil.

| Date | Plant Name | Gas Fired Generation (MW per hour) | Gas (MMBtu) |
| --- | --- | --- | --- |
| Jan-01 | Fort Myers Repower Phase I | 894 | 205,978 |
| Jan-01 | Lauderdale ST4 | 10 | 2,304 |
| Jan-01 | Lauderdale ST5 | 10 | 2,304 |
| Jan-01 | Kelly Repower | 110 | 25,344 |
| Jan-01 | Brandy Branch Generating Station | 382 | 88,013 |
| Jan-01 | CT Peaker | 177 | 40,781 |
| Apr-01 | Northside Unit 2 (Repower) | 177 | 40,781 |
| Jun-01 | Marathon | 4 | 922 |
| Jun-01 | Martin County Expansion | 340 | 78,336 |
| Jun-01 | Cane Island 3 | 250 | 57,600 |
| Jun-01 | Santa Rosa | 240 | 55,296 |
| Jun-01 | Auburndale Expansion | 100 | 23,040 |
| Aug-01 | Crystal Rivers Coal Expansion | 17 | 3,917 |
| Dec-01 | Brandy Branch Generating Station | 191 | 44,006 |
| Dec-01 | CT Peakers (3 units) | 480 | 110,592 |
| Dec-01 | Martin County Generation Station | 170 | 39,168 |
|  | Total 2001 | 3,552 | 818,382 |
|  | Projected 2002 (27 Plants) | 7,515 | 1,541,786 |

Taking into account the fact that most of the planned generation capacity coming online in upcoming years is natural gas fired, the importance of sufficient natural gas supplies in the future cannot be underestimated.

- Florida natural gas transmission infrastructure

One of the major concerns with regard to the Florida energy market is whether current pipeline infrastructure will be able to meet the growing demand for natural gas.

Current natural gas transmission capabilities into Florida are limited to Florida Gas Transmission pipeline (FGT).  FGT flows natural gas from South Texas and Louisiana, through Mississippi and Alabama, into Florida.   Pipeline capacity along FGT varies due to market demand and pipeline issues such as maintenance and operational constraints.  During the peak load summer season, FGT lists their available capacity at 1.575 BCF/d.  This represents the maximum that FGT will release for scheduling purposes.  As demand decreases during the winter months, FGT takes the opportunity to perform yearly maintenance and remedy any other issues that the pipeline anticipates.  During the winter season, available pipeline capacity in the Florida market zone drops to approximately 1.45 BCF/d.

TX

AL

LA

- Although current FGT capacity is not enough to meet the peaking summer demand for natural gas, the pipeline is working on two expansion projects.

The first project, Phase IV expansion, is going to increase FGT's deliverability into Florida by 0.2 Bcf/day. According to pipeline management, the plan is going as scheduled and will be completed by 05/01/2001.

The second project, Phase V expansion, will bring the pipeline’s total deliverability into the Florida market area to 2.2 bcf/day. This project has received preliminary approval from FERC and is to be completed by 04/01/2002.

In addition to Florida Gas Transmission's expansion plans, there were competing pipeline projects to bring gas from Alabama to Florida: Gulfstream , Buccaneer and Sawgrass pipelines.

Sawgrass was to enter Florida through the panhandle and track south to the market area.  This project, however, never saw the light of day.  Several months after initially unveiling the project, on Oct. 28, 1999, Duke announced plans for a joint venture with Williams, creating the Buccaneer pipeline.

In the latest shakeout of Florida pipeline projects, Williams and Duke Energy, purchased the Gulfstream Natural Gas System from Coastal. Taking into account that Gulfstream was the most developed project, they scrapped the Buccaneer project, leaving only one pipeline as a potential competitor to the FGT monopoly.

Gulfstream is designed to run across the Gulf of Mexico from a point near Mobile, Alabama, coming onshore in the Tampa By area and proceeding east to West Palm Beach.

The $1.6 billion Gulstream pipeline project has achieved two key milestones in the FERC approval process: preliminary determination on non-environmental issues and a favorable draft environmental impact statement. It is anticipated that the project will get final FERC approval in first quarter of 2001. The projected in-service date is June 2002. Gulfstream has precedent agreements with 10 large Florida utilities and power-generation facilities representing long-term commitments for the majority of its 1.1 billion cubic feet of natural gas per day capacity. Gulfstream's proposed 744-mile pipeline is designed to primarily serve Florida utilities and power-generation facilities that plan on using high-efficiency, natural gas turbines to meet the incremental demand for electric energy.  If all projects go as scheduled, the total natural gas deliverability into Florida will reach a summer capacity of 1.775 Bcf/day by May 2001, 2.2 Bcf/day by April 2002 and 3.3 bcf/day by June 2002.

Conclusion

In the event of severe natural gas shortages, or market conditions where natural gas is the most expensive fuel, Florida's electric utilities use heating and residual oil. Due to Florida’s proximity to the Gulf Coast and other regions with significant heating oil reserves, currently there is not a heating oil deliverability problem into Florida.  According to the traders at EGM, the availability of heating oil and residual fuel oil from the Gulf Coast as well as overseas markets (Venezuela) present the Florida utilities with fuel switching options in case of natural gas shortages.  However, the supplies of heating oil will be severely impacted in the event of extended cold weather in Europe.  Therefore it will be necessary to monitor the European weather forecast in order to predict potential heating oil supply constraints in Florida this winter.

The following chart illustrates the scenario in which gas prices fall below both heating and residual oil, and switching exclusively to natural gas occurs.  It is important to note that this situation assumes that all gas-fired units are generating power twenty- four hours a day, seven days a week.  The analysis includes new plant burns as well as pipeline expansion for Jan 2001 – Dec 2002.

| Scenario #1 – Natural Gas Cheapest Fuel (Peak Load) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max. Gas Fired Generation (MW per hour) | Gas Consumption (MMBtu) | FGT Pipeline Capacity | Other Consumption* | FGT Capacity MMBtu/D | Balance (MMcf/D) |
| Dec-00 | 15,307 | 3,526,733 | 1,450,000 | 521,065 | 928,935 | (2,597,798) |
| Jan-01 | 16,890 | 3,891,456 | 1,450,000 | 560,871 | 889,129 | (3,002,327) |
| Feb-01 | 16,890 | 3,891,456 | 1,450,000 | 542,310 | 907,690 | (2,983,766) |
| Mar-01 | 16,890 | 3,891,456 | 1,450,000 | 587,613 | 862,387 | (3,029,069) |
| Apr-01 | 17,067 | 3,932,237 | 1,450,000 | 571,233 | 878,767 | (3,053,470) |
| May-01 | 17,067 | 3,932,237 | 1,575,000 | 498,742 | 1,076,258 | (2,855,979) |
| Jun-01 | 18,001 | 4,147,430 | 1,575,000 | 510,867 | 1,064,133 | (3,083,297) |
| Jul-01 | 18,001 | 4,147,430 | 1,775,000 | 500,548 | 1,274,452 | (2,872,979) |
| Aug-01 | 18,018 | 4,151,347 | 1,775,000 | 510,839 | 1,264,161 | (2,887,186) |
| Sep-01 | 18,018 | 4,151,347 | 1,775,000 | 475,833 | 1,299,167 | (2,852,181) |
| Oct-01 | 18,018 | 4,151,347 | 1,775,000 | 494,129 | 1,280,871 | (2,870,476) |
| Nov-01 | 18,018 | 4,151,347 | 1,775,000 | 511,200 | 1,263,800 | (2,887,547) |
| Dec-01 | 18,859 | 4,345,114 | 1,650,000 | 529,000 | 1,121,000 | (3,224,114) |
| Jan-02 | 21,401 | 4,776,703 | 1,650,000 | 570,967 | 1,079,033 | (3,697,670) |
| Feb-02 | 21,401 | 4,776,703 | 1,650,000 | 552,072 | 1,097,928 | (3,678,775) |
| Mar-02 | 21,401 | 4,776,703 | 1,650,000 | 598,190 | 1,051,810 | (3,724,893) |
| Apr-02 | 22,251 | 4,966,423 | 1,650,000 | 581,516 | 1,068,484 | (3,897,939) |
| May-02 | 22,251 | 4,966,423 | 2,200,000 | 507,719 | 1,692,281 | (3,274,142) |
| Jun-02 | 26,375 | 5,886,900 | 2,200,000 | 520,062 | 1,679,938 | (4,206,962) |
| Jul-02 | 26,375 | 5,886,900 | 3,300,000 | 509,558 | 2,790,442 | (3,096,458) |
| Aug-02 | 26,375 | 5,886,900 | 3,300,000 | 520,034 | 2,779,966 | (3,106,934) |
| Sep-02 | 26,375 | 5,886,900 | 3,300,000 | 484,398 | 2,815,602 | (3,071,298) |
| Oct-02 | 26,375 | 5,886,900 | 3,300,000 | 503,023 | 2,796,977 | (3,089,923) |
| Nov-02 | 26,375 | 5,886,900 | 3,300,000 | 520,402 | 2,779,598 | (3,107,302) |
| Dec-02 | 26,375 | 5,886,900 | 3,175,000 | 538,522 | 2,636,478 | (3,250,422) |
| *Includes baseload industrial, residential, and commercial consumption |  |  |  |  |  |  |

In this peak scenario, fuel switching to natural gas results in severe shortages.  Although this situation seems unlikely in today’s price environment, it would not be unreasonable to forecast natural gas price declines along with increases in the cost of heating or residual oil.  One such scenario could be explained by increasing costs associated with new federal emissions requirements.  Under these conditions, Florida market area gas would rise to the equivalent of the higher priced alternate fuel, No. 2 Oil.

The following table represents a more likely scenario.  This scenario assumes that No. 6 is the cheapest fuel, followed by natural gas, and then finally No. 2.  This scenario also assumes that all gas-fired units are generating power twenty-four hours a day, seven days a week.

| Scenario #2 - No.2>NG>No.6 (Peak Load) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max. Gas Fired Generation (MW per hour) | Gas Consumption (MMBtu) | FGT Pipeline Capacity | Other Consumption* | FGT Capacity MMBtu/D | Balance (MMcf/D) |
| Dec-00 | 4,843 | 1,080,958 | 1,450,000 | 521,065 | 928,935 | (152,023) |
| Jan-01 | 6,426 | 1,434,283 | 1,450,000 | 560,871 | 889,129 | (545,154) |
| Feb-01 | 6,426 | 1,434,283 | 1,450,000 | 542,310 | 907,690 | (526,594) |
| Mar-01 | 6,426 | 1,434,283 | 1,450,000 | 587,613 | 862,387 | (571,896) |
| Apr-01 | 6,603 | 1,473,790 | 1,450,000 | 571,233 | 878,767 | (595,023) |
| May-01 | 6,603 | 1,473,790 | 1,575,000 | 498,742 | 1,076,258 | (397,532) |
| Jun-01 | 7,537 | 1,682,258 | 1,575,000 | 510,867 | 1,064,133 | (618,125) |
| Jul-01 | 7,537 | 1,682,258 | 1,775,000 | 500,548 | 1,274,452 | (407,807) |
| Aug-01 | 7,554 | 1,686,053 | 1,775,000 | 510,839 | 1,264,161 | (421,892) |
| Sep-01 | 7,554 | 1,686,053 | 1,775,000 | 475,833 | 1,299,167 | (386,886) |
| Oct-01 | 7,554 | 1,686,053 | 1,775,000 | 494,129 | 1,280,871 | (405,182) |
| Nov-01 | 7,554 | 1,686,053 | 1,775,000 | 511,200 | 1,263,800 | (422,253) |
| Dec-01 | 8,395 | 1,873,764 | 1,650,000 | 529,000 | 1,121,000 | (752,764) |
| Jan-02 | 10,937 | 2,441,138 | 1,650,000 | 570,967 | 1,079,033 | (1,362,105) |
| Feb-02 | 10,937 | 2,441,138 | 1,650,000 | 552,072 | 1,097,928 | (1,343,210) |
| Mar-02 | 10,937 | 2,441,138 | 1,650,000 | 598,190 | 1,051,810 | (1,389,328) |
| Apr-02 | 11,787 | 2,630,858 | 1,650,000 | 581,516 | 1,068,484 | (1,562,374) |
| May-02 | 11,787 | 2,630,858 | 2,200,000 | 507,719 | 1,692,281 | (938,578) |
| Jun-02 | 15,911 | 3,551,335 | 2,200,000 | 520,062 | 1,679,938 | (1,871,397) |
| Jul-02 | 15,911 | 3,551,335 | 3,300,000 | 509,558 | 2,790,442 | (760,893) |
| Aug-02 | 15,911 | 3,551,335 | 3,300,000 | 520,034 | 2,779,966 | (771,369) |
| Sep-02 | 15,911 | 3,551,335 | 3,300,000 | 484,398 | 2,815,602 | (735,734) |
| Oct-02 | 15,911 | 3,551,335 | 3,300,000 | 503,023 | 2,796,977 | (754,359) |
| Nov-02 | 15,911 | 3,551,335 | 3,300,000 | 520,402 | 2,779,598 | (771,737) |
| Dec-02 | 15,911 | 3,551,335 | 3,175,000 | 538,522 | 2,636,478 | (914,857) |
| *Includes baseload industrial, residential, and commercial consumption |  |  |  |  |  |  |

In this situation, gas shortages still exist at peak generation.  In order to meet peak demand, power imports would need to increase or No. 2 would be substituted in the place of gas.  This situation appears likely during peak times from July 2001 through September 2001.  This should lead to strong demand for delivered gas and a strong basis market in Florida during the summer of 2001.

Since we are currently unable to identify the exact hourly generation mix in the Florida market, we need to further analyze the variables that influence fuel switching and the curtailment of generation from lower priced baseload units, mainly No. 6 oil.  An example of this phenomenon is currently taking place.  Under current market conditions, with natural gas as the most expensive fuel, gas-fired generation should only occur when power loads extend beyond 38,097 MW, which is greater than the peak load during summer.  However, based on FGT pipeline flow data, it is clear that at least .5 BCF/d is being consumed for power generation.

Next Steps

As part of the research of Florida’s energy market, the fundamentals group is developing a model to forecast power load and gas burn in Florida (based on heating and cooling degree-days and fuel switching economics).  In order to complete this analysis, we are currently compiling emissions credit data, as well as FGT daily gas flow data.

Once this data is centralized, the model will be complete and back testing will begin.   One output of this model will be an accurate hourly load forecast and generation stack model.  From this data, it will be possible to forecast accurate gas burns and identify periods of gas shortages during baseload and peak times.