Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Sat., March 16, 2002  (Moore, Alvarado, Barnes, Coash, Cochran)

| 0950 | Owatonna | The unit just went down. I’m going to leave it down at this time. It looks like it might have gone down on low suction. The system is turning around pretty good at this time. Cochran |
| --- | --- | --- |
| 0950 | Farmington | Dropped unit #6 because we’re starting to build pretty good there, no demand. Cochran |
| 1215 | North Branch | Just dropped unit #3, no demand. Cochran |
| 1230 | Ogden | Lost unit #15. I am unable to reset it at this time. Probably will go ahead and leave it down but I got a hold of Chris. He is going out to reset it in case we need it. Cochran |
| 1258 | Northrop | Received a call from Larry Crawford. He let me know that he had gotten a page. We looked at the Northrop station and scanned it. It did come up a dehy failure alarm. He will go out and check it. It may take him 45 minutes to an hour to get there. Moore |
| 1320 | Farmington | Dropped unit #1 going west, no demand. Cochran |
| 1344 | Northrop | Received a call back from Larry. He’s got the alarm reset. He said it was due to low air pressure. Moore |
| 1345 | Beatrice | Dropped unit #21 going “D” to “D”, no demand. Cochran |
| 1420 | Faribault | Just dropped the unit, no demand. Cochran |
| 1530 | Reagan Co. 2 | Earlier this morning I had James go out. We had lost unit 1 there about 2:45 or 3:00 this morning it looks like. I got a call from JL Davis to let us know. Sent James out. He’s been working on that all day. Come to find out that there is a freeze up at the processing plant at JL Davis. Right now what they had to do is bypass the plant until the freeze up has been thawed. It could take possibly up to 24 hours so we are bypassed there at this time and it flowing. The unit is back on line but we are not seeing a flow rate. I did call Gary Spraggins to let him now and to make sure that it was OK for this to flow like it is. He said it was OK. James did tell me that when they get that thawed and the processing unit going again that it should open back up automatically and start going through there. He said if we don’t see flow within the 24 hours to give him a call. Moore |
| 1540 |  | I received AC power fail alarms at Beaver Co. 6, Ellis Co. 3 and Ellis Co.1. I got a hold of Gary the on call person. We also contacted Beaver to make sure there wasn’t a problem there, there is not. Gary is still trying to get a hold of the power company, says it has been busy. He feels like they’ve got something out. He will notify us as soon as he finds out something. Moore |
| 1638 |  | All AC power fail alarms at Ellis 1 and 3, and Beaver Co. 6 are clear. Moore |
| 1655 | Guthrie | Just dropped a unit, no demand. Cochran |
| 1700 | Guthrie | Just dropped a unit, no demand. Cochran |
| 1700 |  | Just dropped unit #5 at Farmington, no demand. Cochran |
| 1725 | Ellis Co. 1 | Received a call from Gary. He went out and checked the station. He said everything looks good there. We looked over the station Ellis 3 and Beaver 6 also. Everything appears to be working fine. Moore |
| 1730 | JL Davis Residue | I started seeing flow rate there again. I guess the freeze up may be thawed. We’ll keep an eye on it. Moore |

1830 to 0630 Shift Sat., March 16, 2002  (Munson, Welsh, Cummings, Muntz, Davis)

| 2120 | PG&E | Gas control from PG&E called and requested that we increase the flow rate at Topock. They said they had finished with their work that they were performing on their pipe inspection. They would like to increase to the proper rate to rate out for the day. Munson |
| --- | --- | --- |
| 2337 | Clark Co. 1 | Unit 1 is down. Rick Preisner called in and said it had dumped the crankcase oil out on the floor. He’s going to have to call some help out. He said he would give us a call when they get it ready to come back on. Muntz |
| 0200 | St. Paul 1Q | The pressures and temperature all went to zero, the volume and everything else flatlined. I spoke to Tim Pietsch in Farmington. He said that NSP has a backup measurement at the site and that he will take their measurement and call it into our measurement department Monday and try and fix his communications. Cummings |
| 0239 | Red Cedar | I noticed the CO2 running a little high at Red Cedar. I gave the plant a call. They are aware of it and working on trying to bring it back down within spec. Munson |
| 0320 | Clark Co. 1 | Rick called in and said the unit is back on the line. They found a broken pipe nipple that caused it to leak the oil out. They got everything repaired and they are heading back home. Muntz |
| 0514 | Midkif | Flow rate is back up to 35. Going to go ahead and increase the flow rate up 5 at a time until we get to a 55 rate. Muntz |

slb