Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Tues., March 12, 2002  (Hardy, Henley, Spain, Monden, Muntz)

| 0745 | Perryton Barlow | Units 1 and 2 are down at Perryton Barlow. The flow rate has gone to zero in preparation for the “A” line repair south of Beaver from block valve 10 into the Beaver yard. Muntz |
| --- | --- | --- |
| 0802 | Northrop | Units 1 and 2 are shut down in preparation for today’s outage on the “A” line south of Beaver. Muntz |
| 0810 | Needles | Randy called in and said they will be calibrating transmitters at SoCal Needles interconnect the majority of the day. Monden |
| 0828 | Station 3 | They’ve shut down unit #3 to do their minor maintenance on the unit. Estimated time is four hours. Monden |
| 0830 | Cunningham | I just called and gave Lynn an order to shut the injection rate in. We were doing about 256. We’re calling for 100 million withdrawal today in the plan but I’m just going to shut it in for now. We’re supposed to get 140 less coming from Beaver. We’ll see how it works out, see if we can get around 950 through Clifton. Right now we’re doing about 890. Spain |
| 0830 |  | I spoke to Darrel at Macksville. I asked him to coordinate with Cunningham, when he sees the injection rate go to zero to go ahead and bypass the station. We got scheduled about 360 coming from the south. It’s not enough to support the Macksville turbines. Spain |
| 0845 | Atoka 3 | Unit 762 down for minor maintenance. They’re going to work on exhaust, change studs on compressor valve caps. Spoke with Jerry. Monden |
| 0914 | Crawford | Unit #2 is on line at 0840. Spoke with Bennie. Monden |
| 0926 | NGPL Eddy | Spoke with Darla. Their field had called her and said they are having problems with a coupling on a unit at Eddy and that they will need an estimated four hours. During that four hours, nom is 25.8 MMBtu, they will move that gas and give it to us at Winkler for the four-hour period and then move it back up to Eddy and then cut back at Winkler. I had no problem with this at this time. Monden |
| 1033 | WFS Milagro | Spoke with Mike in their gas control. He said that they have a unit down at Ignacio and they will try to get the majority of our gas through Milagro. Monden |
| 1043 | Beaver Continental | Continental has a 1 million order on the “A” line. Currently we are working on an outage south of Beaver plant on the “A” line. I spoke with Gary and he said it would be OK to have them go ahead and shut in on the “A” line. I called Ronnie Ashcroft back and let him know that they could go ahead and shut down on the “A” line. Currently we show an order of 31 on the “B”. We’ll go ahead and keep the “B” as the order. Muntz |
| 1030 | Clifton | I started unit #31 on the “E” line service. Spain |
| 1308 | WFS | It is running short by approximately 40 million on the nom. I spoke with Mike Ellington in their gas control. He told me that he has a lot of substations down and that they would get them back up as soon as possible. They are doing some pigging. Also they still have the unit down at Ignacio which I couldn’t get any extra gas up there at Ignacio. Monden |
| 1308 |  | PNM called, spoke with Paul. He asked if we could give him some extra gas for awhile until WFS got some of their HP back up. He said he was hurting for some gas for his deliveries. I told him we’d try to accommodate him until the HP with WFS comes back up but I stressed that we still had to come in on the nom and the 5 million under. Monden |
| 1330 | Burdett | Floyd just called in. He informed me that they had a BSD. I called Holcomb and informed them that they might see a little rollback. Floyd said he should have the station back on line within 15-20 minutes so it shouldn’t cause any problems. Spain |
| 1400 | P3 | Asked Denny on the P3 team to put on another small unit. I just can’t make it by pushing the gas up from Roswell. I have concerns of pressuring out Red Bluff and Bitter Lakes. My discharge from Roswell is 984. Had him put a unit on there to help distribute the pack more evenly on the Panhandle lateral, in turn lowering the pressure at Bitter Lakes and Red Bluff. Monden |
| 1425 | Mullinville | Dee called in from Mullinville. He asked me why his Texas “A” line was going down. I informed him it was because of the Beaver drip outage that was going on down there, for him to go ahead and drop one of the Texas “A” to “D” units. Spain |
| 1430 | P3 | Denny got a small unit on, #4 at 1415. Monden |
| 1521 | Pampa | Jerry Dyson called in from Pampa and said that they 30” KB was going to have to be exposed for several days. Oneok crosses under the KB line and they have a leak on their line. He is going to get some further information on the block valves and will let us know the block valve numbers. He said it would be between the airport block valve and Bobbit. I conferenced in Jerry Graves on the call. Jerry said he would get the information out to everybody also. I am going to notify Beaver’s operator and let them know about it. Muntz |
| 1541 | PG&E Topock | Andy in their gas control had called. He said they have a pipeline event, that they need some serious help in cutting back on the flow that we are giving to them because of compressor problems. He requested that we come in short on the present day by about 25 million. I see no problem at this time due to the fact that WFS is shorting us approximately 30 million at present. I did also forward this information on to Jerry Graves so he would be aware of the issue. Monden |
| 1611 | WFS | Spoke with Francis in their gas control, that they were coming in short by roughly 20 million off the nom. Presently needed to be flowing 30 million extra. He said that the noms will probably be reduced. Monden |
| 1620 | Jal | I have an ESD bypass alarm at Jal. I contacted OCC and had them contact someone from the Jal team. Rick called in and said that the station actually did ESD. The station was down, no demand. He said they were going to leave it blown down this evening and would be out in the morning, would we load the pipe in the morning. He said if we ended up needing to run the station later this evening to contact OCC and they would have some guys go out and load the station this evening. Muntz |
| 1625 | Seminole | I received a call from Bobby King Jr., Denver City, phone 915-758-9481. He said that he has an irrigation pipe that’s leaking in the southwest quarter of section 10, block A-8. He said that they needed to dig out the line and he thought it ran underneath our pipe. Contacted OCC and they had Vincent Diaz of Seminole give me a call. He’s going to check with Bobby King and get back with me as to what they decide to do with it this evening. Muntz |
| 1620 | Forest Lake #1 | Dave with NSP called in and said the inlet pressure was dropping down there pretty fast. It was holding steady at 60 and then dropped down to 49 and then 45. I called Ted and Greg with the North Branch team. Greg is going out to take a look at it and find out what the problem is. Hardy |
| 1636 | Highlands Crawford | Bennie Neatherlin with WT1, Carlsbad team had called in about this interconnect and was expressing some control issues. When I spoke with him I asked him about the water because we were showing around 22# of water and they were flowing about 51 million at present. He said he pulled a tube and on the tube they were showing about 18#. I got a conference call with Bennie, Jerry, Darrell and Johnny Lamb with Duke Highlands. We expressed our concerns of the high water to Johnny Lamb. He said they would make their rounds to all of their dehys that feed into this interconnect and get back with Bennie on it. Bennie will get back with gas control. Monden |
| 1720 | Atoka 3 | Unit 762 is back on line at 1630. Mondo called it in. Monden |
| 1723 | GPM Zia | On the measurement the gas temperature is running 142. Spoke with Bennie on the Carlsbad team about this. He said that he doesn’t see it as an immediate issue but he will address it in the morning when he goes to work. Monden |
| 1741 | Highlands Crawford | Johnny Lamb called Bennie back. Told him that at his points that interconnect in the pipe that he had 10# at one dehy and 6# at Black River and he felt like the moisture would go ahead and drop down and get more compliant within tariff. Bennie will check it first thing in the morning. Then we will address the water issue again. Monden |

1830 to 0630 Shift Tues., March 12, 2002  ()

| 1815 |  | PG&E called and said they needed us to cut a little bit more with the 25 million that was originally agreed upon to help them out with their situation. I told them that I could probably do about 15 for right now and will look over my system and see whether or not that would affect me any on my pressures. Right now we have a total of about 40 million short for PG&E Topock. He said the problem may last for a couple of hours or so, the additional 15 million cut. I will check back with him to see the developments on his system and see where we go from there. Braswell |
| --- | --- | --- |
| 1739 | NSP Viking Chisago | They matched their nom for the day and shut in. They ended up at 6 million. Calhoun |
| 1815 | Palmyra | Shut down unit #22. That was on the “CD” to “D” discharge. We had high differential, spread across the units. Also we were building on the “D” line and “BC” so we really didn’t need it going north at this time. Calhoun |
| 1840 | Harrisburg | Called the on call person. Told him that we were pinching out Aberdeen. Asked him if they could set the yard back up by shutting the valve and using the regulator to feed gas on out towards Brookings and let the Aberdeen gas to come in. He was going to go out there and get the yard set up for that. Calhoun |
| 1910 | Farmington | We lost the inlet pressure and the delivery pressure out at SP1Q. I got a hold of Tim Pietsch. He went out to the station and pulled it up on the computer and he was seeing the same thing. It looks like we haven’t had any updates since about 1650. He was able to get a hold of NSP and talked to Gene the gas controller. They also watched that point. He said he had all of his data. Tim said he would wait until morning before he goes out there to see if he can fix the RTU there. He figures he had blown a fuse or something. But if we need any of that information all we have to do is talk with Gene at NSP. Calhoun |
| 2110 | LS Power Whitewater | Got a call from the operator. He said they were on the way to shutting down for the evening. They were going to start about 4:40 in the morning. Calhoun |
| 2100 | Continental | On the Continental “C” line we have a scheduled rate of 46 million for current day. Currently receiving 21. I spoke with the plant operator with Continental. They said they are showing a 37 scheduled. We are doing fine on the “A” and “B” lines as far as scheduled rate. He said he didn’t have enough gas to actually give us the 46 on the “C” line however he said he would raise it up since they were only giving us 21 million. It looks like we’re still going to be about 15 million short. Notified scheduling and Gary Spraggins. Barry |
| 2150 | Phillips Plymouth | Have a schedule of 11 million. We’ve been averaging 19 for the current day. I tried to call their plant, got no answer, to request them to cut back to rate. Barry |
| 2235 | Farmington | I took unit #6 down. It was on the “BC” discharge. Load had died off and we were starting to pack up. Didn’t need it. Calhoun |
| 2245 | North Branch | Shut down unit #4 that was pumping out west. Temperatures were still warm up in the market area and had plenty of pressure at Benson and I was still packing up going out west so I took the unit off line. Calhoun |
| 2328 | Farmington | Shut down unit #3. The west discharge going out to Willmar had packed up and we were still building on the “BC” going north. Didn’t need the unit on. Calhoun |
| 0005 | Monument | Received a station alarm. Called the OCC for the on call. They got a hold of Bennie who called in. I told him that I had received the alarm out there. He said some people had been working on some things out there today and that they will check it out in the morning. Braswell |
| 0023 | NSP Viking | They brought their flow rate back up at the Chisago point. They went from zero to 15. What they are trying to do is get to their total nom there and they are going to batch it and shut it back in. Calhoun |
| 0045 | Minnegasco | They brought back 40 million to inject at Glenwood. We were already doing 19 million there so I had to kick it up to 140 million to come in on rate for the day. Calhoun |
| 0223 | NSP Viking | They shut their flow rate in at Chisago. They had batched a 7.2 rate. Calhoun |
| 0415 | LS Power Whitewater | They brought a unit on line. Calhoun |
| 0445 | Ventura | Got a call from Jerry with NBPL. He said that CC was ready to start testing there. We brought the flow rate up there by 30 million so that we could come in for the new rate on the day. Calhoun |
| 0505 | Belleville | Shut down unit #2 which was in series and put on unit #1 in parallel. That way we’d have two units running in parallel and could flow the higher flow rate. Calhoun |
| 0535 | Madison | They called up and wanted to go from 58 million down to 43. Calhoun |

slb