Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Mon., March 18, 2002  (Hall, Armitage, Barry, Monden)

| 0614 | Beaver Co. 5 | Unit #4 down. Notified OCC. They got a hold of Stanley Luck. He said they’ll put the unit on when they get out to work this morning because it has been going down on low suction for the lack of gas. Monden |
| --- | --- | --- |
| 0846 |  | Ken with Griffith called in. He said they are having a major upset at the Griffith plant. He said the plant will be down until further notice. He will give us a call once they get back to operational status. Barry |
| 0710 | Tescott | Unit 5 has gone down due to high discharge temperatures. Armitage |
| 0934 | Beaver Co. 5 | Unit #4 is back on line. Monden |
| 1052 | Linam Ranch | Spoke with Harvey the operator. Their flow has gone to zero. He said they are trying to bring their plant back on line. Monden |
| 1400 | Lyons | One of the men from Lyons called in and said they had a water leak. They were going to have to shut about 75% of the field in and dig it out and fix the water leak. They should have it done this afternoon because they needed to get it fixed before the water came in over the gas. Armitage |

1830 to 0630 Shift Mon., March 18, 2002  (Munson, Welsh, Cummings, Calhoun, Davis)

| 2020 | NGPL Eddy Co. | NGPL called to say that they have a problem with their compressor at the Eddy Co. site. The compressor had some mechanical problems and is unavailable for the rest of the night. They would like to move the gas to the Winkler location. There is a nom of approximately 30 at Eddy and they’ve accumed about 10 so far so this remainder of 20 will be moved to Winkler. NGPL will adjust the flow rate to come in for the day for both of these locations combined. Munson |
| --- | --- | --- |
| 1750 | Farmington | Shut down unit #6. We were packed up north and I needed to get out from underneath unit #3 at North Branch. Calhoun |
| 1805 | North Branch | Shut down unit #3 because we were stacked up going out west towards Alexandria and Benson. Calhoun |
| 1811 | Oakland | Unit #19 went down. Jeff went out and put unit #18 on in its place. Calhoun |
| 1848 | Ogden | Shut down unit #14, it was on the east discharge, cracking north. That was primarily due to we were stacked up going out east. Calhoun |
| 1856 | Ogden | They lost unit #8 on Redfield. I called the on call person. They were out at the station working on unit #18 so it shouldn’t take them too long to get the unit back on. Calhoun |
| 1905 | Ogden | They put unit #11 on Redfield. Calhoun |
| 1955 | Owatonna | The unit went down on recycle. It was recycling over 75% and probably went down on gas temperature. I called Jim Von Ruden. He was able to reset it from the house. Calhoun |
| 2115 | Ogden | Unit #11 went down on Redfield service. They went ahead and put unit #14 on Redfield. Calhoun |
| 2135 | NBPL | They called and said CC was done testing. Calhoun |
| 2145 | Guthrie Center | Shut the unit down. We had plenty of suction up at Ogden and starting to get stacked up there. Calhoun |
| 2150 | Ogden | Shut down unit #15 that was on the east discharge. Still trying to back off out east. Calhoun |
| 2200 | LS Power Whitewater | They were going to shut down for the night. Got a 0415 start in the morning. Calhoun |
| 0030 | Bushton | Went ahead and shut it in. We’re building pack too fast and the north is backing down so we took it to zero. At 0100 we went ahead and shut unit 31 down at Clifton on the high side, “E” line, no demand. Welsh |
| 2345 |  | The Beaver plant lost power. The whole lean stream is down. Cummings |
| 0200 | Bushton Products | We noticed the volume is starting to fall off on the lean side, the volume that we were getting from Beaver before their power went out. We notified Bushton Products and they are going to slow down. We did talk to our Bushton facility also and they are going orchestrate that between the two of them to see if they are going to slow down, hold back or what, and try to keep it going as long as they can. What I told them was that down at the Beaver station they are waiting to get commercial power back on for the “D” line units out of their system. They don’t want to use up all of their auxiliary due to running the big high side unit there and they’ll lose their air pressure eventually if they are not careful so they elected to just shut those units off on the lean stream on the “D” coming out of Beaver. Welsh |
| 0455 | NBPL Ventura | CC was there getting ready to test. They wanted to go ahead and get us to take half of the change for the upcoming day. We’re going from 754 total to 906 so that’s a difference of 152 million. We’re going to take 75 of that for right now before he gets started testing. Calhoun |

slb