Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Mon., Oct. 15, 2001  (Alvarado, Cummings, Welsh, Calhoun)

| 0656 | Jal | Unit #2 has gone down. I’ve contacted the OCC. Waiting to hear from the on call at this time. Alvarado |
| --- | --- | --- |
| 0731 | Jal | Unit #2 is back on. Russell was at the location and put the unit back on. Alvarado |
| 0610 | Galena | Dave was finally able to get unit #2 on line. He said that #1 was available now also. Calhoun |
| 0745 | Redfield | Cut their order back. They were doing 230 injection. Cut it back to 200. Also told him to run on the low side of that. That’s primarily due to the loads picking up and having cooler temperatures up north. Calhoun |
| 0810 | Guthrie Center | Started the turbine. I had moved most all of the gas off the south at the interconnect and Ogden was running out of suction. I put Guthrie on to get them some suction up there from the south. Calhoun |
| 0830 | Beatrice | I had Ron put another unit on the “D” line. He was going to add 200 HP. We only had 100# differential or spread across the “D” line. We needed more flow north anyway. Calhoun |
| 0900 | Madison | They wanted to go from 23 to 35. Calhoun |
| 0915 | Waterloo | Did a station start. That was 8000 HP. That’s primarily due to we were falling off at Galena and Earlville. Calhoun |
| 0925 | Madison | They wanted to go from 35 to 43 million. Calhoun |
| 1005 |  | Got a call from Greg Lindblom from North Branch. He was out at the MN1D interconnect. He said Minnegasco wanted him to raise the inlet pressure from 175 to 250. That way it would be set up for wintertime operations. Calhoun |
| 0949 | Hugo | Sent a start command to the unit to start it up. We were needing more gas up north to North Branch. They were running out of suction and the discharge was falling off on the unit and also on up north at Carlton. Calhoun |
| 1020 | South Sioux | Got a low alarm of 340 on the “A” line. I called and talked to Darrel Belt. He was going to raise the regulator at the South Sioux yard from 340 up to 375. Also Duane was going to go up to the Fremont #1 NE area. There is another regulator there that they’ve got a 380 in. They are going to raise it up to 400. Calhoun |
| 1040 | Palmyra | Talked to Mike Butts. He was going to split the “B” out from the “CD” suctions. He was going to put a unit directly on “B” to “BC”. That way we can get the block valve open down at Beatrice and start flowing on the “B”. Calhoun |
| 1100 | Faribault | Put a start command in on the unit. Calhoun |
| 1100 |  | We just got notification that CIG Meadowlark, CIG plans to run a smart pig tomorrow on 25 miles of pipe. It will cut CIG’s flow to us down to roughly 8-10 million for 4-6 hour period. Normally our order is 37.8 million there. I did call Rich Vargo with Bushton Products and tell him exactly what is to happen. It will happen at 0730 in the morning and that the flows from CIG will be cut back 8-10 million for 4 hours Tuesday, 10/16. Welsh |
| 1045 | LaCrescent | Got a call from Dave Cammock. He said the was finished with the outage at Tomah and he was raising the regulator pressure back up to 700#. I raised my alarming values. Calhoun |
| 1100 | Faribault | Started the unit. Calhoun |
| 1115 | Alexandria | Put the unit on because our pressures were dropping off out at Benson. Calhoun |
| 1125 | North Branch | Put unit #2 on. We finally started getting some suction and needed some gas up north as well as west. Calhoun |
| 1128 | Portage 1B | Got a call from Don Soppe. He said that he was going to have to blow the run down. He had to check his meter out there. He will call us back when he put it back in service. I inhibited the point. Calhoun |
| 1136 | Pierz | The unit went down. Our pressures dropped below NSP’s so the unit is trying to free flow right now. Calhoun |
| 1200 | Oakland | Talked to Joyce and asked her to put the “D” station on line. We asked for three verticals. Calhoun |
| 1200 | Palmyra | Lost the high side turbine. It was pumping “CD” to “D”. Calhoun |
| 1215 | Galena | Put unit #1 on with unit #2. Our discharge pressure was down to 908 and we were still dropping off on the east. Put Waterloo on earlier in the morning. We starting to see the suction show up at Galena so we just wanted to keep the differential up and also get the discharge pressure up going east. Calhoun |
| 1350 | Ventura | Got a call from Wayne Kozisek. He said he had problems with the program at the Ventura station. He had to lower the setpoint pressure going to the “A” suction and put it in manual so it would catch it. He was going to turn it back over to me. He wanted me to raise the “A” suction from 430 back up to 465. Calhoun |
| 1400 | Pierz | Got an alarm on the unit when it went down. I called Mitch. He cleared the alarm and got the unit back on line. Actually the unit didn’t come on but it started free flowing at the interconnect. Calhoun |
| 1450 | Galena | Sent a stop command to unit #1. Ended up knocking both units down. Had both of them down at 1510. As soon as the unit #1 coasted down and reset I put a start back in it and it came right back on line. Calhoun |
| 1500 | Palmyra | Had them put the “A” line turbine on to the “B” suction. Mike went out and put unit #27 on. That was to help get off MAOP on the “A” line, plus it would help boost him to move a little more gas going north. We needed it on the “B” suction. Calhoun |
| 1500 |  | Got a call from Stacey at Ogden. Said unit #14 will be down, not available until Thursday They had an air aftercooler that was leaking into the air manifolds. They had to take it apart to fix it. Calhoun |
| 1535 | Palmyra | Had Mike add another unit on the “C” suction to the “D” discharge. Calhoun |
| 1630 | Redfield | Called and talked to Brian. He was running low on his suction pressure going out on the Redfield “B” that was actually free flowing into the St. Peter formation. Told him that we were running a little short on suction at Ogden and didn’t look like we’d be able to get him anymore discharge for awhile so we’re just going to cut the total injection rate at Redfield back some more. Decided to take them from 200 down to about 160. That should help. Calhoun |
| 1645 | Oakland | Got a hold of the on call person Joyce. She was going to go out and put the third vertical on. Calhoun |
| 1700 | CC | Got a call that they were done testing. Calhoun |
| 1735 | Oakland | Lost unit #18. Got a call from Jeff who was at the station. He was going to put it back on line. Calhoun |
| 1740 | Ventura Interconnect | Switched service codes from 1 to 2. That’s for higher north flows. Calhoun |

1830 to 0630 Shift Mon., Oct. 15, 2001  (Braswell, Hall, Armitage, Ellington)

| 1823 | Oakland | Just got a call from Jeff. He said the unit that went down out there went down on crankcase explosion. It will be down for awhile. He said he has sent a report into Jerry on it. Hall |
| --- | --- | --- |
| 1850 | Palmyra | Got a hold of the OCC who got a hold of Mike. Going to probably put one on “B” to “BC” with the other one that’s on as well as adding one “CD” to “D” with the turbine and two other verticals. Also got a hold of the OCC for Beatrice. We’ll either add one on the “D” line or put one on “C” to “C” there. Right now I don’t have room to put it on “D” to “D” but we’ll see when he gets out there and we get some of the gas moving at Palmyra. Hall |
| 2023 | Stevens Co. 6 | Unit 5 is down. Ron called in. He received an auto page. He’s on his way out. Ellington |
| 2139 | Hubbard | Called OCC and they got the on call. When I was talking to him it went ahead and did come up on the HP. It is on line. It had come up to 78 and then it went to zero and then I didn’t have anything. The RPM’s weren’t even up at all. The RPM’s are now 5625 so the unit is on line again. Don’t know why it went down. I had called Corn Belt and let them know I was going to restart it. They said there was no storms or anything. Hall |
| 2150 | Stevens 6 | Unit 5 is back on line. Ellington |
| 2335 | Calpine | Received a call from Chuck. He informed me that their unit that they’ve been having problems with all day came on for a little bit tonight. Their nom is 71.8 million and right now they are spotting about 42.9. They were up on their nom at one point tonight, about 71-72 million but now he was just calling to let us know that they were still having problems with their unit and they were down, probably will be down for 5-7 hours at the most. Braswell |
| 0242 | WT1 | Received a unit 3 alarm. Called the OCC for the on call. Braswell |
| 0433 | WT2 | Unit 3 down. Braswell |
| 0505 | Clark Co. 1 | Unit 2 – We lost that unit. Stacey is going out. Moore |

slb