Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Tues., Feb. 5, 2002  (Munson, Welsh, Calhoun, Cummings)

| 0710 | Red Cedar LaPlata | I received a call from Kim with Red Cedar to say that the LaPlata point may be coming in short today between 2-4 million short. They have a compressor that’s going to be down in the field for 4-6 hours. I passed the information on to Darrell. Munson |
| --- | --- | --- |
| 0830 | LaMaquina | WFS LaMaquina is coming back up to rate. They’ve got their problems taken care of at the plant. CO2 level is back down within spec. I touched base with Terry at their gas control. He asked that we set the setpoint on nom. I had shut them in earlier due to the high CO2. I talked with Blackie. He is on location. He confirmed the CO2 is down. I put them on their nom for the day. It shows to be on the low side with the differential. Blackie said it would be all right. Munson |
| 0900 | Valero Ward | We’re going to run Valero Ward about 14 million short today. This is because we overflowed to them yesterday. I OK’d this with John at Valero and Darrell here in gas control. Munson |
| 0940 | Lone Star Pecos | Due to the drop in the nom at this location. I’ve requested a plate change be made. Robert Baker said he would take care of it. I also touched base with Steve in Lone Star gas control and he thinks that’s a good idea also. While I had Steve on the phone I confirmed that we will be batching the Lone Star Ward point a little bit later this evening. Munson |
| 0945 | Citizens Griffith | I confirmed with Hugh at El Paso that they have a nom there of 5 million. We are going to open it up for 50 until we accum the 5 just like we did yesterday. Munson |
| 1003 | NWPL LaPlata | The LaPlata flow just went to zero at NWPL. I talked to Vi at NWPL gas control. She said they are having some problems. They hope to have it fixed very quickly. The flow should recover. I let Terry know at our LaPlata station. Munson |
| 1005 | Canadian River | We’ve got some HP down at the Canadian River station. I informed Chris on that team. He said he or one of his teammates would take care of it. Munson |
| 1030 | Canadian River | Mike Flynn has units 1 and 3 back on line. Munson |
| 1030 |  | The NWPL flow at Ignacio LaPlata has recovered. Munson |
| 1125 | WFS Milagro | The flow rate at WFS Milagro has dropped off. It’s running about 25 million short right now. I called Terry at WFS. He said that he would call the plant and take care of it. Munson |
| 1310 | PNM Wingate | I asked Mike Boatman to check into the control functions on the PNM Wingate station. The night shift had a problem with the station opening up past the control setpoint. PNM complained about getting too much gas. Our night crew immediately shut the station in. I requested that Mike check into it to see if he can find out what might have happened last night that caused it to open up. Munson |
| 1340 | WFS | I called and talked to Mike at WFS. Went over numbers with them. I told him I show he is a little behind for the day. He agreed. He said he will try to pick the pace up at LaMaquina and Milagro. Right now he said he has some supply that is not coming in up there due to some HP being down. As soon as it gets back he’ll be able to increase the flow rate to us. Munson |
| 0745 | Owatonna | Put the unit on because our pressures out west of Farmington going out towards Willmar were getting low. Also we were low going north to Farmington. Calhoun |
| 0745 | Beatrice | They shut unit #19 down on the “C” line. That allowed the gas to free flow up the “B” line. They were able to save fuel by taking the unit down. Calhoun |
| 0907 | Medford | They took their withdraw rate from 50 to 10. Calhoun |
| 0910 | Viking Chisago | We had a nom of 3.9 to them. They were going to bring on Chisago at about a 14 million rate and batch it. Calhoun |
| 1025 | Farmington | Shut #6 down. It was on the “A” to “BC” pulling the Medford gas in. Calhoun |
| 1125 | NBPL Ventura | Reduced the rate from 725 to 700 to come in for the rate for the new gas day. Calhoun |
| 1315 | Owatonna | Shut the turbine down. The loads had backed off and our pressures were back in good shape. Calhoun |
| 1355 | Farmington | Shut unit #5 down. We were in good shape going north of Farmington. Calhoun |
| 1440 | Alexandria | Shut the unit down. Calhoun |
| 1515 | Ventura | Took unit #14 down. Calhoun |
| 1520 | Ogden | Shut #13 down going north. Calhoun |
| 1530 | NWPL | I noticed that the NWPL flow at LaPlata dropped off again significantly. I called Vi and she said that they had people working on their measurement equipment out there and evidently something that they were doing caused the flow to become erratic. She apologized and said that the flow should be recovering very shortly. I notified Larry at the LaPlata compressor. Munson |
| 1540 | Oakland | Had them drop one unit on the “BC” and also one of the verticals on the “D”. We were starting to pack up at Ogden. Calhoun |
| 1600 | Hubbard | Shut the unit down. Called Corn Belt before I did. Calhoun |
| 1605 | Carlsbad | We have a low voltage alarm coming in from the Santa Fe Bilbrey location. I notified Randy on the Carlsbad team. Munson |
| 1630 | WT1 | While making a two for three unit swap at WT1 they lost units 2 and 3. They are both down at this time. The only unit on right now is unit 1 but it’s throttled way back into idle position. There is personnel on site trying to get everything back on. Munson |
| 1710 | Crawford | Unit #1 has gone down due to the problems at WT1. I notified Randy at WT1. He said he would get someone on it. Munson |
| 1615 | Minnegasco | They did a backhaul. They wanted to reduce their withdrawal rate at Glenwood from 14 and took it to zero. Also Trailblazer, they took a 21.742 to zero there. We contacted NGPL and we made those things happen. Calhoun |
| 1630 | Redfield | Talked to Steve Thomas. They were wanting to get Redfield withdrawal rate cut back to 300. I called the Redfield station and talked to Gary. He was going to make that happen. Calhoun |
| 1730 | Ogden | Shut unit #17 down. Calhoun |

1830 to 0630 Shift Tues., Feb. 5, 2002  (Hardy, Armitage, Spain, Muntz)

| 1816 | Monahans Team | I notified Rick Youngs that we have a low battery voltage showing up at the Sid Richardson block 16 fuel meter. Munson |
| --- | --- | --- |
| 1925 | Platteville | I had a low battery voltage come in for Union Center. I called and spoke to Lenny. He was working on software out there today. We just needed to change some new register numbers around for the battery voltage. Ken Poole fixed it for us so everything is all right now. Spain |
| 2000 | Ogden | I took unit #13 off of the Redfield suction. We had three units on there. Looks like we only require two units to pull the 300 in from Redfield. The make up valve 90 was open quite a bit, about 45%. I took 13 down and we really haven’t backed into Redfield but about 10#. I called Redfield before I did that, spoke to Gary. It was all right with him. Spain |
| 2100 | Oakland | I just called and gave Jeff and order to bypass the “BC” station at Oakland. We’re cut back to a minimal amount of units there. We still have 780 on the discharge side. Going to go ahead and let it roll back into Palmyra. Spain |
| 2145 | Farmington | Unit #2 went down. It was on the Willmar service. I don’t need it back on right now but I do need it available towards morning. I called the on call person, spoke to Steve Smith. He is supposed to try to reset it with his laptop and give me a call back. Spain |
| 2330 | Palmyra | I called Cramer out on unit #26. It had an alarm on it for about ½ hour, wouldn’t clear. He went out. Informed me it was real foggy out there. The air inlet filter to the turbine was probably frosting up a little. He was just going to reset it so it wouldn’t go off again. It will show in alarm here the rest of the night. I’ll just have to keep an eye on it. Spain |
| 0035 | Atoka 2 | Unit 837 is down. Armando is going to go check on it and restart it. Muntz |
| 0100 | Farmington | I put unit #2 back on Willmar service. Spain |
| 0113 | Station 7 | Brett is bringing station 7 on line. Need to pull gas away from station 9. Currently have 1004 discharge down at 9. Muntz |
| 0110 | Ogden | Put unit #14 on going from the “BC” suction to the east discharge. Spain |
| 0200 | Owatonna | I started the Owatonna unit. It looks like our zone E pack stayed about 1020 for the last couple of hours, since midnight. Trying to get it to grow up to around 1050 for zone EF. Spain |
| 0253 | WFS Milagro | WFS control called and requested we cut the setpoint at Milagro back from 310 to a 300. They had lost a big compressor. He said that it would be about 45 minutes to an hour before they got the unit back on. When they get it back on they’ll give me a call and have me increase the rate again. Muntz |
| 0300 | Hubbard | I started the unit at Hubbard. I’ve got about 525 for pressure on the suction side of Earlville. Spain |
| 0340 | Sublette | Unit 14 is down. Steve Showers is on his way out. Hardy |
| 0338 | Station 7 | Unit 703 is on the line. Muntz |
| 0455 | Ogden | Unit #16 died. I couldn’t get it reset. It was on the Redfield service. I put unit #13 on in its place. Spain |
| 0500 | Beaver | Have requested Beaver go to 725 on the “KB” line to pressure it up enough so we can bring gas south. Armitage |

slb