Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Mon., Feb. 4, 2002  (Munson, Welsh, Calhoun, Cummings)

| 0615 | Owatonna | Got a hold of Rod. Let him know that Rochester 1D pressure was down below the minimum alarm. We’re trying to get the pressure up on the high side going up the “D” line to see if we can get the pressure turned around. Calhoun |
| --- | --- | --- |
| 0650 | Garner | Gave them an order to go up to 200 million withdrawal rate. We were needing the gas north in the Cities. Calhoun |
| 0645 | Ventura | Had Jim put on one of the turbines. At 0700 we asked him if he could put on the second turbine. Calhoun |
| 0700 | Ogden | Got a hold of Darrel the on call person. Told him we needed some HP north. They still had units 13 and 17 down. He said he was going to try to get something going north. Calhoun |
| 0730 | Beatrice | Asked them to put the turbine on the high side. Calhoun |
| 0745 | Ogden | They are still having problems getting some HP on line so we called them back to find out what the problems were. Told them we were really hurting north and we wanted that HP on. Calhoun |
| 0650 | Heartland Corn | Called them to let them know that our pressures were running low and that they would need to be starting to back off because we weren’t going to be able to hold their pressure this morning. I called them again at 0730 and told them the pressure was getting down to the minimum out there and they needed to cut it back. Calhoun |
| 0730 | Oakland | Had them put on another horizontal on the “BC”. At 0800 had them put on another unit. Calhoun |
| 0810 | Holcomb | Arnie called in and said that she lost unit #11 on the Holcomb field. Said #11 had a knock in it. They weren’t able to start unit #12 so they are trying to get it on line. Also with that unit being down she lost unit #1. Calhoun |
| 0815 | Gallup | We are into a peak alert stage at the Gallup compressor. I slowed the unit down as far as it could go. The RPM’s are now on the bottom at 1142. I also notified Mike Boatman. He is on location to make sure everything is functioning properly on that low speed. Munson |
| 0820 | PNM Belen | Keith checked in from the Mountainair team to say that he was on location at the PNM Belen TBS. He’s going to be repairing a damaged diaphragm at that location. Keith said that he would be able to route the gas through a bypass and that PNM’s flow should be uninterrupted. Keith told me that he would be contacting his counterparts in the field at the location. I notified Norm at the PNM gas control site. Munson |
| 0830 | LaCrescent | Brought the unit on line. Calhoun |
| 0830 | Ogden | They called in and said they had unit #17 on line. They were going to put it on east, cracking north. Also they got one of the horizontals on going north. Calhoun |
| 0830 | River Falls WI | Got a low/low inlet pressure. Also at Baldwin we had a low/low inlet pressure. I got a hold of Bob Tornio. He was going to make a trip down there and look at those stations and also they were going to dial up a few more of the TBS’s and look at them also. Our pressures are low coming out of Farmington and also Hugo. We’re down to about 600# on discharge. We are trying to get the gas coming up from the south. We’ve got Garner coming on with 200 million of withdrawal. They are up to 100 million now and they are still working at getting up to 200. We have had Ventura put on a couple of turbines and Ogden has put on the HP to get pressures up so that we can get the gas up north. We did increase the Emmons flow rate by over 350 million so far. Calhoun |
| 0915 | Willmar | Got a hold of Tim. Let him know on the pressures, I got a low/low alarm at the Willmar TBS. He said it would be all right at the 608# that it’s at. They were going to look at their TBS’s out in their region to see if they had any other problems. I just wanted to notify him and let him know that we had low pressures there right now. Calhoun |
| 0845 |  | Got a hold of Jeff Green from North Branch to let him know we had low pressure at Hudson WI TBS. He was going to run out there and take a look at it. He was going to dial up a few more of his TBS’s just to make sure they are doing all right. Calhoun |
| 0945 | Garner | They got their withdrawal rate from 100 to 190. Calhoun |
| 0945 | Ventura | They got the second turbine on the “C” line so they have three verticals and two turbines running on the “C”. Calhoun |
| 1000 | Farmington | They got unit #6, panel cleared out and we were able to get it over on the “A” line service. We put another start in on it. Calhoun |
| 1000 | Madison | They called in and wanted their flow rate raised from 45 to a 48. Calhoun |
| 1020 | Elk River | Talked to Greg Lindbloom. He had raised the regulator out there on the Princeton tie-over from 510 to 540. That was to help our low pressures at Anoka #1 and also out on the Carver system. That was due to us having low pressures and heavy loads this morning. They were going to leave it at 540 and see how that worked through some of the morning pulls. Calhoun |
| 1020 | North Branch | Put unit #2 on pumping west. Calhoun |
| 1100 | Redfield | Talked to Walling and asked him if they could go to their max withdrawal rate. He said they were there, pretty close to being there, right now at the 325. Originally we did have 350 that we were showing on the orders. He said the field rate has dropped off and possibly they have some wells that might have watered off. They are going to go out and check them but the feeling is pretty much that the max withdrawal is 325. Calhoun |
| 1205 | Bloomfield | Unit #4 has gone down. Jeff is on site with the unit. He’ll try to get it back on as soon as possible. I notified Terry at LaPlata and he’s going to put his units into recycle and try to keep them from going down. Munson |
| 1445 | Station 3 | Station 3 just lost all of their HP. They had two units running. They made a swap with units 1 and 2 earlier. It looked like everything went smoothly and now they just lost #2 and #3 which were the ones they had on. They took 1 down for some gasket repairs and put on 2. 2 and 3 went down and as I’m making this recording #3 has just come back up. It looks like they are trying to get things lined out over there. Munson |
| 1450 | Maljamar | Unit #1 is down. Ernesto is on his way over there. Munson |
| 1115 | Belleville | Shut down units 1 and 2. There wasn’t any load out there. East of Belleville had backed off considerably. LS Power shut down early. Calhoun |
| 1140 | Beatrice | Put unit #20 on “D” to “D”. Calhoun |
| 1220 |  | Got a call from Dan Wilson. He said there would be a peak at Hubbard today so we’ll have to shut the unit down around 4:00. Calhoun |
| 1325 | Palmyra | Unit #24 went down. Got a hold of Dale. He was going to put it back on. Calhoun |
| 1350 | LaCrescent | Shut the unit down. We were turned around real good at Tomah and didn’t need the extra gas there. We needed it up in the Cities. Calhoun |
| 1435 | Hugo | Shut the unit down. That’s primarily due to we were recycling quite a little bit of gas and things were turning around real nice so I went ahead and shut the unit down. Calhoun |
| 1515 | Garner | Cut them from 200 to a 100. Calhoun |
| 1520 | Station 3 | All of the HP is back up at 3. Munson |
| 1530 | Farmington | They got unit #1 fixed and put it back on line on the Willmar discharge. Calhoun |
| 1545 | North Branch | Shut unit #2 down. We were up on discharge MAOP on the west and the units were slowed all of the way down. Calhoun |
| 1550 | Farmington | Shut unit #5 down. We were growing real fast in the market area. Calhoun |
| 1550 | Garner | Gave them an order to go from 100 to zero. Calhoun |
| 1630 | Owatonna | Shut the unit down. We had it slowed down and we were recycling gas. Calhoun |
| 1705 | Ventura | Shut unit #15 down. When I shut Owatonna down the unit was starting to come into surge. Calhoun |

1830 to 0630 Shift Mon., Feb. 4, 2002  (Barry, Armitage, Hall, Braswell)

| 2115 |  | We’ve accumed the flow rate for the Coronado Pecos interconnect. This does require a manual shut in. I called the OCC for the Monahans team on call. I’m going to have someone go out and manually shut the valve. Barry |
| --- | --- | --- |
| 2155 |  | I’ve been in contact with Lone Star gas control. They are showing 27 scheduled. Our schedulers just left. They did confirm the last process tonight at 10:00 that it’s only 20.9. This nom that Lone Star gave us was actually a nom which was set up at 9:00 this morning. It did change during the process of the day. They did send a zero setpoint to the valve but we’re still showing a 7 million flow rate. They did confirm that they are still showing a 7 million rate also. Our concern is that we have low differential on the meter run. Right now due to the heavy pull on the power plants we don’t have the additional gas to give them to raise the flow to where we have good measurement. Their closest person to actually make a valve closure is two hours from there from what I understand. At this time Reagan who is the Lone Star controller is calling their scheduling people to see if they did finalize their nom at 27 or if it’s 20.9 before he goes any further. Barry |
| 2209 |  | Reagan with Lone Star called back in and had scheduler by the name of Gary on the phone also. We did discuss what had happened on the scheduled volumes. At this time Reagan is going to call one of their other people in the field and manually have the block valve closed. Barry |
| 2231 |  | A little bit earlier Chapa with El Paso gas control called in advising me that the Keystone interconnect we were showing high nitrogen. At that time they were shutting us in. We only lacked about 2 million of making our nom. So just going to go ahead and leave the Waha interconnect on just a little bit longer to accum the additional 2 million. Just now Chapa called back. The iso on our scada was showing 5.2% nitrogen at our Waha interconnect. At this time I’m going to increase Estes by 20 million and then give them the same gas back at Halley to decrease the percentage of nitrogen we’re delivering to El Paso at the Waha interconnect. Barry |
| 0255 | WFS LaMaquina | Just lost the flow rate at LaMaquina. Nom was 40.426, it’s gone to zero. WFS gas control called and said that they were trying to get enough help out to get the plant back on and get it back as soon as possible. Muntz |
| 0310 | Meridian | I spoke with Ben at Valverde plant. CO2 is bumping 1.99. He said that he would try to get it back down. He said they were moving about 400 through the plant at this time so he’s running a little bit higher on the CO2 than earlier in the day. He said he would try to get it back lower and keep it below 2.0. Muntz |
| 0310 | Farmington | Due to line pressures and due to 1D flows going up we have put a start in #5 unit at Farmington Armitage |
| 0522 |  | Due to pressures around 1D I have put a start command in Hugo. Armitage |
| 0527 | Oneok Processing | Called and talked to Jack Prather. Let him know that CIG cut back to 20. Lost approximately 30 million up the rich side. They did it about 4:00 this morning. They will be down until about 1:00 this afternoon. Hall |

slb