Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Tues., Jan. 8, 2002  (Munson, Calhoun, Welsh, Cummings)

| 0800 | Cunningham | Gave Gloria an order to decrease the withdrawal rate from 550 to 300. She is going to do it over a 1-2 hour time frame. Calhoun |
| --- | --- | --- |
| 0830 | Clifton | Had Stan take off unit #28 that was running on the high side with the turbine. Calhoun |
| 0910 | Tescott | Had them take unit #5 off line. We’re trying to back off on the Clifton flow rates. Going from about 1430 down to 1125 due to temperatures warming up on the north end. Calhoun |
| 0920 | Tate | Their moisture came up to 25# so I called out and talked to Jeff at the station. They are adding regenerated glycol back to their unit and it was saturated with water. That is why it spiked up. As it runs through the reboiler it should cook that water off and the moisture levels will drop. Calhoun |
| 0925 | MidCon | They called up and were wanting to have us open the valve up at Bushton that goes into the “D” line. They’ve got a couple of units that are scheduled for state tests and they are going to run each one for about 15-20 minutes while they are doing the tests and then they will shut it back in. Calhoun |
| 1220 | WT2 | The crew at WT2 will be making repairs this afternoon on the cracked weld on the piping for unit #1. They are going to require that the whole station be taken down for the welding process. The whole operation from welding to x-ray to putting the station back in service is estimated to take between 3-4 hours. They are going to attempt to get started on the work between 1:00 and 2:00. All of the affected customers have been notified. I spoke with Gary at Valero, Bill at Oneok, William at Lone Star, James at Oasis, Ed with NNG Permian. Darrell Schoolcraft spoke with NGPL personnel. Everyone is aware. They are going to work with us in adapting their systems to handle the loss of gas. Munson |
| 0900 | KP&L Mullinville | They had a nom out there for 1.093 delivery to us. We decided to leave it shut in for the day since it was such a small volume. Whenever we get a large enough imbalance there we will go ahead and flow it. Calhoun |
| 1100 | MidCon | They called and said they were over with their state tests. We had Bushton close in the side valve there. Calhoun |
| 1436 | WT2 | The work on the welding of the pipe at WT2 is taking place at this time. The reason for the delay was the late arrival of the x-ray personnel. Munson |
| 1500 | Macksville | Had Darrel shut down units 2 and 3 on the high side. That’s due to backing off on the north end. Having to cut the Cunningham withdrawal rates and we’re getting down on minimum suction and will also be backing in on top of them from Bushton. Calhoun |
| 1510 | Burdett | They had a station BSD. They had a bad transmitter in the basement that gave them a high gas detection alarm. It shut the units down and blew the building down. They got the units all back up at 1530. Calhoun |
| 1640 | Atoka 3 | Unit 762 has gone down. I notified Jerry through the OCC. I also notified Lee at the Dagger plant to let him know we have somebody on the way out there. Munson |
| 1640 |  | We have a unit alarm on the Monument turbine #1. Even though these units are not on line I still notified Randy on the Carlsbad team and let him know we have an alarm. He’s going to go check things out over there. Munson |

1830 to 0630 Shift Tues., Jan. 8, 2002  (Hardy, Muntz, Spain, Henley, Monden)

| 1920 | Stevens Co. 1 | Tom Bromwell went out earlier. We lost #7 out there at Stevens Co. 1. He is going to leave it down and put #1 on in its place. It went down on low suction. Spain |
| --- | --- | --- |
| 1949 | Ogden | Unit 16 had gone down. Unable to reset. Had done it at Kyle’s request from the Ogden team. Put unit 14 on line and it did go. He’s going to go out and check out 16. Monden |
| 2000 | Norval | Called over and talked to PG&E gas control. Norval’s flow rate has fallen off to zero. He’s going to send a man out to check to see what’s going on. Hardy |
| 2010 | Shell Tippet | That flow rate is down to zero. They’ve got a 45 million nom. Called over and talked to Sid Richardson dispatch. They said the plant there was down. Mike gave me the number to the operator so I could find out more about what was going on. Tried to call over to the plant, no answer. I will try again. Hardy |
| 2018 | Carlton | Had 1 and 2 unit alarms come in. Being unfamiliar with the station and since some stations do have unit alarms hooked up to their other devices, I gave a call to the OCC for the contact person. Spoke with Loren Lilly. He said he would get it checked out, have the technician look at it. Monden |
| 2045 | Shell Tippet | Called and talked to Dennis the operator. He said the problem with the plant being down is that they are trying to dry out the cryo plant. Everything is frozen up there. He said they could possibly be back on line tonight or sometime early tomorrow morning. He said he would call me with any progress and if they got back on. Hardy |
| 2203 | WT2 | Joey just called in and said that they are going to go home for the day. Everything is back to normal at WT2. Muntz |
| 2220 | PG&E | Talked to dispatch up there. They sent somebody out to Norval to see if the delivery problem was on their end. The guy called in and said everything looks fine there. I talked to Les on our side. He is going to check everything out to make sure a valve didn’t close in on us. Hardy |
| 2220 | AEP | Fred called to let us know that Rio Pecos is coming on for 1-2 million and will be flowing all day. Hardy |
| 2312 | Mapco Whiting Interc. | It has low voltage. I called OCC. They got a hold of Darrel Belt on the South Sioux team. We talked about this interconnect. He said the charging but is getting polled too frequency. He said he had brought the issue up Monday to the hot seat person in order to try to get the polling limited. It’s an issue that they need to get more clarified on how often they need to poll this site because it doesn’t pull a lot of volume in gas. They don’t want to try to bring in AC power to the interconnect to keep the battery fully charged. There is not enough hours in the day to keep the battery cell charged from the photocells. Monden |
| 2337 | Westar Oneok | I just spoke with Pat. Their nom is 40.692. Current projection is to come on a 24.3. Pat said that since they have an imbalance of 168,129 due TW that he would just continue to flow at the current setpoint which is 40. This way we’ll come in about 15 short for the current gas day and work it off the imbalance. Muntz |
| 2345 | Norval Pecos | Talked to Mike at PG&E gas control. Told him that I’ve got a guy standing out at the valve going to Norval. He said the valve is wide open and that Norval has just pressured us out. What probably happened was, talking to PG&E, is that when the Oasis delivery went up from 38 to 70 we had to run 102 to get it in and that’s probably what took Norval’s gas away. So far we delivered 20 million to them a 28 million nom. They agreed to let us tack that 8 million on to tomorrow’s nom when the Oasis delivery will go down. Hardy |
| 0000 | PG&E | They just called to say they are going to put HP on to pull that gas in at the Norval delivery. Hardy |
| 0120 | Sublette | Russell called in. He’s got unit 23 back on line. Spain |
| 0142 | Quad Cities | QUAD RTU is in history. Notified OCC for the contact person who was Don on the Platteville team. He will get a hold of Kurt Ritter to give me a call back. Monden |
| 0152 | Quad Cities | Kurt Ritter called back about the scada being out at this point. I had called MidAmerican and spoke with Jerry. They were reading the data off of the interconnect. He said they were showing about 16 million. Spoke with Kurt about this. He said he was having a problem with that modem and first thing this morning when he reports to work he will get out to the site and replace the modem at the interconnect. Monden |
| 0214 | ANR Janesville | It has compression that had gone down. He said they would get it back up as soon as possible. Monden |
| 0330 | Waterloo | Shut down unit #2. The suctions are starting to rise at Galena from Janesville being down. Still don’t have that unit back up. Going to try to keep from pressuring up on my east leg. Monden |
| 0510 | ANR Janesville | It’s back flowing at 410 to try to come in on the nom. Monden |
| 0510 | Warren Eunice | Their flow rate is down to zero. I called and talked to Robbie in the control room. He said they had an ESD there. H2S monitor showed real high. Also it shut the plant down. They are working on getting units back up right now. The H2S is cleared and they’ve got one unit on so far. Robbie said he’d call whenever everything looked OK there. Hardy |
| 0530 | Finney Co. 2 | Got a dehy failure alarm in here. I called OCC. They got a hold of Gary. He’s going to go out for us. Spain |
| 0540 | Clark Co. 2 | Unit 2 is down. Dennis is going to go check it out. Hardy |

slb