Services provided by Northern Natural Gas

And Transwestern Pipeline Company

- Enron Transportation Services

- Gas Control Event Log

0630 to 1830 Shift Sun., Feb. 24, 2002  (Calhoun, Davis, Cummings, Armitage, Munson)

| 1135 | Station 5 | I asked Charlie to put the automatic 6” regulator in service for us so we can control flow through 5 from here in gas control. Munson |
| --- | --- | --- |
| 0910 |  | Put a call out for one of the men to go put the Pampa turbine on due to Plainview having nozzle problems. Armitage |
| 1000 |  | The Pampa turbine is on the line. Armitage |
| 1224 | Hemphill 1 | Unit 1 is down. The men have called in. They are on the way to the station. Armitage |
| 1140 | Red Cedar | I called the plant and talked to Delbert. Asked him to bring his CO2 down with spec. Munson |
| 0600 | Madison | They wanted to go from 40 to 32. Calhoun |
| 0945 | Palmyra | Called the on call person Bob. Asked him to go out and put a unit on “CD” to “D”. That was to help pull some of the pack away from Beatrice to see if we could get Trailblazer coming in. Calhoun |
| 1000 | Ventura | Bumped the rate up from 700 to 725 so that we could get on the new day rate. Calhoun |
| 0900 |  | We cut Glenwood back from 155 to shut in. That was for the new day noms. They didn’t have an order out there and Minnegasco did some backhauls on the previous day. Calhoun |
| 0940 | North Branch | Put unit #4 on pumping west and cracking north. Calhoun |
| 1230 | Alexandria | Put the unit on because our Benson pressure was falling pretty fast. We were down to about 575 so I went ahead and put it on. Calhoun |
| 1340 | Owatonna | Sent a start to the unit. I was still falling off on the suctions at Farmington and was getting into some spread problems on the little unit going west. Calhoun |
| 1355 | Beatrice | Lost unit #24 which was a 4000 pulling on the “D” line and Trailblazer. At 1400 I got unit #23 put on in its place. Also I called the on call person to have him go out and check the unit. Rich Dorn went out there and found that the lubricator system on the compressors had locked up. He was able to get it to recycle and freed up. He started the unit up to make sure things were fine. Calhoun |
| 1435 | Angus Anson | Got a call from the operator who said they were going to bring one unit on line and probably stay on line until about 8:00 this evening. Calhoun |
| 1445 | Palmyra | Had Bob go out and put on one more unit on the “CD” to “D”. Our pack was starting to pull away up at Ogden and also we were still pinching Trailblazer out a little bit. We had enough room to get that unit on at Palmyra. Calhoun |
| 1515 | NGPL | I called them to let them know that we were working to try to get their flow rates in at Trailblazer and was putting on HP. Just wanted to keep them up to date on what was going on. Calhoun |
| 1530 | Station 9 | I requested Jesse through the OCC to go to station 9 and rearrange some of the valving so that we can take the suction off of the West Texas lateral and discharge to the Panhandle only and stop sending gas into the mainline. I wanted to discuss the possibilities with him of arranging the actual suction of gas off the mainline with the West Texas lateral gas to try to relieve some of the pressure on the mainline. Munson |
| 1705 | Station 3 | I asked Scott at station 3 to lower the manual setting on RPM’s for his unit so we could lower that discharge pressure at station 3. Munson |
| 1705 |  | I asked Don in the OCC to locate personnel for station 7. It looks like they have a valve closed and the station is not able to equalize. We’re not able to flow back away from 5 towards 9 because of this. Munson |

1830 to 0630 Shift Sun., Feb. 24, 2002  (Hardy, Henley, Spain, Muntz)

| 1815 | Mullinville | Talked to Dee. We need him to put the turbine on the “E” line to pull gas off of Beaver. They’re about 7# from the station shutting down. They’ve got high pressure out there and they need to have it pulled off of them. Dee is going to do that for us also. Hardy |
| --- | --- | --- |
| 1820 | Bushton | Talked to the operator there to try to get him to pull gas on the “E” line north of us to get pressure off of Beaver also. The best option available is to put a 2000 on and that will keep the “D” discharge from getting close to MAOP. Putting on anything bigger like a 4000 or a 5500 unit would be a bit too much. Hardy |
| 1858 | Station 4 | Unit 401 is down. Henley |
| 1936 | Station 7 | Norm went out and opened up the block for us. Henley |
| 1950 | Beaver | Just spoke with Randy at the Beaver plant. He’s going to have to drop a 2000 that is on the “KB”. He’s bumping MAOP on the discharge side. When we get some gas pulled away at Mullinville he’ll put the unit back on. Muntz |
| 2003 | Gallup | The unit is down. Pat is on his way out. Henley |
| 2003 |  | I called WT1 and spoke with Randy. The Monument Junction now is all 100% open, the control valve. What we need to do is open up the 24 and he’s going to take care of that for me so I can send more gas down to WT2. Suction is falling there, it’s down to 660. Henley |
| 2045 | Guthrie Center | Sent a remote start to Guthrie Center. Need to send a little bit more gas to Ogden. The total line pack is 3453 but it is falling. I believe that weather will move in tonight. We’d like to get over 3500 for the morning pull. Spain |
| 2050 | Farmington | I started unit #1. I’ve only got the turbine on the “BC” service right now. Need a little more gas on Farmington’s discharge. Spain |
| 2116 | WT1 Maljamar Lateral | Just spoke with Randy again regarding the Monument x-over. That is all common now. We are sending more gas down to WT2. I also received an instrument fail alarm at the Maljamar lateral. I told Randy about that. He’s going to pass that on to Bob. They may look at it tonight or they may wait until morning to look at it. Henley |
| 2345 | Ogden | Put unit #15 on going east, cracking north. I need the gas north. Got about 1048 in zone EF. I’d like to build that up a little more before morning. Spain |
| 0130 | Clifton | Brought the Clifton turbine on the “E” line. Hardy |
| 0145 | Cunningham | I got them to take injection from 425 down to about 150 so we can move more gas north. Hardy |
| 0230 | Oakland | I called the on call person Dana and asked him to put the “D” station on out there. Spain |
| 0315 | Stevens Co. 6 | Got a station ESD alarm. All of the units are down. Tom got the page and he’s on his way out. Hardy |
| 0340 | Ogden | I tried to put unit #16 on pumping east and cracking north. It would fire off but it would go right back down. I reset it a couple of times and tried it again. It still wouldn’t go. I went ahead and put 13 on line. It came on line all right. I don’t think I’ll need anymore HP before the crew gets out there this morning so I’ll just pass that problem with 16 on to our day crew. Spain |
| 0430 | Cunningham | I asked Greg to shut in injection, go to zero. Hardy |
| 0500 | Farmington | I tried to start unit #3 about 20 minutes ago. It went into alarm and never did fire off. I sent a start to unit #6. It’s on line now. Spain |
| 0530 | Ventura Interconnect | Leon called from NBPL. We both see about 220 million increase for the new day with our NBPL receipts. He told me that CC wanted to start testing, if we could take 100 now towards that new nom. I went ahead and put it in there to see if we can get it in before the gas day starts. Spain |

slb