# Path-Defined Receipt Point Capacity Structure

# Capacity rights on the SoCalGas system will be determined by paths, which will be defined by interstate entry points and other interconnections along the long-line SoCalGas transmission corridors.  Each existing entry point and/or interconnection will be classified as a primary or a secondary receipt point along each path.  All new or potential entry points along each path will be identified.  Hector Road will be defined as a secondary receipt point on SoCalGas Line 235, and points with similar characteristics will be also be defined as secondary receipt points on other paths.

# System paths may each contain secondary as well as primary points.  All secondary points designated within a system path are available for use by primary receipt point shippers according to the priority system outlined below. Requests for primary new interconnects and/or converting secondary receipt points to primary would include a determination of cost responsibility for SoCalGas system additions.

# Access rights and scheduling priorities under primary and secondary rights would be similar to the operation of FERC interstate pipelines.  Scheduling of SoCalGas transportation service along a system path would require a shipper to have firm primary receipt point rights.  Any secondary receipt points in a defined SoCalGas path would be available for use by primary receipt point capacity holders.  The order of priority for scheduling service would be 1) primary firm receipt point capacity,  2) secondary firm receipt point capacity and 3) as-available receipt point capacity.  A shipper's request for service would be scheduled first at specifically nominated receipt points and then on the SoCalGas system path. To the extent the sum of shippers’ requests for transportation service exceed the capacity of a specific receipt point, then volumes would be reduced in the inverse order of receipt point priority (i.e. as-available off first). If requests for a category of receipt point capacity (either as-available, secondary or primary) can be partially accommodated, those requests would be scheduled on a pro rata basis. This methodology for receipt point capacity will be utilized during the settlement period and re-evaluated when the settlement period ends.