Confidential Information Memorandum

Enron North America

Pastoria

Energy Facility L.L.C.

A 750 MW Combined Cycle Merchant Facility
Under Development near Bakersfield, California

August 2000

Confidential Information Memorandum

Memorandum No.

Enron North America Corp. (the ”Company”) is issuing this confidential information memorandum in connection with the sale (the “Transaction”) of Pastoria Energy Facility L.L.C., the entity through which the Company is developing a 750MW combined cycle generation facility and 250 MW expansion opportunity near Bakersfield, California (the “Project”).

This Confidential Information Memorandum (the “Memorandum”) is being delivered to a select number of parties who have expressed an interest in pursuing a Transaction.  The sole purpose of this Memorandum is to assist the recipient in deciding whether to proceed with an in-depth investigation of the Project in accordance with procedures established by the Company.  This Memorandum does not purport to contain all of the information that may be required to evaluate all of the factors which would be relevant to a recipient considering entering into any Transaction and any recipient hereof should conduct its own investigation and analysis.

The distribution and use by each recipient of the information contained herein and any other information provided to the recipient by the Company is governed by a confidentiality agreement, a copy of which has been executed and delivered by each recipient.  IF YOU HAVE NOT EXECUTED AND DELIVERED SUCH A CONFIDENTIALITY AGREEMENT, YOU HAVE RECEIVED THIS MEMORANDUM IN ERROR.  IF SO, PLEASE NOTIFY US IMMEDIATELY BY TELEPHONE, AND RETURN THE MEMORANDUM TO US AT THE ADDRESS BELOW.

Neither the Company nor any of its affiliates or representatives makes any representation or warranty, express or implied, as to the accuracy or completeness of the information contained herein or any other written or oral communication transmitted or made available to any recipient.  The Company and its affiliates and representatives expressly disclaim any and all liability based, in whole or in part, on such information, errors therein or omissions therefrom.  Under no circumstances shall this Memorandum constitute or be deemed to be an offer to sell or the solicitation of an offer to buy any securities.

In addition, this Memorandum includes certain forward-looking statements provided by the Company with respect to the anticipated future performance of the Project.  Such forward-looking statements reflect various assumptions of management concerning the future performance of the Project, which assumptions may or may not prove to be correct.  The actual results will vary from the anticipated results and such variations may be material.  Important factors that could cause actual results to differ materially from such expectations include development uncertainty, operating uncertainty, acquisition uncertainty, uncertainty related to gas and power markets, weather, uncertainties relating to domestic and international economic and political conditions and uncertainties regarding the impact of regulation, changes in government policy, industry deregulation and competition.  Each Prospective Investor has an obligation to evaluate all such risks for itself.  No representations or warranties are made as to the accuracy or reasonableness of such assumptions or forward looking statements based thereon.  Only those representations and warranties that may be made in a definitive written agreement relating to a Transaction, when and if executed, and subject to any limitations and restrictions as may be specified in such definitive agreement, shall have any legal effect.

Unless otherwise instructed, all communications or inquires relating to the Project should be directed to the following persons:

| David Parquet Vice President Enron North America 101 California, Suite 1950 San Francisco, CA 94111 415.782.7827 | Andrew Kelemen Vice President Enron North America 1400 Smith Houston, TX 77002 713.853.9579 | Mark Fillinger Director Enron North America 101 California, Suite 1950 San Francisco, CA 94111 415.782.7815 |
| --- | --- | --- |
| Ron Coker Manager Enron North America 101 California, Suite 1950 San Francisco, CA 94111 415.782.7827 |  |  |

Table of Contents  [TO BE REVISED]

- Executive Summary

- Overview	1

- The Project	1

- Timeline	4

- Project Attributes	4

- Background to the Transaction	5

- Process Overview	5

- Investment Considerations

- Attractive Location	6

- Growing California Energy Market	6

- Major Tenant	6

- Substantially Certified Site in Difficult Region	6

- Firm Commitment for Turbine Schedule	7

- Significant Barriers to Entry	7

- City of Pittsburg Alliance	7

- Experienced Project Sponsor	7

- Project Overview

- General	8

- Power Plant Technology	8

- Project Site	9

- Project Improvements and Laterals	9

- Project Schedule	11

- Project Budget	11

- Environmental	11

- Litigation	12

- Project Approvals and Agreements

- The CEC Licensing Process	13

- Local Permitting Issues	13

- Project Service Agreements [Update and Add]	15

- Other Project Agreements	17

- Project Operating Assumptions

- [To Come]	20

- Community Support

- Strategic Alliance with PPC	21

- Public Relations	21

- Management Biographies

- Organization Structure	22

- Exhibits

- Exhibit 1:  Industry Overview

- Overview	24

- California Economic Growth	24

- Western Systems Coordination Council	24

- California Market Structure	25

- Market Size	27

- Changing Supply Profile	29

- Exhibit 2: California Generation Facilities

Executive Summary

Overview

Enron North America Corp. (“ENA” or the “Company”), a wholly owned subsidiary of Enron Corp. (“Enron”), desires to divest its ownership in the Pastoria Energy Facility, a nominal 750 MW natural gas-fired combined cycle merchant facility located near Bakersfield, California currently being developed by ENA and sell its contractual right to the 250 MW power island which would expand the facility to 1,000 MW (collectively, the “Project”).  ENA is seeking proposals to acquire the Project (the “Proposed Transaction” or the “Transaction”) from qualified investors (the “Investor” or “Proposed Investor”).  The Project offers an Investor an attractive opportunity to participate in the growing California energy market.

The Project

ENA's interest in the Project is held by Pastoria Energy Facility L.L.C. (“PEF”), a limited liability company organized in Delaware for purposes of development of the Project.  PEF is a wholly-owned subsidiary of ENA, and ENA is currently the sole and managing member of PEF.  The Project will be located on an approximately 30-acre parcel of land that is owned by Tejon Ranchcorp and under an option to lease by PEF.  The site is approximately 30 miles south of Bakersfield, California and about 6.5 miles east of Grapevine, California.

The Project will include two power islands consisting of three combustion turbine generators (“CTGs”) operating in combined cycle mode utilizing a two-on-one configuration plus a one-on-one configuration (collectively, the “Power Islands”).  The Project is being developed to be capable of expansion to 1,000 MW.  Such an expansion, however, must be performed under a separate California Energy Commission (“CEC”) permit application.

The plant will use Best Available Control Technology (“BACT”) to minimize gas turbine emissions. To achieve BACT, PEF proposes to install Catalytica’s XONON™ emission control technology .  PEF will use dry low NOx (“DLN”) combustors and selective catalytic reduction (“SCR”) as alternate emissions control systems, if the XONON™ technology is not commercially and technically available when needed to meet the Project schedule.

Project Interconnects and Supply Agreements

The Project is conveniently located near an existing power substation and reliable sources of gas and water.

SCE Interconnection.  The Project will include a 1.38-mile  double circuit, 230 kV transmission line interconnection to Southern California Edison’s (“SCE”) existing Pastoria Substation.

Gas Supply.  The Project plans to use natural gas supplied via an 11.65-mile pipeline interconnected to the existing Kern River/Mojave pipeline. PEF will utilize up to an estimated 120 million standard cubic feet per day of pipeline quality natural gas.

Water Supply.  PEF will obtain its primary water supply from the Wheeler Ridge Maricopa Water Storage District (“Water District”) and its back-up supply from Azurix Corporation, an Enron affiliate (“Azurix”) (Is AZX still an Enron affiliate?).

Wastewater.  PEF will utilize a zero discharge system to treat wastewater streams.

Project Revenue Opportunities

The Project is a merchant facility located in the SP-15 pricing zone.  It will sell its output into the California Power Exchange (“PX”) or to third parties under bilateral contracts.  If the plant is expanded to 1,000 MW and interconnects at the Pardee Substation, it could be eligible for an reliability-must-run (“RMR”) contract with the California Independent System Operator Corporation (“ISO”).

Project Approvals & Agreements

CEC Permit.  In California, the CEC has the exclusive authority to site and license thermal electric plants with a capacity of 50 MW or greater.  It issues a single permit for construction and operation, which covers most applicable approvals.  PEF’s Application for Certification (“AFC”) was declared to be “data adequate” by the CEC on January 26, 2000.  ENA is now working with the CEC to expedite the process.  ENA currently expects the CEC permit to be issued and by December 31, 2000.

ERCs.  State and federal laws require “major” new sources of criteria pollutants that are locating in non-attainment areas to mitigate their impact on ambient air quality.  The Project will be a “major source” of NOX, PM10, VOC and SO2.  It will mitigate its impact in part through the acquisition of emission reduction credits (“ERCs”).  PEF has acquired all ERCs necessary to build and operate the Project.

[Why was this deleted, KJ?]

Water Agreement.  PEF’s water supply contract with the Water District is currently undergoing a legal validation process in state court.  Upon completion thereof on or about ____, the contract will be signed by the parties.  [Expand to add further details plus Azurix contract.]

Equipment Purchase Agreement.  PEF has made necessary contractual arrangements to purchase from General Electric Company (“GE”) the Power Islands. The delivery schedule will support a June 2003 commercial operations date.

Key Attributes & Success Factors

There are several factors that will make the Project competitive in California’s deregulated market:

Attractive Location.  The Project is strategically located in close proximity to the SCE power transmission lines and the gas transmission line jointly owned by Kern River Gas Transmission Company and Mojave Pipeline Company (“Kern/Mojave”). By interconnecting with the Kern/Mojave pipeline, the Project will intrastate pipeline charges saving the Project approximately $0.30 MMBtu or $2.10 MWh. The Project is also located in the more favorable and less congested SP-15 pricing zone.  Lastly, the Edmonston Pumping Station of California Department of Water Resources (“DWR”) is located less than two miles from PEF and the only direct access is over Tejon Ranchcorp’s property.

Growing California Energy Market.  The fast growing California economy of the 1990’s resulted in a much greater power demand than anticipated.  This demand, coupled with the fact that virtually no capacity has been added in California since the 1980’s, has created a need for capacity.  The CEC expects installed capacity to decline and demand to increase. As a low cost, early entrant to the growing California market, PEF is well positioned to meet California’s growing power requirements as well as to compete with existing generation capacity.

Advanced Stage of Development.  ENA is in the advanced stages of development of the Projectand expects to receive CEC approval for construction and operation of the Project by December 31, 2000.  Other applicable local permits and approvals are expectedin advance of construction.

Emission Reduction Credits Obtained.  PEF has acquired all  ERCs required to construct and operate PEF.  These ERCs represent virtually all credits available in the San Joaquin Valley Unified Air Pollution Control District (the “Air District”) bank that comply with Federal New Source rules..

Opportunity to Expand.  The Project site has been sized to accommodate a 250 MW expansion and the possibility of an RMR contract with the ISO.

Possible RMR Contract.  Given the tremendous congestion in SP-15 pricing zone near the Site, upon expansion to 1,000 MW and interconnecting at the Pardee Substation, PEF may become eligible for anRMR contract from the ISO.

Firm Commitment for Power Islands.  The Company has made contractual arrangements with GE for the Power Islands and they are scheduled for delivery by a date that will support a June 2003 commercial operations date.  Furthermore, the Company is in the process of securing contractual arrangements for the additional 250 MW power islandwhich would accommodate expansion of the Project..

Significant Barriers to Entry.  ENA believes that there are significant barriers that confront any potential power plant development in the region, including:  (i) access to water rights; (ii) availability of ERCs; (iii) air quality standards; (iv) access to industrially zoned property; (v) transmission interconnections, including the construction of new transmission lines and high pressure gas pipelines; (vi) availability of turbines; and (vii) environmental concerns.  The inability to overcome any of these obstacles could prevent or significantly delay competing plants from being constructed or operating economically.

Experienced Project Sponsor.  Enron, through its Wholesale Energy Operations and Services businesses such as ENA, is a leader in developing, constructing and operating energy-related infrastructure projects worldwide.  In North America, ENA is the clear market leader in commodity delivery and risk management, supported by extensive physical natural gas transportation, storage and electric interchange capabilities.

Transaction Overview, Process & Schedule

ENA prefers to sell all of its ownership interest in PEF.  However, in an effort to meet the expected objectives of a potential Investor, ENA will also entertain offers for less than 100% of PEF’s equity.  In either event, ENA strongly desires to retain some material continuing involvement with the Project.  The form of involvement can take the form of one or more contractual arrangements including a scheduling coordination agreement, a gas supply agreement, a transition supply agreement or marketing agreement.

On or about September 1, 2000, ENA intends to offer several potential Investors, including AES, the opportunity to bid on the Project and other projects.  The process and schedule for the sale of the Project and other projects will be set forth in the memorandum released in connection therewith.

## Project Overview

## Pastoria Energy Facility L.L.C.

The Project is being developed by ENA through PEF.  PEF is a wholly-owned subsidiary of ENA which, in turn, is a wholly-owed subsidiary of Enron.  ENA is currently the sole and managing member of PEF, with Tejon Ranchcorp ("Tejon") involved in the Project as a beneficial party.

## Project Description

General.  The Project will be a nominal 750 MW, natural gas fired, merchant electrical generating facility, with an option to expand to 1,000 MW upon receipt of applicable CEC approvals, which will sell its output into the deregulated electrical market either through contractual arrangements or throughPX or to third parties under bilateral contracts.  Electricity will be delivered to the existing electrical grid via a 1.38 mile 230 kilovolt (kV) double circuit interconnection to SCE’s Pastoria Substation.

The Project will incorporate two Power Islands consisting of three, natural gas fired, GE 7FA CTGs operating in combined cycle. Two CTGs will be installed in a “two-on-one” configuration with one GE D11 STG and one CTG will be installed in “one-on-one”, non-common shaft configuration with one GE A11 STG.

The Project is being developed with the expectation of expanding it to 1,000 MW upon receipt of applicable CEC approvals.  The Site, as described below, has been sized to allow for a 250 MW expansion. The western area of Site is allocated for a future CTG, in a one-on-one configuration, which may be added under a separate CEC permit at a future date.

Project Site.	The Project will be located on an approximately 30-acre parcel of land that is owned by Tejon and under option to lease option by PEF (the “Site”). The Site is approximately 30 miles south of Bakersfield, California and about 6.5 miles east of Grapevine, California.  Access will be provided from Edmonston Pumping Plant Road, which can be accessed from Interstate 5 via a new plant access road that will be constructed as part of the Project.

Project Location. This location and the configuration of the Project have been selected to best match operating needs for the transmission grid and the competitive power market. The Site takes advantage of nearby access to a natural gas fuel supply, water for cooling, and a tie-in location to the SCE Pastoria Substation. The Pastoria Substation, located 1.38 miles away, places the Project in the more favorable SP-15 pricing zone.  The Project’s power output will primarily flow south towards Los Angeles, thereby avoiding the congestion of or affecting Path 15 or Path 26. A gas transmission pipeline and water are only 11.65 miles and one mile away, respectively.  Also, the DWR’s Edmonston Pumping Station, one of the state’s largest power consumers, is located less than two miles from the Site and is a potential power customer.

Plant Equipment & Layout.  The Project will produce a nominal 750 MW based on a 75oF ambient temperature and 39 percent relative humidity. Each CTG will produce approximately 168 MW, the STGs will produce approximately 185 MW and 90 MW, respectively, and plant auxiliary equipment will consume approximately 15 MW. The net heat rate of approximately 7,000 Btu/kWh, based on the higher heating value (HHV) of natural gas, corresponds to an overall efficiency of approximately 50 percent. The Project is expected to have an overall availability of 95 percent or higher and could operate up to 8,760 hours per year.

The hot exhaust gas from each CTG flows through a heat recovery steam generator (“HRSG”). The HRSG extracts heat from the exhaust to produce the steam that powers the STG. The facility will employ proven gas turbine technology that is currently in use in California and in other locations around the country.

The plant will use Best Available Control Technology (“BACT”) to minimize gas turbine emissions. To achieve BACT, PEF proposes to install Catalytica’s XONON™ emission control technology, if commercially and technically available. This innovative technology is being developed and commercialized by Catalytica, ENA, and General Electric Company. This ongoing development effort is supported by the U.S. Department of Transportation, the CEC, and the California Air Resources Board . Completion of testing of the XONON™ system for GE 7FA turbines is scheduled to occur during the first quarter of 2003.

Recognizing the inherent risks associated with technology development, PEF proposes an alternate emissions control system, which will be used if XONON™ technology for F-Class turbines is not ready when needed to meet the Project schedule. The alternate emissions control system would use state-of-the-art DLN combustors in combination with post-combustion emission controls located in the HRSGs. These downstream controls include SCRs to control NOx and an oxidation catalyst to control CO. The height of the exhaust stack on each HRSG will be 213 feet, or less, as required to satisfy air quality requirements while minimizing visual impact.

The plant will use use a multi-cell mechanical draft, wet cooling tower to provide heat rejection. The cooling tower is expected to be a standard, induced draft, counter-flow type with fiberglass structure and film fill. The tower will incorporate plume abatement coils and high efficiency drift eliminators. The estimated average annual cooling water tower makeup rate is 2,925 gallons per minute (gpm).

Project  Interconnects and Supply Agreements

SCE Interconnection.  The Project will include a 1.38-mile double circuit, 230 kV transmission line interconnection to the existing SCE Pastoria Substation. The majority of the proposed line will parallel an existing transmission corridor.

Gas Transportation.  The Project plans to use natural gas supplied via an 11.65-mile 144- to 16-inch diameter pipeline interconnected to the existing 42-inch diameter Kern River/Mojave pipeline. The existing line is pressurized at 700-900 psig. PEF will utilize up to an estimated 120 million standard cubic feet per day of pipeline quality natural gas.

Water Supply.  PEF will obtain its primary water supply from the Water District and its back-up supply from Azurix.  The Azurix agreement has been executed, and the Water District agreement will be executed once validation proceedings for the Water District agreement become final.

Water District is a public agency that serves the southern portion of Kern County, managing the delivery and distribution of water from the California Aqueduct and underground storage facilities to local users. Water District currently has approximately 197,000 acre-feet of entitlements to receive water from the State Water Project and delivers 190,000 acre-feet in a normal year. Water District is also developing additional water storage capacity in the White Wolf basin that would add up to 50,000 acre-feet/year of extraction capacity to Water District’s current supplies.

Under the contract with Water District, Water District will provide District Pool water and conveyance through its distribution facilities.  District Pool water is water to which Water District entitled from the California Aqueduct in excess of its members’ needs.

PEF’s Water Supply and Management Contract with Azurix provides PEF with a guaranteed backup supply of water at a fixed price when Water District does not have water available for PEF.  Azurix will also manage the logistical aspects of scheduling both the Water District supply and the backup supply.

Wastewater.  The proposed PEF will generate an average wastewater stream of 760 gpm which will consist of cooling tower wastewater treatment and HRSG blowdown. PEF will utilize a zero discharge (“ZD”) system to treat wastewater streams with the exception of sanitary and storm water streams. The sanitary stream would continue to be discharged to the sanitary leach field. The stormwater stream would continue to be discharged to the onsite detention basins and then to Pastoria Creek.

The ZD will enable a portion of the wastewater stream to be recycled to the plant’s cooling water system. This will reduce the plant’s water consumption by approximately five to ten percent.

## Project Revenue Opportunities

### Wholesale Revenue Opportunities.  Once interconnected with the Western Systems Coordination Council (“WSCC”) and the ISO, bilateral wholesale revenue opportunities will be available from numerous market participants, including power marketers, public power agencies, municipal power utilities and energy service providers.  Further, the Project is located in the SP-15 zone of the PX and power sale revenues are available from sales into the PX.

Incremental Revenue Opportunities. Because the Site is located near the highly congested NP-15 and ZP-26 pricing zones, areas in need of reliability services, PEF could be an attractive candidate for the provision of an RMR contract.  PEF would most likely have to expand to 1,000 MW in order to qualify for such a contract.  If PEF is expanded to 1,000 MW, the Pastoria Substation would be inadequate for the entire load so interconnect at the Pardee Substation, which is 20 miles away.

Project Assumptions

Plant Performance Assumptions

Estimated performance assumptions for the Project are provided in the table below.  Assumptions are subject to change.

| Category |  | Amount |
| --- | --- | --- |
| [TO COME] |  |  |

Capital Cost Assumptions

Estimated capital costs for the Project are provided in the table below.  The amounts listed are subject to change prior to finalization of design, EPC agreement, interconnect agreements, etc.

| Category |  | Amount |
| --- | --- | --- |
| [TO COME] |  |  |

Operating Cost Assumptions

Estimated annual operating costs for the Project are provided in the table below.  Estimates are subject to change.

| Category |  | Amount |
| --- | --- | --- |
| [TO COME] |  |  |

Project Approvals and Agreements

## The CEC Licensing Process

In California, the CEC has the exclusive authority to site and license thermal electric power plants with a capacity of 50 MW or larger.  Pursuant to Title 20 of the California Code of Regulations and the Warren-Alquist Energy Resources Conservation and Development Act, the CEC’s Energy Facilities Siting and Environmental Protection Division (the “Division”) is tasked “ . . . to ensure that a reliable supply of electrical energy is maintained at a level consistent with the need for such energy for protection of public health and safety, for promotion of the general welfare, and for environmental protection.”  The Division prepares all environmental documentation for the CEC as required by the California Environmental Quality Act (“CEQA”).

The CEC issues a single permit for construction and operation, which covers most applicable approvals. PEF filed an AFC with the CEC to construct and operate the Pastoria Energy Facility—California Energy Commission Docket Energy Docket Number: 99-AFC-7.  The AFC was declared by the CEC to be “data adequate” on January 26, 2000—a significant milestone in that the CEC can then officially begin review of the application with a final decision required within 365 days.  The current CEC permitting schedule is set forth below:

CURRENT CEC PERMITTING SCHEDULE

| Date | Milestone |
| --- | --- |
| 11/30/99 | AFC Filed with CEC |
| 1/26/00 | CEC Declares AFC Data Adequate |
| 3/13/00 | Information Hearing and Site Visit |
| 7/26/00 | SJVUAPCD issues Final Determination of Compliance |
| 8/31/00 (est) | CEC Staff Final Staff Assessment |
| 9/18-19/00 | CEC Hearings |
| Not scheduled | CEC issues Presiding Members Report |
| Not scheduled | CEC issues “Final Decision” |

## Other Permitting Issues

While the CEC has overriding authority in California to site and license new power plants, the CEC process requires simultaneous review and full participation by all state and local agencies, as well as coordination with federal agencies.  In addition, the CEC works closely with interested public organizations and individuals to ensure full participation in the licensing process.

Below are briefs descriptions of the some of the major issues surrounding the Project:

### Agricultural Restriction. The Site is currently subject to the Williamson Act, a state land use policy that serves to preserve open space and agricultural land and prevent commercial development.  It is implemented by creating a voluntary contract between property owners and the county.  In return, the property owners receive favorable property tax treatment.  PEF is seeking a cancellation of a Williamson Act contract between Tejon and Kern County that currently encumbers the Site.  The Kern County Planning Department is currently scheduled to make its recommendation concerning such cancellation on August 24, 2000, and the Kern County Board of Supervisors is expected to render its decision concerning such cancellation on September 12, 2000.  Other Tejon land in the area has been successfully removed from the Williamson Act; however, none was used as a power plant site.

### Air Quality. The air quality aspects of the Project are subject to lead agency review and approval by the Air District which has principal responsibility for meeting federal and state air quality standards in the region and for permitting new resources of air pollution.  The Project will be a major source subject to Air District new source review requirements which require (1) an air quality impact analysis, (2) offset of pollutant emission increases in excess of specified thresholds and (3) application of BACT standards.

The Project location is in an area designated as a federal and state non-attainment area for ozone (and NO2 and VOC as precursors) and PM10 (NO2 and SO2 as precursors). The area is attainment for all other criteria pollutants, although most are designated precursors which are essentially treated as non-attainment pollutants under state and local regulations. While they are precursors under state/local rules, both NO2 and SO2 are officially attainment pollutants and are, thus, also subject to PSD requirements under federal regulations. Approximately 49 km (31 miles) north of the Site, Bakersfield is currently designated as non-attainment for CO. However, the Air District expects Bakersfield will be re-classified as a CO attainment area in response to recent air quality improvements.

The Air District is in the Environmental Protection Agency’s Region 9 Zone, along with the South Coast Air Quality Management District (“SCAQMD”).  For the purposes of new source review, SCAQMD has essentially defined BACT to be identical to the Lowest Available Emissions Rate (“LAER”).  Recent SCAQMD guidelines implemented in August 1998 require a BACT NOX standard of 2.5 ppmvd for new generation facilities.  PEF is being permitted to meet the same stringent air quality requirements as those promulgated by the SCAQMD.

State and federal laws require “major” new sources of criteria pollutants that are locating in non-attainment areas to mitigate their impact on ambient air quality. The Project is classified as a “major” new source of precursors of ozone (NOx and VOC) by Rule 2201 – New and Modified Stationary Source Review.  Kern County is classified as a Serious Ozone Non-Attainment Area. The Project, therefore, will be required to mitigate its impact on attainment of ambient air quality standards for ozone. This will be accomplished by acquiring ERCs.

Computer modeling approved by the U.S. Environmental Protection Agency (“EPA”) was used to predict air pollutant concentrations within six miles around the Site—all are below state and federal regulatory standards created to protect public health and safety.  Emission of NOX, VOC and PM10 will be fully offset by the acquisition of emissions reduction credits (“ERCs”) that have been certified or “banked” by the Air District.

The Project is also classified as a major source for PM10.  Kern County is classified a Serious PM10 Non-Attainment Area. Air District rules require PM10 emission offsets from all new sources which emit greater than 15 tons of PM10 annually. PM10 precursors include NOx, VOC and SO2.

The Project is classified as a major source for CO. Under Rule 2201, Section 4.2.1.1, the Project is exempt from offset requirements for sources in CO attainment areas. The Project’s air quality impact assessment for CO demonstrates that the Project will not cause or contribute to a violation, nor interfere with the attainment or maintenance, of any state or national ambient air quality standard for CO.

Offsets are required for SO2 because the annual potential to emit is greater than 150 lb/day.  SO2 allowances are also required under Title IV of the Clean Air Act Amendments and they will be acquired by the project under the Title IV compliance program.

PEF has acquired rights to all ERCs required for the Project.  The EPA and Air District have approved the ERCs for use in the Project.  [True?]

Biological Resources.  The San Joaquin kitfox, a protected species, is known to forage on and around the Site.  Construction and operation of the Project could attract or alter the patterns of the innately curious animal.  PEF has presented to the US Wildlife & Fishery Service (“USWFS”), and they are currently negotiating, a Habitat Mitigation Plan (“HMP”) to address any impact on the kitfox.  The HMP is expected to be approved by the USWFS prior to the commencement of construction.

Water District Water Contract.  PEF has filed with a California Superior Court for a declaratory judgment to validate the Water District contract.  When a contract is validated it is determined to be valid and enforceable by a court.  A validation proceeding is necessary when …………..[KJ??] The court has offered the contract for public comment.  The comment period ends on August 24 with a final hearing to be held on August 25, 2000.  Upon expiration of a 30-day waiting period, the validation will be final.

## Project Agreements

### Engineering, Procurement and Construction Turnkey Contract.  The Project will be built pursuant to an Engineering, Procurement and Construction turnkey contract (“EPC Agreement”) with a contractor to be identified. ENA will identify contractors but anticipates that an EPC Agreement will not be executed until after financial close..

### Labor Agreements.  ENA has committed to use union labor to construct, operate and maintain the Project, and agreements are in place with all requisite unions for construction, operation and maintenance of the Project.  [KJ, do you have the details?]

### Equipment Purchase Agreements.  On May 1, 2000, PEF and GE executed a contract under which PEF has agreed to purchase  the Power Islands .  In connection with the proposed 250 MW expansion, PEF is in the process of securing rights from GE to an additional power island.

### Tejon Agreements.

### PEF and Tejon and certain of their affiliates have entered into a “Transaction Agreement” and an “Option Agreement” concerning the Project.  The Option Agreement provides for a “Ground Lease” and “Easement Agreement” which will be entered into if PEF exercises its option under the Option Agreement.

The “Transaction Agreement” outlines structural elements of the relationship between PEF and Tejon which overlay the arrangements contained in the other agreements.  The Transaction Agreement gives PEF control over development and operation of the Project, and requires Tejon to actively support the Project in various ways .  The Transaction Agreement obligates Tejon, for a limited period of time, not to allow other merchant projects, or certain transmission lines or pipelines serving other merchant projects, to be located on Tejon Ranch.  The Project Agreement establishes rules for the assignment of interests in the agreements with Tejon, and gives Tejon the option to acquire certain development assets in the event the option under the Option Agreement should terminate without being exercised.

The “Option Agreement” gives PEF the right to lease the  Site under the Ground Lease and to acquire easements under the Easement Agreement.  It establishes a 6-year option period and option consideration PEF can at any time terminate its obligation to make future installment payments by giving up the option.  The unpaid balance of the option installment payments will be due at the earlier of financial closing (as defined in the Transaction Agreement) or option exercise.

The “Ground Lease” establishes the terms under which the Project will occupy the Site.  The Ground Lease is for an initial term of 25 years running from the “Commencement Date,” which is the earlier of commercial operation or the third anniversary of option exercise.  The Ground Lease would be signed when the Option is exercised.

The Ground Lease provides for fixed and variable rent.

The fixed rent begins at the Commencement Date, and is subject to escalation for inflation over the term of the Ground Lease.  The fixed rent is is to be scaled up or down if the capacity of the Project is more or less than 750 MW.

The variable rent also begins at the Commencement Date.  The variable rent will be calculated as a percentage  of the “spark spread” based on PX energy prices and the price of gas at the California/Arizona border.

A fixed amount subject to a cap is required to be advanced against the fixed rent, at a fixed monthly rate], beginning in the month the option is exercised.  These advances are repaid, with interest, by way of credit in equal monthly installments against the first 4 years of fixed rent payments.

The Ground Lease makes PEF responsible for all property taxes payable on either the land or the improvements for any period during the term of the Ground Lease.

The Ground Lease obligates PEF to decommission the Project and return the land to its preconstruction condition at the end of the lease.  (Buried pipelines may be abandoned in place in accordance with and to the extent allowed by applicable law and regulation.) At lease signing, PEF must post security for the decommissioning costs in the amount of $10 million.  The security will be a guarantee from the equity owners, a surety bond or other security satisfactory to Tejon.

The “Easement Agreement” establishes the terms under which the Project will be permitted to occupy laterals running from the Project site for access, transmission and gas lines and utilities required by the Project.  All these easements are included in the rent payable under the Ground Lease, except for any transmission lines required for direct service to the Edmonston pumping station or other users.  Those would be extra, and be paid for at the rates specified in the Transaction Agreement

ISO Participating Generator Agreement.  In order to supply energy into the ISO controlled grid, PEF will be required to enter into a Participating Generator Agreement (“PGA”) with the ISO.  Under the PGA, PEF is required to comply with the ISO Tariff and ISO Protocols.  The ISO Tariff requires that generating units such as PEF schedule energy, ancillary services and other services through a Scheduling Coordinator.  ENA’s affiliate EPMI is a certified Scheduling Coordinator for California capable of providing scheduling coordination services to the Project.  This agreement would not be executed into until after financial close

### ERC Option & Purchase Agreements.  ENA has entered into purchase and option purchase agreements to acquire all ERCS required to meet the Air District’s requirements for emissions offsets.

### Gas Transportation Agreements.  PEF has initiated discussion with Kern/Mojave on pipeline interconnection.  An agreement is expected to be executed prior to December 2000.

## :

Timeline See SAM

Upon receipt of the CEC permit, PEF expects the Project timeline to be as follows:

| Year | Quarter | Milestone |
| --- | --- | --- |
| 2000 | 4 | Obtain CEC Permit & Air Permit |
| 2000 | 4 | Financial Closing |
| 2001 | 2 | Begin Construction |
| 2002 | 4 | Start-Up Testing |
| 2003 | 2 | Commercial Operation |

Key Attributes & Success Factors

## Attractive Location

The Project’s premium location provides several upsides.  The Project is strategically located in close proximity to the Pastoria Substation and the Kern/Mojave gas pipeline. PEF will interconnect with SCE at the Pastoria Substation and Kern/Mojave 1.35 and 11.8 miles from the Site, respectively. The Project will avoidpaying  intrastate pipeline charges by interconnecting with the Kern/Mojave pipeline.  The wil save the Project saving the project approximately $0.30 MMBtu or $2.10 MWhThe Project is also located in the more favorable and less congested SP-15 pricing zone.Also, the DWR’s Edmonston Pumping Station, one of the state’s largest power consumers, is located less than two miles from PEF.  The pumping station requires up to 657 MW of power and the only direct access is over Tejon property, which represents an opportunity for sale of power from the Project.  PEF has obtained from Tejon exclusive easements from the Site to Edmonston until the earlier of April 30, 2005 of one year after commercial operations.  PEF’s direct access to Edmonston would allow it to supply Edmonston without incurring ISO or SCE transmission charges. SCE currently provides Edmonston its power under a contract which expires in 2004. The DWR’s Chrisman and Teerlink Pumping Stations whose aggregate demand is approximatley 200 MW are only 10 to 15 miles from the Project.

## Growing California Energy Market

The fast growing California economy of the 1990’s resulted in a much greater power demand than anticipated.  This coupled with the fact that virtually no capacity has been added in California since the 1980’s has created a need for capacity.  The CEC expects installed capacity to decline and demand to increase.  The major reasons for capacity decline may include (1) restructuring of QF contracts, (2) retirement of aging and noncompetitive plants, (3) retirement of nuclear plants and (4) increasing power demand from neighboring states.  California is a net importer of about 6,000 MW of electric power on an average.  Much of the in-state generation relied upon to meet California’s 30,000 MW average demand is relatively inefficient thermal, nuclear and renewable resources.  A CEC report concluded that growing California requires 6,700 MW of new generating capacity to meet reserve requirements. As a low cost, early entrant to the California market, PEF is well positioned to meet California’s growing power requirements as well as to compete with existing generation capacity.

## Advanced Stage of Development

### ENA is in the advanced stages of development of the Project.  The Project’s AFC was deemed Data Adequate by the CEC on January 26, 2000.  As a result of its efforts to expedite the process, ENA expects to receive CEC approval for construction and operation of the Project by December 31, 2000.  Other applicable local permits and approvals are expectedin advance of the start of construction.

## Emission Reduction Credits Obtained

PEF has acquired all ERCs required to build PEF.  These credits represent virtually all ERCs in the Air District bank that comply with the Federal new source review standards.  The EPA and SJUAPCD have agreed that the ERCs can be used for the Project.

## Opportunity to Expand

The Site has been sized to accommodate a 250 MW expansion.  ENA expects to sign a contract with GE for one power island including one GE 7FA and one GE A11 STG that will allow for the expansion.

## Possible RMR Contract

If PEF expands to 1,000 MW and it interconnects into the Pardee Substation, the Project may become eligible for an RMR contract from the ISO.  Any expansion cannot begin until all applicable approvals, including that of the CEC, are obtained.

## Firm Commitment for Equipment

The Company has contracted with GE for two Power Islands and they are scheduled for a delivery that would support a June 2003 commercial operations date.  Furthermore, the Company is in the process of securing an additional power island, which will allow expansion of the Project to 1,000 MW.  As availability of turbines is expected to be severely limited during the next few years, any project under development without firm rights to equipment is at risk of substantial delays.

## Significant Barriers to Entry

ENA believes that there are significant barriers that confront any potential power plant development in the region.  PEF has encountered and has addressed, or is in the process of addressing, obstacles which would be encountered by future power plant developers, including:  (i) access to water rights; (ii) availability of ERCs; (iii) air quality standards; (iv) access to industrially zoned property; (v) transmission interconnections, including the construction of new transmission lines and high pressure gas pipelines; (vi) availability of turbines; and (vii) environmental concerns.  The inability to overcome any of these obstacles could prevent or significantly delay competing plants from being constructed or operating economically.

## Experienced Project Sponsor

Enron, through its Wholesale Energy Operations and Services businesses which includes ENA, is a leader in developing, constructing and operating energy-related infrastructure projects worldwide.  In North America, ENA is the clear market leader in commodity delivery and risk management, supported by extensive physical natural gas transportation, storage and electric interchange capabilities.

## Project Sponsor

## Enron Corp.

Enron Corp. is an integrated natural gas, electricity and communications company with headquarters in Houston, Texas.  Enron Corp.’s operations are conducted through its subsidiaries and affiliates, which are principally engaged in the transportation of natural gas through pipelines to markets in the United States; the generation and transmission of electricity to markets in the northwestern United States; the marketing of natural gas, electricity and other commodities and related risk management and finance services worldwide; and the development, construction and operation of power plants, pipelines and other energy related assets in international markets. In addition to these operations, Enron Corp. has added Enron Energy Services, Azurix and more recently Enron Broadband Services. Enron Energy Services provides direct sales of energy products and services to end-use customers. Enron Energy Services offers a broad range of products including natural gas, electricity, and facilities management to commercial and light industrial customers.  Azurix was formed to pursue opportunities in the global water business.  As its first step in establishing this new water business, Azurix acquired Wessex, a water and wastewater services company based in England.  Enron Broadband Services provides high-quality high-bandwidth transmission over the Internet.

## Enron North America Corp.

ENA’s activities are segmented into two categories:  (i) Commodity Sales and Services and (ii) Energy Assets and Investments.

ENA provides integrated energy-related products and services to customers throughout the US, Canada, Mexico, and Europe including the development, construction and operation of power plants, natural gas pipelines and other energy-related assets, energy commodity sales and services, risk management products and financial services.  ENA works closely with other units under the Enron Corp. umbrella to leverage industry expertise to provide comprehensive engineering and construction solutions for power and pipeline facilities, often serving as turnkey contractor or Project manager for such facilities.

The North American deregulated energy market is growing rapidly and becoming increasingly complex.  ENA intends to leverage its relationships with customers, dealers, and other industry players combined with its extensive portfolio of generation assets, pipelines, proprietary systems, and experienced people to maintain its position as the leader in the deregulated North American energy markets.

Commodity Sales and Services.  The commodity sales and services operations include the purchase, sale, marketing and delivery of natural gas, electricity, liquids and other commodities. Also included in this business is the management of certain operating assets that directly relate to the business, including non-regulated domestic intrastate pipelines and storage facilities.  ENA’s physical energy business provides reliable delivery of energy commodities at predictable prices.   ENA has been the leading marketer of electricity and natural gas for the past several years.

The risk management activities consist of the management of ENA’s long-term energy commodity contract portfolios (transactions greater than one year) and restructuring of existing long-term contracts. ENA manages its commodity and asset portfolios in order to maximize value, minimize the associated risks and provide overall liquidity.  In addition, ENA provides risk management products and services to energy customers that hedge movements in price and location-based price differentials.  ENA’s risk management products and services are designed to provide stability to customers in markets impacted by commodity price volatility.

Energy Assets and Investments.  This business includes the development and construction of power plants, pipelines and other energy infrastructure.

ENA’s financing and investing activities provide capital to energy-related businesses seeking debt or equity financing, including loans and equity investments.  In conjunction with the other functional groups, ENA helps to create innovative financing solutions to best take advantage of capital markets to bring value both to ENA and its clients.

## Pre-Operations Risk

Inherent in the fact that the Project is still under development are the following risks:

- Permitting, including CEC approval and Tejon’s finalization of the kitfox HMP with the USWFS

- Removing the Site from the Williamson Act

- Receiving a validation of the water supply agreement with the Water District

- Construction delays

- Changes to California deregulated power market

Transaction Overview, Process & Schedule

ENA prefers to sell all of its ownership interest in PEF.  However, in order to meet the expected objectives of a potential Investor, ENA will also entertain offers for less than 100% of PEF’s equity.  In either event, ENA strongly desires to retain some material continuing involvement with the Project.  The form of involvement can take the form of one or more contractual arrangements including a scheduling coordination agreement, a gas supply agreement, a transition supply agreement or marketing agreement.

Financial close is not expected to occur until the final CEC permit is received.  Until such time, ENA will continue its development efforts to achieve that key milestone.

On or about September 1, 2000, ENA intends to offer several potential Investors, including AES, the opportunity to bid on the Project and other projects.  The process and schedule for the sale of the Project and other projects will be set forth in the memorandum released in connection therewith.

[.

Exhibits

Exhibit 1:  Industry Overview

## The information in this section has been included based on a variety of third party sources.  The Company has not independently verified the information contained herein.

[Still updating]

## Overview

Since 1990, the world electricity markets have begun to undergo an unprecedented wave of privatization, disaggregation (separation by spin-off, sale or other means of segregating a utility’s generation, transmission and distribution assets into separate companies) and regulatory change.  In an effort to reduce inefficiencies and lower costs, in the past five years, the United Kingdom, Australia, Argentina and Brazil, among others, have privatized and disaggregated their electric utility monopolies and introduced competition into their electric power sectors.

The United States market is evolving in similar fashion, with privately owned monopolies being restructured.  Historically, regulated electric utilities have provided bundled electricity service (generation, transmission and distribution) within exclusive franchise service territories.  Since the passage of federal legislation in 1992, the Federal Energy Regulatory Commission (“FERC”) has implemented measures to facilitate competition in the wholesale power marketing arena.

Deregulation of the power markets in California, and the shift towards pricing based on market principles, has fundamentally changed the structure of the California energy industry.  New gas-fired resources will be well placed to benefit from their cost advantage over older, less efficient power plants.  The continued load growth  projected to occur in California will lead to higher power prices and larger margins for efficient gas-fired generation assets such as the Project.

## California Economic Growth

According to the California State and Commerce Agency, California has the largest economy in the U.S. and the seventh largest in the world.  The state currently provides 13% of the U.S. Gross Domestic Product and generates more goods and services than all of the other western states combined.  Twenty percent of U.S. import and export trade is handled through California ports.  Since 1995, California has outperformed the rest of the U.S. in both job growth and personal income growth by a considerable margin.  From 1999 to 2005, California is projected to add to its already large job base of 13.6 million at an average annual rate of 2.4% and to its personal income base of $930 billion at an average annual rate of 2.6%.

California has the largest population in the U.S.—its 33 million inhabitants encompass approximately 12% of the U.S. population.  According to the U.S. Census Bureau, the state’s population is expected to grow at an average annual rate of 1.7% between 2000 and 2020.

## Western Systems Coordination Council

The California market is part of the Western Systems Coordination Council (“WSCC”), which promotes electric service reliability in the western portion of North America. The WSCC develops and monitors planning and operating criteria for the interconnected transmission system.  Membership in WSCC is voluntary and open to major transmission utilities, transmission dependent utilities, and independent power producers/marketers.

The WSCC region covers an area of nearly 1.8 million square miles.  It represents the electric power systems serving all or part of the 14 western states of the U.S. (including California), as well as the provinces of Alberta and British Colombia in Canada and the northern portion of Baja California in Mexico.  The WSCC’s 86 members and 22 affiliate members provide electric service to approximately 65 million people throughout the region.

California Market Structure

California is at the forefront of the trend towards energy industry deregulation in the U.S.  California opened to competition on April 1, 1998 when electricity consumers in the ISO-controlled territories of PG&E, Southern California Edison Company (“SCE”) and San Diego Gas and Electric Company (“SDG&E”) were allowed to choose their electricity supplier directly.   The load of these three utilities accounts for approximately 70% to 75% of the total California electricity market.  It is anticipated that the in-state municipal entities, which account for 20%–25% of California’s electrical load, will eventually join the ISO.

Under the new competitive environment, generation owners are able to sell electric energy to the Power Exchange (“PX”) as well as through bilateral contracts directly to end-use customers or power-marketers.

In addition to energy sales, generators are able to obtain revenues for:

- Capacity, option or fuel tolling agreements with power marketers;

- Sales of ancillary services to the ISO including sale of reserves (part of the Energy Management Agreement), voltage support and black start capability; and

- Must-run and other local reliability services with the ISO (described below).

Other states in the U.S. are also moving towards deregulation.  This means that owners of generation facilities in the California market will have access to the deregulated California market as well as the opportunity to take advantage of other western energy markets as they deregulate in the U.S.

### Fixed Transmission into California.  As illustrated below, the transmission capacity supporting the importing of power into California from other WSCC load/transmission zones is currently limited to 20,116 MW. However, operating practices and simultaneous constraints currently limit the simultaneous import capability to California to substantially less than 20,116 MW.

transmission capacity into california (MW)

Source: Western Systems Coordinating Council

North to South imports into Northern California from the California-Oregon border are limited to about 4,800 MW from the California-Oregon Transmission Project (“COTP”) and Pacific Interchange (“PACI”) transmission projects (with actual rating levels generally set at lower thresholds).  South to North imports into Northern California from Southern California on Path 15 are limited to about 3,000 MW.  Further, depending upon the generation level of PG&E’s Diablo Canyon plant at the Midway-Vincent interchange, total imports into Northern California can be limited to as little as 6,278 MW or below under some load and generation conditions.

These transmission constraints severely limit the ability of imports to satisfy current and projected supply shortfalls.

Power Exchange.  The PX is a FERC-jurisdictional wholesale electric market.  The PX is a multi-faceted pool designed to serve both as a physical delivery mechanism and as a spot market.  The PX provides a competitive auction for day-ahead and hour-ahead market prices for electricity with the objective of ensuring that adequate generating resources are available for physical delivery to meet PX loads.  Generators submit supply bids by hour, which are matched with demand bids from energy purchasers.  The PX determines a Market Clearing Price (“MCP”) for each hour and determines demand and supply schedules, which are submitted to the ISO for integration with non-PX generation and load schedules.

Through 2001, PG&E, SCE and SDG&E must bid their non-nuclear generation into the PX and obtain electricity on behalf of full-service customers from the PX.  These restrictions do not apply to non-utility owners of divested assets or to owners of new merchant generation.  However, the load of these three utilities accounts for approximately 70% to 75% of the total California electricity market on a energy consumption basis so the current trading volumes in the PX are substantial.

Independent System Operator (“ISO”).  While nationwide ownership of transmission systems may remain with the electric utilities, many utilities are preparing to transfer control of their transmission systems to newly formed ISOs.  Federal regulations require ISOs to ensure that all participants have equal and fair access to the transmission system and that high levels of system reliability are maintained.  Once power is scheduled for delivery, it is the ISO’s responsibility to get the power to the distribution company and the distribution company’s responsibility to deliver power to the end user.  The ISO exercises operational control of all facilities determined by FERC to constitute transmission facilities.  The ISO is tasked with maintaining system reliability and with ensuring open and non-discriminatory access to transmission facilities.  In addition, the ISO operates energy markets for Imbalance Energy, Ancillary Services (Automatic Generator Control, Spinning Reserves and Non-Spinning Reserves), Replacement Reserves, Black Start Capability, Voltage Support and Transmission Congestion Management which are relied upon to provide real time balancing of load and generation.  Finally, the ISO coordinates day-ahead and hour-ahead schedules for all market participants including the PX and other participating Scheduling Coordinators.

### Reliability “Must-run” Resources.  As part of its mandate to ensure system reliability, the ISO must assess the need for “reliability-must-run” (“RMR”) generating units necessary to support operation of the transmission system.  The ISO has the right to call on RMR units for operation pursuant to a “must-run” agreement, which provides for payment by the ISO of specified amounts for fixed, variable and start-up costs of the unit.  A recent ISO study recommends the designation of 245 units as RMR units for 2001.  These units represent over 15,000 MW of generating capacity.  In the long-run, the ISO would like to minimize reliance on “must-run” agreements with generators and instead ensure reliability through competitively procured ancillary services. Therefore, the “must-run” agreements are currently cancelable with 90 days notice if the ISO determines that a given RMR unit is no longer required for reliability.  [Is last sentence true?]

### Regulatory “Must-take” Resources.  The rules governing the deregulation of the California market call for certain generation resources within the state to be designated “must-take”.  These resources will be dispatched regardless of their economic competitiveness.  These “must-take” resources encompass approximately 5,000 MW of nuclear plants, approximately 10,000 MW of QF’s, as well as certain other entities and plants with pre-existing contracts.  “Must-take” generation will decline significantly after 2001 as PG&E’s 2,200 MW Diablo Canyon nuclear plant and SCE’s approximately 580 MW share of the Palo Verde nuclear plant will cease to be “must-take” after 2001.  The state’s remaining nuclear capacity will lose its “must-take” status after 2003.  QFs will maintain “must-take” status for the remainder of their contract lives; however, these contracts could be restructured or bought out, and most contracts will expire by 2015.  In addition, many of the QFs under the Standard Offer (“SO4”) contracts may cease to be competitive when PX prices are phased in as the determinant of the short-run avoided cost (“SRAC”) payments in their contracts.

## Market Size

### Loads and Resources.  According to the WSCC, The California/Mexico region of the WSCC consumes roughly 35% of all power utilized in the WSCC.  In 1998, this region consumed 254,566 Gigawatt hours (“GWhs”) of electricity.  Between 1988 and 1998, energy usage in the California/Mexico region grew at a compound growth rate of 1.3% per year with usage forecast to increase by 1.8% per year through 2008 to over 300,000 GWhs per year.

As illustrated below, the California region contains a variety of generation resources including fossil, nuclear, hydroelectric, geothermal, cogeneration and renewable resources.  Total production for the region of 211,655 GWhs was substantially below consumption due to a heavy reliance upon net imports to serve the California market.  According to the CEC, California imported 51,185 GWhs of electric energy in 1997.

1997 electric energy production (california)

Source: California Electricity Commission

The ISO Draft 1998 Transmission Reliability Report stated that the total 1998 installed capacity in California is approximately 50,002 MW, which is expected to decline to 49,186 MW in the year 2003. Although installed capacity is expected to decline, demand is expected to increase.  The CEC estimates total energy demand growing by approximately 1.5% per annum to the year 2015. The projected increases in both peak incident and total demand will tend to exacerbate the shortfall of supply over demand.  This shortfall will worsen over the next several years as load will continue to grow prior to the availability of new generation resources.

The CEC expects that California will have a serious electricity reliability problem from 2002-2005.  The table below shows capacity needs in California during peak hours.

|  | Temperature Conditions |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1-in-5 Year Condition | 1-in-10 Year Condition | 1-in-40 Year Condition |
| Year | Average |  |  |  |
| 2001 | 1,541 | 3,524 | 4,924 | 6,394 |
| 2002 | 2,707 | 4,731 | 6,159 | 7,658 |
| 2003 | 3,897 | 5,961 | 7,418 | 8,947 |
| 2004 | 5,111 | 7,216 | 8,702 | 10,262 |
| 2005 | 6,349 | 8,496 | 10,012 | 11,603 |

Source:  California Electricity Commission

The capacity shortages anticipated by the CEC have been realized on several occasions during the summer of 2000.  Through August 22, the ISO has regularly declared a Stage One and a Stage Two Emergencies times asking customers to voluntary curtail their usage.  A Stage One Emergency is when oppressing reserves fall below seven percent or are expected to do so within two hours. A Stage One Emergency is when oppressing reserves fall below five percent or are expected to do so within two hours.  When operating reserves fall below one and one-half percent a Stage Three Emergency will be declared.

As discussed below, supply may also be affected by the closure of a number of plants that are unable to economically generate in the new marketplace, and by the transmission constraints relevant to the California / Mexico Power Area of the WSCC.

## Changing Supply Profile

### Increasing Cost of Imports.  The cost of imports is also expected to rise. It is most effective for owners of generation resources to utilize the output of efficient generation resources within an area for the needs of that particular area, with the output of less efficient resources available for export. The growth of loads in the WSCC will absorb a higher portion of output of efficient or low cost resources, worsening the average efficiency or cost of plants used for exporting energy to Northern California.  This will lead to rising energy rates, and supports the construction of efficient plants such as the Project in Northern California.  In addition, as the remaining states deregulate and cease to provide plants with reimbursement of stranded capacity costs, plants exporting energy will have to recover these capacity costs in the wholesale market, increasing energy prices as a result.

### Potential Closing of Non-Competitive Plants.  Under the new market-based system, if a generation facility is unable to compete at the market-clearing price or if the facility is not required for RMR purposes, then that facility is likely to be uneconomic for continued operation.  In 1997, nuclear, geothermal, wind, solar and organic waste resources accounted for over 22% of the total electric energy generated in California.   The following table lists the total installed capacity of these generating resources. As discussed below, a large portion of this capacity is unlikely to survive the transition to a competitive marketplace.

Further, the units designated as RMR units by the ISO are largely the relatively high-cost, vintage fossil units placed in service by the California investor-owned utilities.  The system cost of these resources can be displaced by less costly, new combined cycle technology.  The ISO is interested in supporting the development of new generation to help eliminate the contingency deficit in the short run and to displace of existing RMR resources over the longer term.  This represents a potential opportunity for efficient facilities such as the Project.

### Nuclear Plant Competitiveness.   Nuclear plants have high cost structures, are subject to restrictive regulatory requirements including requirements for the disposition of spent fuel, and are burdened with stranded costs.  Pursuant to AB 1890, California’s utilities have the opportunity to recover CTCs relating to nuclear stranded costs during the “rate freeze” period through March 31, 2002.  Exceptions are the SONGS settlement which extends through 2003 and ongoing CTC recovery for nuclear decommissioning costs.  However, at the end of 2003 California utilities will no longer be able to recover going forward operating costs for nuclear facilities from ratepayers but rather continued operation will be governed by market forces.

The high costs associated with nuclear power generation, the regulatory restraints and the expected future market clearing prices lead us to believe that certain nuclear plants may shut down once the CTC recovery is completed by the end of the year 2003.  To the extent that particular nuclear plants are required by the ISO for reliability such generation capacity may be retained and supported through RMR contracts.  However, this would likely be a very controversial course of action.

NUCLEAR AND RENEWABLE RESOURCES

| Resource | Capacity | Capacity |
| --- | --- | --- |
|  | MW | MW |
| Diablo Canyon | 2,160 |  |
| San Onofre | 2,044 |  |
| Subtotal nuclear |  | 4,204 |
| Municipal solid waste | 320 |  |
| Wind | 1,676 |  |
| Geothermal | 2,428 |  |
| Biomass | 650 |  |
| Solar | 363 |  |
| Subtotal renewable |  | 5,437 |
| Total Resources |  | 12,966 |

Sources:	Resource Data International, CEC “California Power Plant Data Information” dated June 1998, Utility Cogeneration and

Small Power Production Semi-Annual Reports to the CPUC

### Aging Resource Base.  California has an aging generation fleet. As the table below shows, 62% of California’s generation in 1998 had been in service at least thirty years. As plants age, plants require more maintenance and are more prone to forced outages, decreasing their competitiveness in a deregulated market.  The increased maintenance time and decreased reliability forces the ISO to increase reserve margins.

### California QF Contracts.  The QFs, which account for approximately 25% of the total resource mix for the utilities in California, possess long term contracts where the QFs receive both energy and capacity payments from the California utilities.

Source: Southern California Edison, PG&E, SDG&E Annual Status Reports

Under a large portion of these contracts, the energy price was fixed at a relatively high level for the first ten years of the contract, reverting to the market price thereafter.  In general, for most QF’s, the energy price components will revert to the PX market price on or before the year 2000 with capacity payments remaining in place.  Although the QFs will continue to have a portion of their revenue allocated as “must-take” (the above market capacity prices), many QF projects will become commercially at risk once the energy price received reverts to the PX price.  In particular, QFs relying upon organic waste, wind, solar and geothermal technologies appear to be the least competitive, and may not be able to cover their operating costs even with the capacity payment plus the PX price for energy.  In addition, the long-term depletion of the steam fields supplying a number of the geothermal power plants will result in their eventual closing.

### Hydroelectric Relicensing Impacts.  FERC administers the hydropower program in the U.S. and is responsible for issuing and reissuing of licenses for the hydropower projects among other things.   When a license issued to a private entity expires, the FERC may issue a new license (relicense) to the original holder of the license, or to a new licensee.  The relicensing process thus enables the FERC to effect changes in the ownership of the hydroelectric plants, vary any restrictions in operation and even close some hydroelectric projects.  When undertaking a relicensing determination, the FERC must consider any new environmental regulations that are in force.  Many hydro projects were originally licensed prior to the development of federal and state environmental regulations.  The impact of the new regulations on licenses is commonly an increase in marginal costs of production and a decrease in the capacity of production.

HYDROELECTRIC PROJECTS FACING RELICENSING IN NORTHERN CALIFORNIA

| License Expiration Year | Number | Capacity MW |
| --- | --- | --- |
| 1999 | 1 | 99 |
| 2000 | 2 | 23 |
| 2001 | 0 | 0 |
| 2002 | 1 | 20 |
| 2003 | 4 | 462 |
| 2004 | 5 | 513 |
| 2005 | 3 | 34 |
| 2006 | 0 | 0 |
| 2007 | 5 | 1,596 |
| Total | 21 | 2,747 |

Source: Federal Energy Regulatory Commission

### Air Pollution Concerns.  Technology incorporated into newer gas-fired plants limit the air emissions by these plants.  This technology is not present in older oil and coal fired plants, leaving these plants subject to potentially costly retrofits to bring emissions control within current standards.  The potentially costly retrofits leave old oil and coal fired generation facilities in jeopardy of closure, or alternatively will increase the ISO’s interest in new, more efficient generating plants.

For the purposes of new source review the South Coast Air Quality Management District (“SCAQMD”) has defined Best Available Control Technology (“BACT”) to be identical to the Lowest Available Emissions Rate (“LAER”).  As a result, cost is not taken into consideration by SCAQMD in the determination of BACT standards. If new market entrants are able to meet this strenuous standard, it is likely that existing generation will come under increasing pressure to comply with the BACT/LAER standards applicable to new generation.  Retrofitting would be costly, and may lead to closure of plants.  Alternatively, an ISO faced with paying for these retrofits to ensure an RMR plant availability will also become more interested in new, more efficient and inherently cleaner generating plants.

Recent Market Developments

California has experienced energy prices significantly higher this summer than it have expected.  Prices in the PX’s Day Ahead market and the ISO’s real time energy market reached the ISO’s $750/MW price caps during many hours.  The ISO examined the marketto determine the reasons for the high wholesale prices during the months of May and June and concluded that several factors contributed including (1) unusually high demand for electricity region-wide due to unseasonably high temperatures and economic growth, (2) a doubling of gas prices over last year, (3) unusually high number of scheduled and forced outages and (4) the fact that no signficant new supply has been added in California in recent years.  The combonation of very tight supply and demand conditions – in conjunction with very limited ability of consumers to reduce consumption in response to rising prices – created absolute shortages of supply.  The ISO alleges that this supply shortage created the opportunity to exercise market power which some large generators exercised to artificially inflate wholesale prices.

Rising power prices have caused much uproar throughout the state.  This is especially true in SDG&E’s service territory where price prices have double since last year.  SDG&E is the only IOU who is not currently subject to the statuory rate freeze, and therefore subject to true market pricing, as it has been paid for all of its stranded costs.  In response to a large public outcry, ISO has temporarily lowered the wholesale price cap from $750/MW to $250/MW.  This cap reduction expires in October.

In an effort to improve market performance and lower energy costs the ISO recommends actions in addition to lowering the price cap including:

- Accelerating the permitting and siting of generation and transmission projects.

- Placing a priority on developing load responsive programs and developing barriers to load’s ability to hedge.

- Ensuring UDCs have adequate incentives to conduct their wholesale market activities in the most efficient manner possible to keep rates low.

- Promoting robust retail competition by removing key regulatory barriers.

(?)

(?)

(?)

(?)

(?)

(?)

(?)