EXHIBIT "A"

Scope of Work

Instromet, Inc. and Enron Machine and Mechanical Services, Inc. Subcontract for the Fabrication of Meter Spools

- Procure material for fabrication of meter spools as per Instromet Specifications from Instromet's drawings and prints.

- Inspect material upon delivery to Enron Machine and Mechanical Services, Inc. (Subcontractor) Clifton, Kansas machine shop and ensure material meets specifications & that heat numbers and paper work match.

- Store material

Pipe and flanges will be stored outside Subcontractor's machine shop facility

Nozzle material will be stored inside Subcontractor's machine shop facility

- Meter Spool Manufacture and Weld-up Per Instromet Design Specifications

Saw spool pipe to rough length

Machine spool pipe to finish length and turn OD to finish size

Weld flanges onto spool pipe according to Instromet specifications

Weld “weld-o-lets" in place on pipe

Install nozzles onto pipe and weld in place

Drill holes through pipe for transducer

X-Ray flange welds and magnaflux nozzle welds

Hone ID of pipe to specification

Inspect nozzle placement

Hydro test

Sandblast and Paint as per specifications

- Up and Down Stream Spools Manufacture Per Instromet Design Specifications

Saw spool pipe to rough length

Machine spool pipe to finish length

Weld flanges onto spool pipe according to Instromet specifications

Weld “weld-o-lets in place on pipe

X-Ray flange welds

Hydro test

Sandblast and Paint as per specifications

- Fill out final inspection sheet for Instromet

- Mark spools with correct serial numbers

- Package X-Ray film, mill test reports and hydrostatic test documents with each spool assembly

- Crate and deliver fabricated spools at Point of Delivery described in this Agreement for shipment  of spools byInstromet to Instromet's destination