Bakersfield Area Power Plant Projects

SUMMARY

Seven power plant developers have filed, or are expected to file, with the California Energy Commission (CEC) to construct approximately 4818 megawatts of “combined cycle” generation in the Bakersfield, California, area.  If all are built, the associated gas load may approach 810 MDth/day.

PG&E-NEG’s La Paloma Generating Project is the first Bakersfield area project approved by the CEC.  Located in McKittrick, it’s expecting completion in 2002.  Other project completions will follow.

Of the seven projects, five propose to obtain gas supply from Kern/Mojave, one from Occidental Petroleum/SoCalGas, and the last project will connect to Southwest Gas, but take supplies from SoCalGas, Kern/Mojave Pipeline or PG&E.  If all these power projects are constructed, there is not enough existing capacity to serve the load.

BACKGROUND

Forty miles west of Bakersfield, at McKittrick, we find an amazing confluence of prolific gas/oil fields, six natural gas pipelines, and PG&E’s electric transmission system. For this reason, four power projects, 2348 megawatts or over 370 MDth/d, propose siting in the area.  Three additional projects, 2470 megawatts or about 440 MDth/d, propose siting south and east of Bakersfield.   All seven projects are located south of the “Path 15” in the electric grid.

POWER PROJECTS

Developers selected the McKittrick area to take advantage of the existing infrastructure and to minimize environmental impacts.  With a ready natural gas fuel supply and closeness to the existing PG&E 230 kilovolt breaker, four projects have proposed connecting to the electric grid at PG&E’s Midway Substation near Buttonwillow.  Three McKittrick area power projects state their intent to source natural gas supply from Kern/Mojave Pipeline and a fourth from Occidental Petroleum’s Elk Hills Naval Oil Reserve and/or SoCalGas.  The three power projects south and east of Bakersfield state their intent to source gas supply from either Kern/Mojave or Southwest Gas.  Project details are shown below.

| Table No 1 Bakersfield Area Power Projects |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description/ Sponsor | Location/ Service Territory | Status | Megawatts | Approx. Mdth/day: Supplier |  |
| McKittrick Area |  |  |  |  |  |
| La Paloma/ PG&E NEG | McKittrick/ SoCalGas | Approved | 1048 | 160 Kern/Mojave |  |
| Sunrise/ Texaco Global | Fellows/ SoCalGas | Approved | 300 | 50 Kern/Mojave |  |
| Elk Hills/ Sempra & Occidental | McKittrick/ SoCalGas | Approved | 500 | 80 Kern/Mojave |  |
| Sunset/ Mission Energy Subtotal Others | Fellows/ SoCalGas | Approved | 500 2,348 | 80 Kern/Mojave 370 |  |
| High Desert/ Inland Energy & Constellation Power | Victorville/ S.W.Gas | Approved | 720 | 130 SoCalGas, Kern/Mojave, or PG&E |  |
| Antelope Valley/ Enron | California City/ Open | Expected | 1,000 | 180 Kern/Mojave |  |
| Pastoria/ Calpine | Tejon Ranch/ SoCalGas | Filed | 750 | 130 Kern/Mojave |  |
| Subtotal |  |  | 2,470 | 440 |  |
| Total |  |  | 4,818 | 810 |  |

Source:   California Energy Commission (CEC)

Website - http://www.energy.ca.gov/maps/siting_cases.html

McKittrick Area

On October 6, NEG’s $500 million La Paloma Generating Project received final CEC approval for construction and operation.  To be completed in 2002.  The 1048-megawatt plant will be the largest of the four McKittrick area projects.  A 370-foot pipeline to Kern/Mojave Pipeline will be constructed for gas supply.  A proposed 15-mile bundled 230-kilovolt double circuit overhead electric transmission line will be built to interconnect to PG&E’s Midway Substation at Buttonwillow.  La Paloma is in SoCalGas’ certificated service territory.

The Sunrise Cogeneration Power Project has filed with the CEC to construct a 300 megawatt generating and steaming facility in Fellows, just south of McKittrick.  Hearings to approve the project are expected to conclude in early November 1999.  Sunrise plans to source gas supply from Kern/Mojave.  Sunrise plans to build an additional 230-kilovolt transmission line in parallel to the existing 230 kilovolt line and connect the plant to PG&E’s Midway Substation at Buttonwillow.  The site is in SoCalGas’ certificated service territory.

The Midway Sunset Cogeneration Project is expected to file with the CEC to construct a 500 megawatt facility in Fellows, just south of McKittrick. The site is in SoCalGas’ service territory.  Sunset plans to source gas supply from Kern/Mojave.  A new 15 mile electric transmission line (the Sunrise-Valley Acres transmission line) will be built to the new Valley Switching Station for connection to the existing California Department of Water Resources and PG&E 230 kilovolt transmission line.  The owners also own and operate the existing 10 megawatt North Midway Sunset project served by Texaco’s foreign gas pipeline.

The Elk Hills Power Project is a 500 megawatt combined cycle facility located 25 miles west of Bakersfield in the center of the 74 square mile Elk Hills Oil and Gas Field operated by Occidental of Elk Hills.  The project will include a 2500-foot, 10-inch gas pipeline owned and operated by Occidental and a new nine-mile overhead transmission line to a new electric substation near Tupman.  A SoCalGas eight-inch line crosses the Elk Hills field near the project site.  Occidental will provide its own gas supply with SoCalGas as a backup.

Other Power Projects

Three power projects are planned south and east of Bakersfield: High Desert, Antelope Valley, and Pastoria.

High Desert Power Project has filed to construct a facility in Victorville at the Southern California International Airport, formerly George Air Force Base.  The area is certificated to Southwest Gas.  Kern/Mojave has filed to build a lateral to the plant.  The SoCalGas and PG&E systems already serve the Victorville area.  A 7.3- mile electric transmission line will need to be built to connect with the Southern California Edison (SCE) 230 kilovolt Victor Substation.

Antelope Valley is expected to file with the CEC to construct a 1000- megawatt plant about 60 miles southeast of Bakersfield near California City and Edwards Air Force Base. The plant is designed to help reduce the CA-ISO’s reliance on “reliability must run” contracts by supporting voltage in the SCE Vincent 500/230 kilovolt transformer.  Antelope Valley is in open gas territory.  Enron proposes to locate the facility only one mile from both the Kern/Mojave and PG&E’s mainline.  However, the Antelope Valley petition references only a Kern/Mojave connection.

Pastoria Power Project is expected to file with the CEC to construct  a 960- megawatt plant on the Tejon Ranch property about 25 miles south of Bakersfield, approximately 6.5 miles east of the Grapevine interchange on Interstate Highway 5.   The developer would connect to SCE 230 kilovolt line at the Pastoria substation that feeds into the Midway Sunset Control Center via PG&E’s transmission facilities.  Pastoria is located in SoCalGas service territory.

GAS SUPPLY

Each of the seven projects has various gas supply options: interstate, intrastate or new California indigenous supply.

Natural Gas Pipelines

About a mile north of McKittrick, at a location dubbed 17-Z, at least six pipeline systems interconnect, one interstate, one intrastate, and four foreign systems.  They are Kern/Mojave, SoCalGas, Occidental, Texaco, Chevron, and Mobil/Aera.  The diversity of gas supply and the proximity to PG&E’s Midway Substation are primary reasons power developers chose the McKittrick area.

Interstate Pipeline

In 1992 Kern River Gas Transmission Co. and Mojave Pipeline Operating Co. extended a competitive FERC jurisdictional interstate pipeline, called Kern/Mojave, into PG&E’s service territory in the Bakersfield, California, area.  Kern/Mojave provided the market an additional 1100 MMcf/d of supply: 700 MMcf/d Rocky Mountain and 400 MMcf/d Southwest.  Since the effective Kern/Mojave’s rates were lower than PG&E’s, approximately 300 MDth/d of PG&E’s load switched to Kern/Mojave service.  In addition, new load in the Bakersfield area connected to Kern/Mojave.

Kern River has run at virtually full capacity since beginning operation.  Kern River has had several open seasons to expand capacity to support a booming Las Vegas market and a proposed extension to Los Angeles.   Occidental-Elk Hills supply is supporting Kern/Mojave’s west end operations and that Kern/Mojave may lose this support when the Occidental Elk Hills Power Project comes on line.

Intrastate Pipelines

SoCalGas is certificated (CPCN) by the CPUC to serve most of the Bakersfield area.  Discussion with PG&E’s Bakersfield operations confirms that the four McKittrick projects and Pastoria are in SoCalGas’ service territory.  High Desert is in Southwest Gas’ certificated territory.