Enron   			Transportation

& Services

| To: | Kevin Hyatt |  |  |
| --- | --- | --- | --- |
| From: | Mansoor Abdmoulaie | Department: | S. W. Facility Planning |
| Subject: | TW/ Agave | Date: | 2-22-2001 |

This is in response to our meeting of Feb-8-2001 with Agave regarding the following projects:

1): Determining the existing configurations of compressor units at Atoka-3

The existing valves/ piping on unit #762 and #822 upstream of Atoka-3 are capable of receiving gas from low and high

suction services.

Therefore the LEC service with two-assigned compressor units can handle up to 60 mmcf/d from Atoka-3 to Atoka-2.

No modifications are required at this time.

2): Build a new 25 mmcf/d interconnect for Aid State at discharge side of Atoka-2

To build a new tap at this location, TW needs to loop the existing approximately 400 feet of 12”-10” with

New 400 feet of 16” piping from the discharge side of Atoka-2 to 24” mainline. The existing pressure drop, with a

155 currently flowing across these segments of pipes, is 6 #. Assuming another 25 mmcf/d is added the

pressure drop would reach to 9-10#  which will have an adverse impact on TW-1. By modifying it, the pressure drop

Would be reduced to 2-3#. The existing measurements at Aid-State will be used for this operation.

The estimated level “A” release cost is $ 120,000.

3): Facility requirements to handle an additional 75 mmc/d of Agave’s gas at Atoka-3 ( see 3-Alternatives)

Criteria used for the hydraulic calculations were:

● Agave pressure of 1000 psig at Atoka-3

● TW-36” mainline operating pressure of 940 to 970 psig.

● Design volume of 75 mmcf/d.

Alternatives			Facility requirements				Level “A” ( ± 30%)

#1				21 miles of 16”					   $9,500,000

Measurements, Communication facilities	      	      $450,000

And a tap/ valve at TW 36” mainline

(No Compression)

Total										   $9,950,000

#2					21 miles of 12”  					$7,560,000								Measurements					   $450,000

300 hp						   $500,000								(Ps=900#,Pd=970#)

Total											$8,510,000

#3					21 miles of 10”					$5,880,000

Measurements					   $450,000

1,100 hp					$1,700,000

(Ps=722#,Pd=970#)

Total											$8,030,000

If you have any questions please call me at 713-853-3158.