# To:  	Eric Groves

From:  Robert Lee

Date:	October 10, 2000

Subj: 	LNG Spreadsheet Model Review

The LNG spreadsheet model was reviewed for accuracy and completeness.  No errors or inconsistencies were identified, except that the values for shipping cost and terminal cost are interchanged on sheet ELBA INCOME.  The total cost column is not affected, so this interchange has no effect on the overall result.

No independent model definition was available for comparison.  However, the overall model appears to be a complete and reasonable representation of all cost and revenue elements.  A written description of the model might be useful in facilitating a broader review as the model is further developed and expanded.  The attached description of the spreadsheet was developed in this review, and may be useful in introducing the spreadsheet to others that work on its development.

I found some relevant documentation that may be of interest to you on the following sites:

http://www.fairplay.co.uk/markets/MFLNG.htm	LNG Outlook

http://www.lngexpress.com/two.htm		New liquefaction technologies

http://www.shipsim.com	Simulation model description - list of model input variables for shipping, etc.

The simulation model input description offers some insight on additional variables that might become important as you get to the point where you want to optimize your ship/routes/markets to maximize income.  For example, there is a speed fouling factor that increases with time from drydock – the examples suggests a typical addition of 1 day to a round trip after a year.  Scheduled and unscheduled maintenance and warm vs cold loading/unloading conditions add to trip times.  The addition of these and other stochastic variables, such as weather, port delays, etc. will impact trip times.

The other two articles review new technologies to load gas from offshore platforms and improvements in new ship design (e.g., boiloff rates as low as 0.09%/day).

# Attachment

# LNG Model Review

# The following pages provide a description of the calculations in the spreadsheet LNG_CurvesMod.  It is organized in three sections by individual worksheets:

# Basic Data

# Ship and Terminal Cost Curves

# Income Data

The tables are meant to act as a guide to following the calculation flow of the spreadsheet.  A symbol is defined for the values (or series of values as a function of date) to define the cell calculations.  The first one (or two) characters in the symbol refer to worksheet containing the variable.

# Pxx	- 	PORTS

# Rxx	-	ROUTES

# Sxx 	- 	SHIPS

# Fxx 	-	FREIGHT

# CLxx 	- 	CURVELOAD

# CCxx	-	CURVECALC

# SCxx	-	SHIP CURVES

# TCxx 	-	TERMINAL CURVES

# EBxx	-	ELBA BOOK

# EIxx	-	ELBA INCOME

# Mxx	-	SUMMARY

# Hxx	-	SHIP POSITIONS

# A. Basic Data

# Sheet PORTS

Table  Port Capacity Charge and Processing Fees

| Port City | ##### Symbol | ELBA | LAKE CHARLES |
| --- | --- | --- | --- |
| Annual Capacity (mmbtu) | PAC | 62,906,725 |  |
| Fuel and Return Gas (% of Unloaded) | PFR | 2.500% |  |
| Terminalling Fee | PECC | 2.660% | $0.300 |
| Terminalling Fee Inflation | PTPI | 1.250% | 1.250% |
| Utilization Rate | PUR | 100.000% | 100.000% |

| Tables | ### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Port Charges by Ship | PCS | $ |  |
| Fixed Capacity Charge | PCPF | $/Month |  |
| Escalating Capacity Charge | PECC | $/Month | PECC(PreMonth)*(1+Escalation/12) |
| Terminal Processing Fee | PTPF | $/Month | PTPF(PreMonth)*(1+PTPI/12) |

# 2. Sheet ROUTES

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Distance (Supply to demand Pt) | RD | NautMiles |  |
| Loaded MMBtu/Day | RLM | MMBtu/Day | SC/RD |
| Laden Voyage Days | RLV | Days | RD/SSD |
| Round Trip Voyage | RRT | Days | 2*RD/SSD + SL + SU + SST(if Suez) |
| Bunker Fuel Cost/Cargo | RBF | $/cargo | CLHS*{(RD/SSD)*SBL + (RD/SBB)*SBB + (SL + SU)*SBP + SST*SBP(IF Suez)} |
| Max Capacity /Yr Unloaded | RMC |  | RLM * 365.25 * (1 – SSBL) |

3. Sheet Ships

| SPECS | ##### Symbol | UOM | 1 | 2 |
| --- | --- | --- | --- | --- |
| CAPACITY mmbtu | SC | MMBTU | 2,041,266 | 3,215,400 |
| CAPACITY m3 |  | M3 | 87,608 | 138,000 |
| SPEED mph |  | MPH | 17.5 | 18.4 |
| SPEED mpd | SSD | MPD | 420.0 | 441.6 |
| BOILOFF-LADEN | SSBL | DAILY | 0.275% | 0.150% |
| BOILOFF-PORTDAY | SSBP | DAILY |  |  |
| LOADING | SL | DAYS | 1 | 1 |
| UNLOADING | SU | DAYS | 2 | 2 |
| SUEZ TRANSIT (RT) | SST | DAYS | 3 | 3 |
| BUNKERS-LOADED | SBL | Tons/Day | 145 | 170 |
| BUNKERS-BALLAST | SBB | Tons/Day | 140 | 160 |
| BUNKERS-PORT | SBP | Tons/Day | 40 | 50 |
| BOILOFF AS FUEL |  | YES/NO | YES | YES |
| BOILOFF TO BUNKER | SBTB | TONNE/M3 | 53% | 53% |
| SHIP UTILIZATION |  | RATE | 100% | 100% |
| CPI ESCALATION |  |  | 2.5% | 2.5% |

3. Sheet SHIPS (Cont)

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Fixed Charter Cost | SFC | $ |  |
| O&M Charter Cost | SOC | $ |  |

4. Sheet FREIGHT

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Other Costs (Import Fees, etc) | FOC | $/mmbtu |  |

5. Sheet CURVELOAD

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| INT (Interest Rate) | CLIN | % |  |
| NG - PR | CLNGP | $ |  |
| NG - VOL | CLNGV | /Yr |  |
| IF-HEHUB – PR - D | CLHHD |  |  |
| IF-HEHUB – PR - I | CLHHI |  |  |
| HSNF - PR | CLHS | $/ton |  |

6. Sheet CURVECALC

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| AA LIBOR | CCL | $ | CLIN |
| Discount Percentrage | CCDP | %/Yr | 1/(1 + CCL)^(t/182.625) |
| Henry Hub Price | CCHH | $ | CLNGP +CLHHD + CLHHI |
| Average Annual Discount | CCAD | %/Yr | Sum of CCDP/Nr of months |

# B.	Ship and Terminal Cost Curves

- Sheet SHIP CURVES

This sheet provides the shipping cost/mmbtu (SCSC), which has the following components:

Fixed_Charter_Cost/Day + OM_Charter_Cost/Day			SFC + SOC

=

Loaded_mmbtu/Day					     RLM

+

###### Fuel_Cost/Capacity(mmbtu)						RBF/SC

Port_Charge  +  Port Charge  +  Suez_Charge   	   	          PCS   +    PCS   +   PCS

(Origin)	       (Destination)			  	         (Origin)    (Dest.)    (Suez)

+

###### Capacity(mmbtu)						SC

+

Other_Costs/Loaded_mmbtu						FOC

- Sheet TERMINAL CURVES

This sheet provides the All-in Terminal Cost/unloaded mmbtu (TCTC), which has the following components:

Elba

Fixed Capacity  +  Escalating Capacity

Charge	       	   Charge			  	        PCPF   +    PECC

=

###### Total Capacity * Utilization Rate				EBTC*PUR

+

Terminal Processing Fee						PTPF

Lake Charles

=

Terminalling fee + inflation				        PECC*(1 + PTPI/12)

D.    Income Data

- Sheet ELBA BOOK

Table Elba Capacity

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Total Elba Capacity | EBC | mmbtu | (PAC/365.25)*t in period |
| Open capacity | EBOC | mmbtu | EBC – EBU(sum all supply points) |

Table Algeria supply to Elba via Suez with Hoegh Galleon

| Tables | #### Symbol | Values | Calculation |
| --- | --- | --- | --- |
| Gas Index |  | IF-HEHUB | (PAC/365.25)*t in period |
| Basis | EBBA | 0.02 |  |
| Marginal Cost |  | Lake Charles |  |
| Elba vs LC | EBVS | 0.09 |  |
| Netback Sharing % | EBNS | 0% | EBC – EBU(sum all supply points) |

Table Algeria supply to Elba with Hoegh Galleon

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Loaded Volumes | EBL | mmbtu | (PAC/365.25)*t (one month) |
| Boiloff | EBB | mmbtu | EBL * SSBL * RLV |
| Unloaded | EBU | mmbtu | EBL + EBB |
| Fuel & Return | EBFR | mmbtu | - EBU * PFR |
| Available for Sale | EBAS | mmbtu | EBU + EBFR (EBFR is negative) |
| Revenue | ##### EBR | $ | EBAS * (CCHH + Basis) |
| Shipping | EBS | $ | - EBAS * SCSC |
| Terminal Charge | EBTC | $ | - EBU * PTPF |
| FOB Supply Cost (This calculation may be replaced. The current form assumes Enron will pay the sale price – costs with some sharing of price differentials) | EBFB | $ | -EBL*{CCHH * EBAS/EBL – SCSC - TCTC * EBU/EBL - EBNS * [(EBVS-EBBA) - (SCSC(Elba) – SCSC(Lk Charles)) -(TCTC(Elba)-TCTC(LC)*EBU/EBL)]} |
| Total Operating Cost | EBTO | $ | EBS + EBTC + EBFB |
| Percent of Elba Capacity Charge | EBPC | $ | - [(EBU/PAC)/(t/365.25)] * [PCPF + PECC] |
| Total Cost | EBTC | $ | EBTO + EBPC |
| Income | EBI | $ | EBR + EBTC (EBTC is negative) |

Two other route/ship pairs are included with a structure the same as the above table.  The only difference is the calculation of loaded volume, unloaded volume and boiloff.  Unloaded volume is the minimum of 1) remaining capacity (total capacity – amts unloaded by the first ship/route) and 2) amount the ship has available to unload.  The loaded volume is the amount unloaded divided by (1 - %boiloff*NrDays).  Boiloff then applies only to the volume actually unloaded.

A final (1-column) table provides the sum of PTPF + PECC, which is used in sheet SUMMARY.

2.   Sheet ELBA INCOME

This sheet sums the data from ELBA BOOK to provide yearly data for each quantity from 2000 to 2025.

Table Volumes

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Total Capacity |  | mmbtu | EBC by year |
| Total Volumes Loaded |  | mmbtu | EBL (all ship/route pairs) by year |
| Total Unloaded |  | mmbtu | EBU (all ship/route pairs) by year |
| Available for Sale |  | mmbtu | EBAS (all ship/route pairs) by year |
| Elba Capacity available for Spot Sale |  | mmbtu | EBOC by year |

Table Income Summary

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Total Revenue | EITR | $ | EBR (all ship/route pairs) by year |
| Supply Cost | EISP | $ | EBFB (all ship/route pairs) by year |
| Shipping Cost* | EISH | $ | EBS (all ship/route pairs) by year |
| Terminal Cost* | EITE | $ | EBTC + EBPC |
| Total Cost | EITC | $ | EISP + EISH + EITE |
| Net Income | ##### EINI | $ | EITR - EITC |
| PV Net Income | EIPV | $ | EINI * CCAD |

* The calculation of Shipping Cost and Terminal Cost is as it is intended to be.  The spreadsheet reviewed interchanges these two components from the ELBA BOOK sheet. The end result, Total Cost, is correct.

- Sheet SUMMARY

This sheet sums the data from ELBA INCOME and ELBA BOOK to provide yearly data for each quantity from 2000 to 2025.

Table LNG Summary

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Elba Revenue |  | $ | EITR |
| Elba Expense (used capacity) |  | $ | EITC |
| Undiscounted Net Margin |  | $ | EINI |
| PV Net Margin |  | $ | EIPV |
| Fixed Cost Elba Open Capacity | MEOC | $ | PTPF + PECC + EBPC(all ship/route pairs) |
| Fixed Cost HG Open Capacity | ##### MHG | $ | (MDA/365.25) * (SFC + SOC) * T (days per month) |
| Fixed Cost EXMAR Open Capacity | MEX | $ | (MDA/365.25) * (SFC + SOC) * T (days per month) |
| Total Fixed Cost of Open Capacity |  | $ | MEOC + MHG + MEX |
| Mmbtu’s open capacity |  | mmbtu |  |
| Days avail. – Open capacity (by ship) | MDA | days | HOC |

The fixed cost + operating cost by ship (SFC + SOC) is stored on this sheet to use in the calculation of each ship’s fixed cost open capacity.

4,   Sheet SHIP POSITIONS

| Tables | #### Symbol | Units | Calculation |
| --- | --- | --- | --- |
| Days available per year | HDA | Days | (1 – EBU/RMC) * 365 |
| Total open capacity by ship | HOC | Days | HDA (rounded) |