Archer Daniels Midland

Confidential Information

| Location | Demand (KW) | Generation Capability (KW) | Utility Tariff | Interconnect Agreement | Real Time Metering | Interconnect Voltage (KV) |
| --- | --- | --- | --- | --- | --- | --- |
| Southport, NC | 25,000 KW | 52,500 KW | CP&L Buy: Interruptible standby w/ RTP rider{LGSTOU92 & Riders RTP & 57} Sell: Based on market as quoted by CP&L or standard QF price | No; parallel generation agreement | No | 13.2 KV at ADM Sub; 40 MVA Transformer; 230 KV supply |
| Peoria, IL | 50,000 KW | 60,000 KW | Cilco Buy: Confidential based on RTP Sell: Based on market as quoted by Cilco (FERC filed) | No; parallel generation agreement | No | 13.8 KV at Edmund St. Sub; 25 MVA Transformer; 69 KV supply |
| Clinton, IA | 120,000 KW | 26,000 KW | Alliant – Interstate Buy: Bulk Power Supply with Standby Sell: Not Applicable | No; parallel generation agreement | No | 13.8 KV & 4.16 KV at ADM Sub; 100+ MVA Transformers; 69 KV supply |
| Cedar Rapids, IA | 120,000 KW | 155,000 KW 100,000 KW being added | Alliant – IES Buy: modified LGS Sell: standard QF price | No; parallel generation agreement | No | 69 KV & 34.5 KV at Beverly Sub; 200 MVA Transformer & 100 MVA Transformer; 161 KV & 69 KV supply |
| Decatur, IL | 400,000 KW | 230,000 KW | Illinois Power Buy: Ameren Energy supplier Trans/Delivery: 300 MW network service Sell: standard QF price | No; parallel generation agreement | Yes | 138 KV at Faries Parkway & ADM North; 138 KV supply |
| Des Moines, IA | 10,000 KW | 5,500 KW | MidAmerican Energy Buy: Confidential rate Sell: standard QF price | No; parallel generation agreement | No | 13.8 KV at ADM Sub; 15 MVA Transformer; 69 KV Supply |