NAPOLEONVILLE STORAGE #1

MECHANICAL INTEGRITY TEST PROCEDURE

WELL DATA:

# TUBLARS		WEIGHT		DEPTH		DESCRIPTION

16”			122.15 #/FT.		2,009 FT.		LAST CEMENTED

CASING

8 5/8”		  36 #/FT.		3,211 FT.		HANGING STRING

PROCEDURE:

- The test will be run at the highest possible test pressure, based on the natural gas market conditions, but no higher than the equivalent pressure at the casing seat of 0.85 psi/ft gradient.

- Shut-in the well for several days, to allow stabilization.  Monitor the pressures and check the wellhead components for leakage.  Correct leakage problems.

- Run a pressure and temperature survey.

- Monitor the cavern pressure for 72 hours and then run another pressure and temperature survey.

- Prepare and submit the appropriate government reports.