MEMORANDUM

To: 	Michael Guerriero

Cc: 	Jeff Kabel

From. 	Mario Aguilar Benitez

Date: 	November 10th,  2000

# Ref. :	Arcor / Epec Contract Price Evaluation

### Contract Price Breackdown

  - Base Price:

| Energy Price: | $ 31.70 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity Price: | $ 5.30 |  |  |  |  |  |  |
| Variable Transportation Cost | $ - | (Close to cero because there is a net from the node they buy and sell) |  |  |  |  |  |
| Toll | $ - |  |  |  |  |  |  |
| Total Base Price | $ 37.00 |  |  |  |  |  |  |

- Actual Price: 	$	      37.71 (October Price)

- Formula:

- Last Prices:

| Month | US$/MWh |
| --- | --- |
| Jan-00 | 38.29 |
| Feb-00 | 38.30 |
| Mar-00 | 38.30 |
| Apr-00 | 38.42 |
| May-00 | 37.70 |
| Jun-00 | 37.67 |
| Jul-00 | 37.67 |
| Agu-00 | 37.73 |
| Sep-00 | 37.73 |
| Oct-00 | 37.71 |
| Average | 37.95 |

### Current PPA Valuation

Market Valuation with forward curve from Oscar and Modreg Simulation Model.

Out Of The Money PPA of 2.7 MM$ ($13.2 /MWh)

### Alternatives Analysis

- TG Rio Cuarto + Capacitors . (Las Ferias Substation)

They need to implement both projects because none of them solve the problem by itself.

TG Rio Cuarto:

2 Gas Turbines of 16 MW of Installed Capacity located in Rio Cuarto 5 Km from Arcor Power plant.

They are very old machines that will probably be not feasible to reconvert. They will have to turn this units on even though they are very inefficient (forced dispatch).

Dispatch Data:

TG Rio Cuarto Valuation:

This solves 50% of the problem and the Installation of Cacitors solve the rest of it.

Capacitors:

Installation of Capacitors or “Banco de Capacitores” for making the necesary Reactive Capacity in order to mantain the level of tension in the area. The size should be of 20 MVAR (Maga Volts Ampere of Reactive capacity). The aproximative cost of this equipment is around $ 2.5 MM.

TG Rio Cuarto & Capacitors Value:

- High Voltage 132 KV Line (Almafuerte – Rio Cuarto)

High Voltage 132 KV line that will connect Rio Cuarto to the 500 KV line and will alloud Rio Cuarto

- New Substation in Villa Maria

New Substation in Arroyo Cabral Va. Maria. “Subestación de Rebaje y Maniobras”.

Epec was working in this project some time ago this year and they suspended. This project was going to release Rio Cuarto to supply Va. Maria. These means that there is going to be a decrease on the Power and Tension needs in the Area. Va. Maria will be supply by this 500 KV System conection.

Some works needs to be done in Va. Maria Substation and a new Transformer needs to be bought.

#### Summary

The difference against the cheapest option is (k$) 963.4 but this is the longer term option.

These means that although these Current PPA is way Out of the Money, the option of this contract compare to other options is fair enough base upon this assumptions and rough numbers.

Another alternative that they could be thinking is in the Project of installing a 250 MW Combine Cycle in Villa Maria that could really avoid the dependance that EPEC has with Modesto Maranzana. (Not a 100%, but it will eliminate most of it). Inepar and Gecor shareholders were behind these proyect some time ago. ENEL from Italy was interesting too. (Ansaldo Equipments).

Consider this analysis a first aproximation because none of this numbers are completely check and revised due to the time we had. We will probably need some time to make up these numbers and come out with a better aproximation. This is just the best information that we have right now.

Sincerely,

Mario.