# MIDLAND, TEXAS

Congratulations to Olan Smith on his new job as Pipeline Specialist.

## SAFETY

This Week			Year to Date 2001

| Vehicle Accidents | 0 | 4 |
| --- | --- | --- |
| Lost Time Injuries | 0 | 0 |
| OSHA Recordable | 0 | 0 |

- Teams working on  ESMS action items.

## COMPRESSION

- WT2 202 Compressor alignments complete. Unit 201 & 203 brick communication failure. AC power failure on 10/12 – generator put on line (lost purchase power)  Purchase power back on 10/13 – generator taken off line.  Cooper finished job on 10/13; Quail didn’t tighten flange bolts on bottom flanges resulting in ESD on plant when attempting to pressure up Unit 202.  10/14 Unit 3 brick communication failure.  10/15 tightened pressure packing on compressor 2 Unit 202 packing was still loose.   10/16 worked on ESD blow down valves and pulled wire to 201 evaporative cooler fan.

- Keystone Unit 3 down 10/13 due to high discharge pressure.  Unit 2 put on.

- Compelx 30” project is 85% complete.

- Replaced radiator & worked on pump at the Brant Adkins Pump.

- Completed minor overhaul on unit 8 at Spraberry.

## PIPELINE

- Wink Station power failure.  Back in service.

- Witnessed Apache line crossings.

- Crane completed 12” ROW clearing on Crane to McCamey Rancho and Crane to Hwy 67.

- Installed 2 gates on ROW towards Iraan.

- PLC was down at the McAfee Station.

- Pump #1 at Colorado City knocking.

- Installed tank mixer on tank 56005 McElroy Station.

## ENVIRONMENTAL

- Teams attending Environmental Roundtable.

- Leak on June Tippet.  Two barrels  out 0 recovered

- Leak reported on EK Queen. Three barrels out 0 recovered.

- Ongoing leak on Garza line. 1- two bbl, 1-25 bbls.

## CUSTOMER SERVICE

- Crane & Monahans Public awareness program.

- Eunice Team met with Larry Strain on old TNM lines on his land, to see about getting rid of the lines.

## OTHER

- Team members attending Facilitating Training.

- Mark Ethridge attending self directed work team task force.

- Teams attending Corrosion Meeting.

- Lance Nash has a Navajo lease to connect at Abell Sour 700 bbls day.

- Teams attending Asbestos Training.

- Teams attending Mechanical roundtable

- Teams attending UAF Meeting in Amarillo.

### SCHEDULE (Week of October 22nd)

|  | Monday | Tuesday | Wednesday | Thursday | Friday |
| --- | --- | --- | --- | --- | --- |
| Scott Clark | Region | Region | Region | Midland | Midland |
| Bob Bandel | Jal / Eunice | Lovington | Eunice | Midland | #### Vacation |
| James Lawrence | Seminole / Plains | Midland | Brownfield / Andrews | Midland | Midland |
| Randy Mobley | Big Lake | Midland | Crane | Midland | Midland |
| Robert Newman | Midland | Midland | Big Lake | Midland | Midland |
| Don Bishop | Houston | Houston | Houston | Houston | Houston |