Enron Murphy Medusa Bid

PCS Scope of Work

# Cost estimates for the following cases

- Assume a SPAR at MC 538 and a fixed shelf platform receiving host

- Assume a pipeline MAOP of 2200 psi unless PCS significant reason for change

- Assume a 1300 psi receiving pressure for Cases 1-3

- ** Note: Murphy has assumed 12” SCRs for both oil and gas…Murphy would be interested to know if there is significant savings with smaller sizes.

- Include a list of components to be included at both SPAR and receiving host (metering, chemical injection, pigging, hull piping, SCR, etc.)

- Calculate and document expected operating pressures at the Medusa facility for 40 MBOPD in Case 3 and 60, 80, 110 MMSCFD in Case 1 (same pipe size for all three gas volumes)

- 110 MMSCFD gas pipeline from MC 538 to WD 152

- 110 MMSCFD gas pipeline from MC 538 to WD 143

- 40 MBOPD oil pipeline from MC 538 to WD 143 (note maximum allowable oil throughput in the designated line size).

- Oil pipeline one incremental line size up from Case 3 from MC 538 to WD 143 (note maximum allowable oil throughput)

# Proposed installation schedule for Cases 1 and 3

Include time allotments for permitting, route surveys, engineering, design, materials procurement, offshore installation of prelays (plan for 6/02), SCR hook-up from prelaid pipelines to SPAR

Resume of PCS’s deepwater pipelines (designed and or installed)

Include dates that pipelines were installed

Briefly describe recommended plans for corrosion protection and future surveys for both the pipelines and their associated SCRs.

Note: Enron will prepare the maps involved.