- Observations and Recommendations

- Station 4

- Unit 403E

- 1/8/2002

- Power cylinder 4L seems to have a large head gasket leak. This needs to be repaired as soon as possible to stop any more damage from happening.

Priority: 2

- Power cylinder 1L has indications of cylinder pressure leakage and may need to have the rings changed. The liner needs to be inspected for wear.

Priority: 3

- Power cylinder 1R’s ultra sonic trace shows a very large response at the fuel valve closure. The lifter may need to be readjusted.

Priority: 3

Note: I will attempt to use a number system that will assign a priority to the items listed in the report:

1 = Damage has already occurred to the equipment with possible safety concerns.

2 = If condition is not corrected ASAP damage to equipment and loss of production will occur.

3 = Conditions exist that are causing the equipment not to operate at the best efficiency. Repairs should be accomplished at the next opportunity.

4 = Abnormal conditions exist, maintenance is required but can be planned and scheduled.

5 = Indications of possible abnormal conditions. Will monitor and allow to develop further before action required.