| Distribution and Attendee List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mike Nelson |  | Earl Chanley |  | Rod Boschee |  | Mel Wong |
|  | Lou Soldano |  | Ray Smith |  | Dave Anderson |  | Tony Estrada |
|  | Rich Jolly |  | Larry Campbell |  | Curt Russell |  | Glen Riddle |
|  | David Roensch |  | Duane Luzum |  | Rich McCurdy |  | Bob Beckam |

10:15 AM     Refreshments – “Laughlin Room” – 3rd floor

10:30 AM 	Meeting Begins

12:00 PM	Luncheon Buffet – Laughlin

| Agenda |  |  |
| --- | --- | --- |
| Nbr. | Topic | Responsible |
|  | PG&E/TW Gas Quality Analysis/Pigging Update | TW / PG&E |
|  | PG&E / TW Project Update - Drip Installations - Customer Tap Separators - PG&E A&B Mainline Separators | TW / PG&E |
|  | PG&E Report on Downstream Testing | PG&E |
|  | Options / Next Steps | PG&E / TW |
|  | Follow-up Meeting Date |  |

## Annotated Notes

### Topic 1.  PG&E / TW Gas Quality Analysis/Pigging Update

|  | Discussion: TW continues to bring in weekly pigs runs dry and no liquids have been picked up at either |  |  |
| --- | --- | --- | --- |
|  | TW's or PG&E's Topock filter / separator. A hard copy of all 2000 collected analytical data was provided |  |  |
|  | to Dave Anderson. PG&E cancelled the last two sample runs based on other work load issues. |  |  |
|  | Conclusions: TW continues to deliver very dry gas at the Topock point. PG&E is satisfied with the results |  |  |
|  | at Topock and recognizes the cooperation and efforts to date, but is unwilling to increase volumes until |  |  |
|  | upper management is satisfied with longer term efforts. |  |  |
|  | Action items: Both parties need to remain dedicated to running the weekly | Responsible: | Deadline: |
|  | sample run. | TW / PG&E |  |

### Topic 2.  PG&E / TW Project Update

Drip Installations, Customer Tap Separators, PG&E A&B Mainline Separators

|  | Discussion: Drip installations on PG&E's A&B lines at MP 27, 63, and 84 have been completed . To date, |  |  |
| --- | --- | --- | --- |
|  | only the drip at MP 63 A has produced liquids, with analysis in the 3 PPM range. Customer tap filter / |  |  |
|  | separators have been installed at 5 locations. The PG&E A&B mainline filter / separators are expected to be |  |  |
|  | installed as a turn-key project managed by ET&S. |  |  |
|  | Conclusions: Engineering drawings for the filter /separator installation were approved at the meeting. |  |  |
|  | Action items: The new drip installations and customer tap filter /separators | Responsible: | Deadline: |
|  | will be added to the sample run data as official sample points. | David Roensch | now |
|  |  | Curt Russell |  |

### Topic 3.  PG&E Report on Downstream Testing

|  | Discussion: PG&E has gathered liquid and wipe samples for analysis downstream of Pisgah (the proposed |  |  |
| --- | --- | --- | --- |
|  | Site of the mainline filter / separator) as far as MP 225.91. There were some points indicating positive for |  |  |
|  | PCB's, with a range of non-detect to 38 PPM. Most liquid sample analysis indicated <1 to 6 PPM. |  |  |
|  | Conclusions: A hard copy of all gathered data was provided. This represents a first ever collection of liquids |  |  |
|  | for analysis at these points. Future samples will be collected and analyzed for potential additional customer |  |  |
|  | filter / separator installations. |  |  |
|  | Action items: PG&E will collect additional samples at these new points for | Responsible: | Deadline: |
|  | review. Each tap where liquids were collected will be scoped for a potential | PG&E |  |
|  | filter / separator installation, which is aligned with a containment strategy. |  |  |

### Topic 4.  Options / Next Steps

|  | Discussion: PG&E plans to meet with direct and industrial customers to notify them of the potential for |  |  |
| --- | --- | --- | --- |
|  | PCB's. They may meet with Southwest as early as next week and So Cal a bit later. PG&E also believes |  |  |
|  | deconning the Topock Compressor Station is imperative to minimize customer and employee exposure . |  |  |
|  | Conclusions: TW stated that deconning the Topock Compressor Station is outside the containment strategy |  |  |
|  | that was approved, not cost effective nor is it realistic to expect total decontamination. PG&E plans |  |  |
|  | to contact Vector to develop a scope of work for Topock, per PG&E's upper management. |  |  |
|  | Action items: Lou Soldano will contact So Cal's Dr Komai to potentially | Responsible: | Deadline: |
|  | share practices and find out how So Cal handles PCB's at the customer | Lou Soldano | asap |
|  | level. PG&E will provide a report on the cooling tower decomissioning which includes the cooling tower analytical. | Mel Wong |  |

Next Meeting Date: Rod Boschee will contact Mike Nelson