PIRQUE SPILL – FACT SHEET

Incident

Time of occurrence-		July 10 at 4:30 pm

System Isolated		4:40 pm

System Stabilized		5:25 pm

Amount lost			approx.  2000 bbls

Type of Product Lost		Mix medium crude and condensate

Cause	Third Party Spill – Contractor’s (Petrosur) backhoe hit a by-pass valve severing the conection.

Notification to 		SIRESE			July 10th 20:48

Environment Vice Ministry	July 10th 20:56

Cochabamba Prefecto		July 10th 17:50 (Telef.)

VMEH				July 10th 20:58

Insurance			July 11th 09:00

Shareholders			July 10th 17:15 (email)

Affected Area

Total area impacted			approx. ¾  hectare

Sprayed vegetated hill slope		approx.  5000 sq. mts.

Trench					600 sq. mtrs

Risk to Resources

Human		None (Caserio a 400 metros – aprox. 60 houses)

Flora		Matorrales (75), Cactus (15), Arboles (10)

Fauna		No damage to domestic stock – low risk and exposure

Aquifera	No contact with bodies of water (low risk/high exposure)

# Response

First Response 	Immediate by Contractor personnel (within minutes).  Alerted operator system at the same time.

Response Crew	70 including workers (50) and support (20)

Personel with protective gear in place

Liquid Hydrocarbon	Being pumped into oil trucks

Total Hydrocarbon

recovered to date	500 bls	(July 11th 6 pm)

Liquid Hydrocarbons are being taken to Sayari

Meetings held with	Prefectura Cochabamba

Dept. Medio Ambiente Cbba.

Brigada Parlamentaria  Cbba.

Environmental 	Contracted Dames & Moore for sampling

Water samples being taken at irrigation canals

3 taken on July 12th at the irrigation channel

Soil samples being taken at hill (3 on July 12th)

Contracted Polaris for Technical & Scientific Support

Contracted GA for Geotechnical and Biological Support

Main Issues:	Final decision on contaminated vegetation at the hill

Final disposal of solid waste

# Assesment

Spill contained by natural resources and trench

Minimal observed ecological impact

Potential seepage into farm land and river valley – needs to be monitored and analyzed.